# HP Insight Management WBEM System Temperature Sensor Provider

| Description | The HP Insight Management Web-Based Enterprise Management (WBEM) Management System Temperature Sensor provider extends the management capability of referencing profiles by adding the capability to represent HP Management Processor on HP servers. This provider implements the following profiles and installs the necessary files: |  |
| --- | --- | --- |
| Profile Name | Organization | Version |
| HP Sensor Profile | HP WBEM TC | 1.0 |

| Requirements | For each hardware architecture listed, this provider requires the following distributions HP Integrity managed servers; SLES 10 and later RLEL 5.0 and later. HP ProLiant managed servers; SLES 11 and later RHEL 5.3 and later. |
| --- | --- |
| Release History | Initial release with HP Insight Management WBEM Providers for Linux v2.0. |

## Setting Up the Provider

| Installing the Provider | There are no special installation instructions for this provider. It is installed by default as part of the HP Insight Management WBEM providers. |
| --- | --- |
| Configuring the Provider | This provider does not accept specific configuration adjustments beyond standard HP Insight Management WBEM support. |

## Using the Provider

| Namespaces Supported by the Provider | This provider returns instances in the root/hpq namespace. |
| --- | --- |
| Schema Supported by the Provider | This provider supports the following classes: - SMX_NumericSensor - SMX_SensorCollection - SMX_SystemSensor - SMX_HostedSensorCollection - SMX_MemberOfSensorCollection |

The tables in the following sections describe the properties of the supported classes. The classes are categorized by the class or superclass that defines the property, the first column is the Property Name (including type and units) and the second column describes how the provider determines the properties implementation. When the Property Implementation value is a number, the number given is the default behavior and the Managed Object Format interpretation is within parenthesis. If other values are returned, a problem is indicated.

Unless otherwise noted, all of the property implementation values given are for HP ProLiant and HP Integrity (cellular and non-cellular) systems. The location related properties and implementation values are determined based on the server type so they may differ.

### SMX_NumericSensor Class

The SMX_NumericSensor class implements the HP_NumericSensor and is used to model temperature sensors on servers.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | “Temperature Sensor <sensorID>” Where: <sensorID> is the name displayed by the MP or iLO. For example: Temperature Sensor Temp 1 |
| Description | “Temperature Sensor <sensorID> detects for NumericSensorType.StatusDescription” Where: <sensorID>” is the name displayed by the MP or iLO represents the number of the temperature sensor instance, NumericSensorType represents the value of NumericSensorTypeand StatusDescription represents the value of StatusDescriptions[0]. |
| ElementName | “Temperature Sensor <sensorID>” Where: <sensorID> is the name displayed by the MP or iLO. For example: Temperature Sensor Temp 1 |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | 2 (OK) – if temperature reported by the sensor is within normal operating range; 2 (OK) – if temperature reported by the sensor has exceeded its non-critical threshold and has set fans to full speed; 6 (Error) – if temperature reported by the sensor has exceeded its threshold and the system is gracefully shutting down. |
| StatusDescriptions[0] | Temperature reported by the sensor is within normal operating range – if temperature reported by the sensor is within normal operating range; Temperature reported by the sensor has exceeded its non-critical threshold and the fans have been set to full speed – if temperature reported by the sensor has exceeded its threshold and has set fans to full speed; Temperature reported by the sensor has exceeded its threshold and the system is gracefully shutting down – if OperationalStatus is 6 (Error) |
| HealthState | 5 (OK) – if OperationalStatus[0] value is 2 (OK); 20 (Major Failure) – if OperationalStatus[0] value is 6 (Error) |
| Name | Temperature Sensor n where n represents the number of the temperature sensor instance(s). |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_NumericSensor |
| DeviceID | Unique identifier for this temperature sensor instance. |
| SystemCreationClassName | HP_ComputerSystem.CreationClassName |
| SystemName | HP_ComputerSystem.Name |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_Sensor |  |
| SensorType | 2 (Temperature) |
| CurrentState | Normal – if OperationalStatus[0] is 2 (OK); Critical – if OperationalStatus[0] is 6 (Error); |
| PossibleStates | Array can contain any of the following values: PossibleStates[0] = Normal; PossibleStates[1] = Critical; |
| CIM_NumericSensor |  |
| CurrentReading | Current reading of temperature sensor |
| BaseUnits | 2 (Degrees C) |
| RateUnits | 0 (None) |
| UnitModifier | 0 |
| SupportedThresholds | SupportedThresholds[0] = 1 (UpperThresholdNotCritical) if the system will set fans to full speed if the threshold is crossed; SupportedThresholds[0] = 3 (UpperThresholdCritical) set if the system will gracefully attempt to shutdown the system if the threshold is crossed; SupportedThresholds[0] = 5 (UpperThresholdFatal) if the system will immediately power down the system if the threshold is crossed. |
| SettableThresholds | empty |
| UpperThresholdNonCritical | Contains the threshold temperature that CurrentReading must exceed to be in a non-critical state. This is only set if the system will set fans to full speed if the threshold is crossed. |
| UpperThresholdCritical | Contains the threshold temperature that CurrentReading must exceed to be in a critical state. This is only set if the system will gracefully attempt to shutdown the system if the threshold is crossed. |
| UpperThresholdFatal | Contains the threshold temperature that CurrentReading must exceed to be fatal. This is only set if the system will immediately power down the system if the threshold is crossed. |
| HP_NumericSensor |  |
| NumericSensorType | Enumeration describing the type of temperature sensor. Refer to the Managed Object Format. |

### SMX_SensorCollection Class

The SMX_SensorCollection class implements the HP_SensorCollection and is used to model temperature sensor collections on servers. There is one instance of this class on systems modeling sensors.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Temperature Sensor Collection |
| Description | This is a collection of temperature sensors. GroupStatusDescription. Where: GroupStatusDescription represents the value of the GroupStatusDescriptions[0] property. |
| ElementName | Temperature Sensor Collection |
| CIM_SystemSpecificCollection |  |
| InstanceID | Unique identifier for this sensor collection instance. |
| HP_GroupSystemSpecificCollection |  |
| GroupOperationalStatus[0] | Represents the most severe sensor group operational status: 2 (OK) – if temperatures reported by the sensors are within normal operating range; 2 (OK) – if temperatures reported by the sensors have exceeded their non-critical threshold and has set fans to full speed; 6 (Error) – if temperatures reported by the sensors have exceeded their non-critical threshold and the system is gracefully shutting down. |
| GroupStatusDescriptions[0] | Temperatures reported by the sensors are operating within normal operating range if temperatures reported by the sensors are within normal operating range; Temperatures reported by the sensors have exceeded their non-critical threshold and the fans have been set to full speed if temperatures reported by the sensors have exceeded their threshold and has set fans to full speed; Temperatures reported by the sensors have exceeded their threshold and the system is gracefully shutting down if temperatures reported by the sensors have exceeded their threshold and the system is gracefully shutting down. |

### SMX_MemberOfSensorCollection Class

The SMX_MemberOfSensorCollection class implements the HP_MemberOfSensorCollection class and is used to represent an association between the SMX_NumericSensor and SMX_SensorCollection classes.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Collection | References SMX_SensorCollection |
| Member | References SMX_NumericSensor |

### SMX_HostedSensorCollection Class

The SMX_HostedSensorCollection class implements the HP_HostedSensorCollection class and is used to represent an association between the SMX_SensorCollection class and the computer system that contains the collection.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | References SMX_ComputerSystem |
| Dependent | References SMX_SensorCollection |

### SMX_SystemSensor Class

The SMX_SystemSensor class implements the HP_SystemSensor class and is used to represent an association between the SMX_NumericSensor class and the computer system that contains the sensor.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | References SMX_ComputerSystem |
| PartComponent | References SMX_NumericSensor |

## Provider Indications

| Indications Generated by the Provider | The following tables describe the SMX WBEM System Temperature Sensor Provider indications that are implemented for HP ProLiant server platforms where available. |
| --- | --- |

### HP_AlertIndication: Temperature Sensor Threshold Exceeded

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication. |
| PerceivedSeverity | 5 (Major) |
| IndicationTime | Time of indication. |
| CIM_AlertIndication |  |
| Description | A temperature sensor has detected that the temperature has exceeded the normal operating range. <SensorCaption> Where: <SensorCaption> is the SMX_NumericSensor.Caption for the sensor that has failed. |
| AlertingManagedElement | WBEM Path of the SMX_NumericSensor instance representing the temperature sensor detecting the exceeded threshold. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 1 |
| EventTime | Time of the event or time of the indication if event time unknown |
| ProviderName | HP Temperature |
| RecommendedActions | Ensure that the system is adequately ventilated. Check for proper room temperature and internal/external airflow. Add or repair fans and air baffling if necessary. |
| SystemName | HP_ComputerSystem.Name |
| SystemCreationClassName | HP_ComputerSystem.CreationClassName |
| HP_AlertIndication |  |
| Summary | Temperature threshold exceeded detected |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 51 (Temperature Unacceptable) |
| ProbableCauseDescription | Temperature Threshold Exceeded Detected |
| ProviderVersion | For example: 2.2.0.0 |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication. |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: major.minor.build |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication. |
| SystemSerialNumber | Serial number of the computer system generating the indication. |
| SystemProductID | Product ID of the computer system generating the indication. |
| SystemModel | Model name of the computer system generating the indication. |
| SystemGUID | Platform GUID of the computer system generating the indication. |
| VariableNames | Contains the temperature sensor threshold property name that sets a value: UpperThresholdNonCritical UpperThresholdCritical UpperThresholdFatal |
| VariableTypes | 9 (sint32) |
| VariableValues | SMX_NumericSensor, UpperThresholdNonCritical, UpperThresholdCritical or UpperThresholdFatal property value that is set for the temperature sensor detecting the exceeded threshold. |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties indicated with (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists. |
| RackUID * | Rack Unique Identifie, if one exists. |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment. |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment. |

### HP_AlertIndication: Temperature Sensor Recovery from Threshold Exceeded

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication. |
| PerceivedSeverity | 2 (Information) |
| IndicationTime | Time of indication. |
| CIM_AlertIndication |  |
| Description | A temperature sensor has detected that the temperature has returned to a normal operating range. <SensorCaption> Where: <SensorCaption> is the SMX_NumericSensor.Caption for the temperature sensor detecting a recovered exceeded threshold. |
| AlertingManagedElement | WBEM Path of SMX__NumericSensor instance representing the temperature sensor detecting a recovered exceeded threshold. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 2 |
| EventTime | Time of the event or time of the indication if event time unknown. |
| ProviderName | HP Temperature |
| RecommendedActions | No action is recommended. |
| SystemName | HP_ComputerSystem.Name |
| SystemCreationClassName | HP_ComputerSystem.CreationClassName |
| HP_AlertIndication |  |
| Summary | Threshold exceeded condition recovery |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 52 (Threshold Crossed) |
| ProbableCauseDescription | Threshold Exceeded Condition has Recovered |
| ProviderVersion | For example: 2.2.0.0 |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication. |
| OSType | SMX_OperatingSystem.OSType |
| OSVersion | The operating system version of the computer system generating the indication in the following format: major.minor.build |
| SystemFirmwareVersion | Firmware version of the computer system generating the indication. |
| SystemSerialNumber | Serial number of the computer system generating the indication. |
| SystemProductID | Product ID of the computer system generating the indication. |
| SystemModel | Model name of the computer system generating the indication. |
| SystemGUID | Platform GUID of the computer system generating the indication. |
| ImpactedDomain | 3 (Enclosure) for c-Class blade systems 4 (System) for all other systems Note: When ImpactedDomain is 3 (Enclosure) the properties indicated with (*) below will be populated. |
| BladeBay * | HP_BladeCSLocation.LocationInformation[0] |
| BladeName * | HP_ComputerSystem.Name |
| EnclosureName * | HP_BladeEnclosureCS.Name |
| RackName * | Rack name, if one exists. |
| RackUID * | Rack Unique Identifier, if one exists. |
| SystemVirtualSerialNumber | Conditional property containing the virtual system serial number string when running in a Virtual Connect environment. |
| SystemVirtualUUID | Conditional property containing the virtual system UUID when running in a Virtual Connect environment. |