
Network Video Alarm Panel

 Quick Start Guide 
Version 1.0.0
Table of Contents

11
FEATURES AND SPECIFICATIONS


11.1
Overview


11.2
Features


22
INSTALLATION


22.1
Project Installation


22.2
Tamper Switch Installation


43
STRUCTURE


43.1
Mainboard


53.2
Connection Sample


84
OVERVIEW OF NAVIGATION AND CONTROLS


84.1
Login & Logout


94.2
Network


114.3
Zone


184.4
Remote


194.5
Abnormality


204.6
Arm/Disarm


214.7
Bell Output


214.8
Zone Status


224.9
Broadcast


234.10
MCU


245
WEB OPERATION


245.1
Network Connection


245.2
Login


255.3
LAN Mode


265.4
WAN Login


28Appendix Toxic or Hazardous Materials or Elements




Welcome
Thank you for purchasing our network video alarm panel!

This quick start guide will help you become familiar with our alarm panel in a very short time.

Before installation and operation, please read the following safeguard and warning carefully!

Important Safeguard and Warning
1．Electrical safety

All installation and operation here should conform to your local electrical safety codes. 

The product must be grounded to reduce the risk of electric shock.
We assume no liability or responsibility for all the fires or electric shock caused by improper handling or installation. 

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Keep upwards. Handle with care. 

Do not apply power to the alarm panel before completing installation. 

Do not place objects on the alarm panel.
4．Qualified engineers needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

The alarm panel should be installed in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

This series product shall be transported, storage and used in the environment ranging from -10℃ to 55 ℃.
6.  Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components are included.

Contact your local retailer ASAP if something is broken in your package.
1 FEATURES AND SPECIFICATIONS

1.1 Overview

This series product is an alarm panel of multiple functions. It integrates the on-off alarm signal input and output, and video process. It is mainly used in the bank, school, market and residential district and etc. 
1.2 Features
This series product has the following features:

· Support IP network, 3G network, upload alarm information via PSTN network 

· Support arm/disarm settings, support 8-channel alarm input and 4-channel alarm output at the same time. 

· Support 4-channel video input and 1-channel video output, 4-channel audio input and 1-channel audio output. 

· Support 1-channel VGA output, 1-channel HDMI video output, and 1-channel BNC video output. 

· Support analog video input and network video input. 

· Support SATA HDD, support video local storage and playback. 

· Support 1-channel bidirectional talk and broadcast. 

· Support wireless module. 

2 INSTALLATION 
2.1 Project Installation 

This series product supports wall mount installation. Please refer to the steps listed below. 

a) Open the device package and then take out the plastic expansion bolts and self-tapping screw. 

b) Please refer to Figure 2‑1 to dig five holes in the wall. 

c) Input four expansion bolts and then insert four self-tapping screws. DO NOT INSERT THE BOLT TO THE MIDDLE HOLE. 

d) Put the device on the screws and then insert the last self-tapping screw at the middle hole. 

[image: image1.png]245.8

200. 0

270.0

214.5

163. 0





Figure 2‑1
2.2 Tamper Switch Installation 
Before the installation, please prepare the accessories. It includes: tamper switch, three screws and three nuts. 

Open the cover of the device; secure the tamper switch on the device. See Figure 2‑2.

[image: image2.jpg]



Figure 2‑2
Insert the tamper switch to the main board. See Figure 2‑3.

[image: image3.png]04460
94V-0



  [image: image4.png]



Figure 2‑3
3 STRUCTURE 
3.1 Mainboard 

The interface is shown as in Figure 3‑1.
[image: image5.png]23 2 2‘II 20 19 18

ard Slot | N

T — eoal ] wonna| [ — 17
3G Card Slot i KO 0S8 VGA - P e 3 T
S wera| | — 16

WQMMUU

fllqauuoyfl @Eﬁsl’mtdownt?ed;v‘ue and unplug T — Pover suppling e 15
e power cable before you operate! here shaibe operated by the
qualified service engineer! Improper
1 2 3 cable connection may resultin 100 poverspptigron | © || —— 4 4
it Input Poit d‘m — system malfunction or damage!
4 utpu Pon -

R

Y — 13
| Audio Output Port

SATAHDDPort

CU(‘

[—]

C\. e o
= TEoSReYSeEReg B g Za=o
== .\f\m
NE AN 3/\ TTTTTT TTT hiahdhdhd TT habdhdhd habdbdbi imididid

Video QQutput Port ideo Input Port All

1 2 AN

1l
LWL LL LL o 1
| E¢

EOLare 2. ZR )hm Siten Siren Yren Siren Sifen

4 5 6 7 8 9 10 1112





Figure 3‑1
Please refer to the following sheet for detailed information.
	SN
	Name 
	Function 

	1
	Audio input port 
	4-channel audio input port 

	2
	Bidirectional talk output port 
	Bidirectional talk 

	3
	Audio output port 
	BNC audio output. Bidirectional talk output 

	4
	Video output port 
	BNC video output 

	5
	Video input port 
	4-channel video input 

	6
	LAN
	Ethernet port 

	7
	Alarm input port
	8-channel alarm input. 

G: the public end. 

Z1-Z8: Protection zone input.

	8
	Alarm output port 
	4-channel alarm output.

	9
	RS485 port 
	A1B1: Connect to the PTZ. 

A2B2: Connect to protection zone extension module. 

A3B3 +12V G: Connect to alarm programming keyboard. 

	10
	Siren port 
	Connect to siren.

	11
	Accumulator 
	Lead-acid battery 12V DC 7AH.

	12
	User cable port 

Phone port 
	Connect to user cable or phone. 

	13
	SATA HDD port 
	Support SATA HDD

	14
	HDD power supplying port 
	Provide power to the HDD. 

	15
	Power supplying port 
	Power supplying. 

	16
	User cable port 
	RJ11 user cable port. 

	17
	Phone port 
	RJ11 phone port. 

	18
	Wireless alarm module
	Connect to wireless alarm module. 

	19
	VGA port 
	Connect to VGA displayer. 

	20
	USB port 
	Connect to USB mouse. 

	21
	HDMI port
	HDMI port. Connect to the LCD and etc.

	22
	SIM card slot 
	Support SIM card

	23
	3G card slot
	Support 3G module. 


3.2 Connection Sample
Please refer to the following figures for detailed information. See Figure 3‑2 and Figure 3‑3.

[image: image6.png]Network Video Alarm Panel

FOMI USB

3G Card Slot

‘Caution! Plasa shu dor

SIM Card Skt

S—

o

the povve cable before ot

AudiolnputPort  Bidirectional Talk

e
OOOOBG

DOOOO

(2
(o5 —udio Ouputort

‘Warning! All e csbl
here el

Fonasgsrng o

HOD P

=3 X

HMT i

Tz 3 7

H"{

Viden Output Port  Video Input Port

1 1
AL LLTLLTT,

AIEOLare 2.2K Ohm,

- ’





Figure 3‑2
[image: image7.png]HDMI Output VG Outpu‘wircless Alarm
3G Card Slot  SIM Card Slot USB Port Ma‘dule
L [ P gjﬂ !@ \\’Arelesl Alarm @)‘ Ll j:] —User Cable Port
3G Card Slot HOMI LS5 VGA t Medile J
R Phone Por ::]7 Phone Port
' “— — Power Supplying Port
Caution! Please shut down the devide and unplug Wezning! Al the cable connections Poversuppiying Por ||
; the power cable before you operate! here shall be operated by the .
Audio Output qualified service engineer! Improper B
cable connection may resultin yop ey | © || DD Power
system malfunction or damage! o | Supplying Port
g a0 S o
Bidirectional 4 2 3 4 =
TalkOutput |~ N 2N (O (D (D (D iR E, SATA HDD Port
( (o) o)(¢) (e > i
g NZANANAY \/Q Satnazfaen.s 85888805z atn 2nT.B.B. 2000 I
3 ( H.‘C.\G /9@ g j
\I/\\/ = W M DA S A b DDA TR b bR I b Us‘lerCable
T k| FowLL L L] = ) “f————— Phone Cable

!

FrocetonZone o Siren Lead-Acid Battery

Extension Modul

If ‘\/—\‘

Video Output Video Input 4 Alarm Tnput Alarm Output RS485 Programming
Network Switch B1Z Keyboard
Control

Online User Onlinie User Network Keyboard

Online User




Figure 3‑3
3.2.1 Detector Cable Connection 
Please refer to the following figure for detailed information. See Figure 3‑4.

[image: image8.png]Alarm Alarm

Peripheral Detectors Peripheral Detectors

Panel Panel
CCT T T T T, [
| | : |
z N | i £ NO 1
12.2kQ [ ! I
! | I 2.2kQ |
| | ! |
@ I ] : 6l— G !
| | ! |
| | ! |
| | | |





Figure 3‑4
3.2.2 Alarm Output Connection 

Please refer to the following figure for detailed information. See Figure 3‑5.

[image: image9.png]DC 12V

NO/NC

Circuit Breaker
COM Peripheral Devices

O ®

Peripheral DC Load





Figure 3‑5
4 OVERVIEW OF NAVIGATION AND CONTROLS 
Before operation, please make sure:

· You have properly installed HDD and connect all the necessary cables. 

· The provided input power and the device power are matched.

· Always use the stable current, if necessary UPS is a best alternative measure.  

4.1  Login & Logout 

4.1.1 Login 
After system booted up, system pops up the startup wizard. 

Click the Cancel button; you can go to the system login interface. 

Click the Next Step button; you can go to the startup wizard interface. Here you can set the system basic information. See Figure 4‑1.

[image: image10.png]Startup Wizard

Startup Wizard:General, Encode, Schedule,
Record Control, Network.

B Startup





Figure 4‑1
The system login interface is shown as in Figure 4‑2. 

System consists of four accounts: 
· Username: admin.  Password: admin. (administrator, local and network)

· Username: 888888. Password: 888888. (administrator, local only)

· Username: 666666. Password: 666666(Lower authority user who can only monitor, playback, backup and etc.)

· Username: default. Password: default(hidden user)

You can use USB mouse, front panel, remote control (not included in the accessory bag) or keyboard to input. About input method: Click [image: image11.png]


 to switch between numeral, English character (small/capitalized) and denotation.

Note: 
For security reason, please modify password after you first login. 
Continuous three times login failure will result in system alarm and six times login failure will result in account lock!

[image: image12.jpg]SYSTEM LOGIN

User Name (888888 5

Password 123}

3
6
9 ncel
=





Figure 4‑2
4.1.2 Logout 
In the main menu, click shutdown button, you can see an interface is shown as below. See Figure 4‑3.

[image: image13.jpg]UTDOWN

‘Logou( menu user
Password is needed to re-enter the menu
| after logout.

OK ] | Cancel |





Figure 4‑3
There are several options for you. See Figure 4‑4.

[image: image14.png]Logout menu user
Shutdown

Restart system
Switch user





Figure 4‑4
4.2 Network 
From main menu->System->Network, you can go to the network interface. Here is for you to input network information.

The IPv4 interface is shown as in Figure 4‑5 and the IPv6 interface is shown as in Figure 4‑6.

· IP Version: There are two options: IPv4 and IPv6. Right now, system supports these two IP address format and you can access via them. 

· MAC address: The host in the LAN can get a unique MAC address. It is for you to access in the LAN. It is read-only. 

· IP address: Here you can use up/down button ((() or input the corresponding number to input IP address. Then you can set the corresponding subnet mask the default gateway. 

· Subnet prefix: The input value ranges from 0 to 128. It is to mark a specified network MAC address. Usually it includes an organization of multiple-level. 

· Default gateway: Here you can input the default gateway. Please note system needs to check the validity of all IPv6 addresses. The IP address and the default gateway shall be in the same IP section. That is to say, the specified length of the subnet prefix shall have the same string. 

· DHCP: It is to auto search IP. When enable DHCP function, you can not modify IP/Subnet mask /Gateway. These values are from DHCP function. If you have not enabled DHCP function, IP/Subnet mask/Gateway display as zero. You need to disable DHCP function to view current IP information.  Besides, when PPPoE is operating, you can not modify IP/Subnet mask /Gateway. 
· TCP port: Default value is 37777. You can change if necessary.
· UDP port: Default value is 37778. You can change if necessary.
· HTTP port: Default value is 80.

· RTSP port: Default value is 554.

Important: System needs to reboot after you changed and saved any setup of the above four ports. Please make sure the port values here do not conflict.
· Max connection: system support maximal 128 users. 0 means there is no connection limit.
· MTU: It is to set MTU value of the network adapter. The value ranges from 1280-7200 bytes. The default setup is 1500 bytes. Please note MTU modification may result in network adapter reboot and network becomes off. That is to say, MTU modification can affect current network service. System may pop up dialog box for you to confirm setup when you want to change MTU setup. Click OK button to confirm current reboot, or you can click Cancel button to terminate current modification. Before the modification, you can check the MTU of the gateway; the MTU of the alarm panel shall be the same as or is lower than the MTU of the gateway. In this way, you can reduce packets and enhance network transmission efficiency. 

The following MTU value is for reference only. 

· 1500: Ethernet information packet max value and it is also the default value. It is the typical setup when there is no PPPoE or VPN. It is the default setup of some router, switch or the network adapter. 

· 1492: Recommend value for PPPoE.

· 1468: Recommend value for DHCP.

· Preferred DNS server: DNS server IP address.

· Alternate DNS server: DNS server alternate address.

· Transfer mode: Here you can select the priority between fluency/video qualities.

· LAN download: System can process the downloaded data first if you enable this function. The download speed is 1.5X or 2.0X of the normal speed.
Important

For the IP address of IPv6 version, default gateway, preferred DNS and alternate DNS, the input value shall be 128-digit. It shall not be left in blank.

After completing all the setups please click save button, system goes back to the previous menu.

[image: image15.png]IP Version IPv4

IP Address 10 . 15 - 144 DHCP

Subnet Mask 255 . 255 0

Default Gateway [10 . 15 1

TCP Port 37777 HTTP Port 80

UDP Port 37778 HTTPS PORT (443

RTSP Port 554 MTU 1500

Preferred DNS [ 10 1 - 80

Alternate DNS 10 1 - 81

Max Connection (128 LAN Download





Figure 4‑5
[image: image16.png]IP Version IPvé 9
LinkAddr
IP Address /|64

Default Gateway

TCP Port

HTTP Port 80
HTTPS PORT (443
MTU 1500

UDP Port

RTSP Port

Preferred DNS

Alternate DNS

Max Connection (128 LAN Download

NETWORK SETTING





Figure 4‑6
4.3 Zone 

From main menu->Alarm->Zone, you can go to the zone interface.
It is to set protection zone and its corresponding alarm trigger operation. See Figure 4‑7.
· Zone No. Please select a zone number from the dropdown list. 

· Zone name: Please select a zone name from the dropdown list. Support customized setup. 

· Enable: Check the box here to enable zone function. 

· Type: Click settings button, you can see there are three types: realtime zone (Figure 4‑8), delay zone (Figure 4‑9), 24-H zone (Figure 4‑10). 

· Realtime zone: On the arm status, once the detector connected to the zone is trigged, the alarm controller can generate an alarm immediately. There is no delay time. 

· Delay zone: Please set two parameters: Enter delay and Exit delay.

a) In the entry delay mode, there is no trigger operation after the alarm is trigged for M seconds (M refers to the in delay value you set here). You can disarm during the M seconds. If you do not disarm, the protection zone is activated after M seconds. For example: after you enabled arm status and then set the in delay is 10s. System only enables link alarm after the alarm occurred for 10 seconds no matter the alarm has disappeared or still exists. 

b)  Exit delay: After you enable arm status, the arm operation becomes valid after N seconds (N refers to the exit delay value you set here). During the N seconds period, the alarm link operation is null. For example, after you enable out delay time 15s, there is no alarm link event during the first 15 seconds until the 15-second countdown ends. 

· 24-H zone: The detector of current protection zone is armed for all the 24-H, no matter it is armed or disarmed. But you can arm/disarm manually. There is no arm period setup and the arm button on the front panel is null. You can only set on the menu interface. 
· PTZ activation: When an alarm occurred, system can activate the PTZ operation. The PTZ activation lasts an anti-dither period. 

· In the Pan/Tilt/Zoom interface (Main menu->Setting-> Pan/Tilt/Zoom), please set video channel, speed dome protocol and etc. 

· Select the channel of current speed dome as current monitor video and the right click mouse to select Pan/Tilt/Zoom item. Now you can set preset, tour pattern.

· In Figure 4‑7, click “setup” button, you can see an interface is shown as in Figure 4‑11. Here you can set the activation operation such as preset tour, pattern and enable. 
· Period: Click setup button, you can see an interface is shown as in Figure 4‑12. Here you can set for business day and non-business day. In Figure 4‑12, click set button, you can see an interface is shown as in Figure 4‑13. Here you can set your own setup for business day and non-business day.
· Alarm output: when an alarm occurs, system enables peripheral alarm devices.
· Anti-dither: Here you can set anti-dither time. The value ranges from 5 to 600s. The anti-dither time refers to the alarm signal lasts time. It can be seem as the alarm signal activation stays such as the buzzer, tour, PTZ activation, snapshot, channel record.  The stay time here does not include the latch time. During the alarm process, the alarm signal can begin an anti-dither time if system detects the local alarm again. The screen prompt, alarm upload, email and etc will not be activated. For example, if you set the anti-dither time as 10 second, you can see the each activation may last 10s if the local alarm is activated. During the process, if system detects another local alarm signal at the fifth second, the buzzer, tour, PTZ　activation, snapshot, record channel will begin another 10s while the screen prompt, alarm upload, email will not be activated again. After 10s, if system detects another alarm signal, it can generate an alarm since the anti-dither time is out.  
· Show message: System can pop up a message to alarm you in the local host screen if you enabled this function.
· Alarm upload: System can upload the alarm signal to the network (including alarm centre) if you enabled current function.
· Send email: System can send out email to alert you when an alarm occurs. 

· Record channel: you can select proper channel to record alarm video (Multiple choices). 
· You need to set alarm record mode as Schedule in Record interface (Main Menu->Advanced->Record). Please note the manual record has the highest priority. System record all the time no matter there is an alarm or not if you select Manual mode. 

· Now you can go to the Schedule interface (Main Menu->Setting->Schedule) to set the record type, corresponding channel number, week and date. You can select the record type:Regular/MD/Alarm/MD&Alarm. Please note, you can not select the MD&Alarm and MD(or Alarm) at the same time. 

· Now you can go to the Encode interface to select the alarm record and set the encode parameter (Main Menu->Setting->Encode). 

· Finally, you can set the alarm input as the local alarm and then select the record channel. The select channel begins alarm record when an alarm occurred. Please note system begins the alarm record instead of the MD record if the local alarm and MD event occurred at the same time. 
· Latch: When the anti-dither time ended, the channel alarm you select in the alarm output may last the specified period. The value ranges from 1 to 300 seconds. This function is not for other alarm activation operations. The latch is still valid even you disable the alarm event function directly.  .

· Tour: Here you can enable tour function when an alarm occurs.  System supports one-window tour. Please go to chapter 4.4.2.6 Display of the user’s manual for tour interval setup. 
· Snapshot: You can enable this function to snapshoot image when an alarm occurs.
· Video matrix: Check the box to enable this function. The video matrix output can display the alarm video of current channel when there is an alarm signal. 
· Buzzer: Highlight the icon to enable this function. The buzzer beeps when an alarm occurs.
· Snapshot: For snapshot operation, please refer to chapter 4.3.1 for detailed information.
· Audio broadcast: System triggers audio function when generating an alarm. 

· Output port: It is to set output port: speaker/MIC output port. 
· Siren output control: System triggers siren when an alarm occurs. 

· Call centre: System can upload the alarm to the call centre when an alarm occurs. 
[image: image17.png]No. 1 ~| Name
Enable . Local |~
Type Settings

Period E

W Alarm Out BRRE Ext Alarm Out
@ Show Message [ Alarm Upload [_)Send Email
[WRecord Channel B(2)(3)(2

WPTZ Activaton  (__Setup ) Delay

W Tour B2RE

@ Snapshot BRORE

WBuzzer

B Audio Broadcast [N/A ~| OutputPort

(Bl Output Control
W Call Centre 12

Zone 1

El

Talk Port





Figure 4‑7
[image: image18.png]Type Realtime N

Zone Source Type |Gas Sensor |~





Figure 4‑8
[image: image19.png]Type
Zone Source Type

Entry Delay
Exit Delay

Delay v
Gas Sensor |~/
0 sec.
0 sec.





Figure 4‑9
[image: image20.png]Type 24H 7

Zone Source Type |Gas Sensor |~





Figure 4‑10
[image: image21.png]CAM 1
CAM 3

None

0 CAM 2

None

0 CAM 4

o)

None

None





Figure 4‑11
[image: image22.png]Work Day ~| [ Setwp | x
W (0000 24 :00 0000 24 00
00 :00  -24 :00 00 :00 24 :00
00 :00  -24 :00 00 :00 24 :00
0.3 6 9 12 15 18 21 24
Sun S
Mon I ——
Tue 1 ——
Wed I ——
Thu 1 ——
Fri I ——
Sat ]
0 3 6 9 12 15 18 21 24





Figure 4‑12
[image: image23.png]Sun  Mon Tue Wed Thu Fri Sat
Work Day ® L L L L (o] (o]
FreeDay O O O O O L] L]





Figure 4‑13
4.3.1 Snapshot Operation
4.3.1.1 Schedule Snapshot 

In Encode interface, click snapshot button to select snapshot mode as timing, and then set size, quality and frequency. See Figure 4‑14.

In Schedule interface, please enable snapshot function. See Figure 4‑15.
[image: image43.png]Security Setting:

nternet Zone

Settings

O Disable
© Enable

9] Download signed ActiveX controls (not secure)
O Disable
© Enable (not secure)
O Prompt (recommended)

9] Download unsigned Active controls (not secure)
© Disable (recommended)
© Enable (not secure)
O Prompt

19 Intiaize and scrpt ActiveX contros ok merked as sae for s
O Disable (recommended)
© Enable (not secure)
O Prompt

4] Run ctiveX contros and pug-ins
3 bbb e

< I

*Takes effect after you restart Internet Explorer

Reset custom settings




[image: image24.png]Mode

Image Size
Image Quality

Snapshot Frequency

Timing

CIF

4

1SPL





Figure 4‑14
[image: image25.png]sec. Redundancy(_)Snapshot [l Holiday [l

Channel (1 [~]PreRecord(4

Period  [Thu 3 Record Type  Regular MD
Period 1 [00 :00 24 :00 | ] O
Period 2 [00 :00 24 :00 @] O
Period 3 [00 :00 24 :00 @] O
Period4 [00 :00 24 :00 @] O
Period 5 [00 :00 24 :00 @] O
Period 6 [00 :00 24 :00 @] O

1" Regular I MD || Alarm

MD&Alarm

Alarm
(]
(]
(]
(]
(]
(]

00o0o0ooo

I MD&Alarm

6

9

12

15

18 21 24





Figure 4‑15
4.3.1.2 Activation Snapshot

Please follow the steps listed below to enable the activation snapshot function. After you enabled this function, system can snapshot when the corresponding alarm occurred. 

· In Encode interface, click snapshot button to select snapshot mode as trigger, and then set size, quality and frequency. See Figure 4‑16.
· In Detect interface please enable snapshot function for specified channels. See Figure 4‑17. Or in alarm interface please enable snapshot function for specified channels. See Figure 4‑18.
Please refer to the following figure for detailed information. See Figure 4‑18.
[image: image26.png]Mode

Image Size
Image Quality

Snapshot Frequency

Trigger

CIF

4

1SPL





Figure 4‑16
[image: image27.png]Event Type Motion Detect | ~| Channel 1 S

Enable | ] Sensitivity 3 -

Region Setup
Period Setup Anti-dither 5 sec.

W Alarm Out BRORE Latch 10 sec.
@ Show Message [ Alarm Upload [_)Send Email
[WRecord Channel B(2)(3)(2

WPTZ Activation E Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~

(Bl Output Control

Toa ) (o) (G





Figure 4‑17
[image: image28.png]Event Type NEE ~| Alarm In 1 9
Enable [ ]

Period E

W Alarm Out BRORE Latch 10 sec.
B Show Message Send Email

[WRecord Channel B(2)(3)(2

WPTZ Activaton  (__Setup ) Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~
(Bl Output Control





Figure 4‑18
4.3.1.3 Priority

Please note the activation snapshot has the higher priority than schedule snapshot. If you have enabled these two types at the same time, system can activate the activation snapshot when an alarm occurs, and otherwise system just operates the schedule snapshot. 

4.4 Remote 
From main menu->Alarm->Remote, you can go to the alarm remote interface. 
The alarm remote interface is shown as below. See Figure 4‑19.

· Wireless ID: It refers to the wireless ID of the remote control. 

· User name: It is the user name of the remote control. 

· Modify: It is to change remote control status. Y means current remote control is valid. N means it is null now. 

· Delete: It is to delete the registered remote control. 

· Status: It is to display remote control status. Y means current remote control is valid. N means it is null now. 

Click wireless code button at the bottom of the interface to search remote controls. Follow the dialogue box to click any button of the remote control to connect. After successful connection, you can change the registered remote control information. 

[image: image29.png]0 |WirelessID UserName Modify Delete Status

Wireless Code





Figure 4‑19
4.5 Abnormality 
From main menu->Alarm->Abnormality, you can go to the abnormality interface. 
Abnormality interface is shown as in Figure 4‑20.

· Event type: There are several options for you such as disk error, no disk, disconnection, IP conflict, MAC conflict and etc. 
· Less than: Please input HDD space threshold value here (For disk no space type only). 
· Alarm output: Please select alarm activation output port (multiple choices). 

· Latch: Here you can set corresponding delaying time. The value ranges from 10s-300s. System automatically delays specified seconds in turning off alarm and activated output after external alarm cancelled.
· Show message: system can pop up the message in the local screen to alert you when an alarm occurs. 

· Alarm upload: System can upload the alarm signal to the network (including alarm centre) if you enabled current function.

· Send email: System can send out email to alert you when an alarm occurs. 

· Buzzer: Highlight the icon to enable this function. The buzzer beeps when an alarm occurs.
· Audio broadcast: System triggers audio function when generating an alarm. 

· Output port: It is to set output port: speaker/MIC output port. 
· Siren output control: System triggers siren when an alarm occurs. 

[image: image30.png]Event Type No Disk 9
Enable
W Alarm Out BRORE Latch

@ Show Message [ Alarm Upload [_)Send Email

WBuzzer

B Audio Broadcast [N/A [+] Output Port

(Bl Output Control

sec.

Talk Port |

Save Cancel

El





Figure 4‑20
[image: image31.png]WBuzzer

Event Type Disk No Space |
Enable

Less Than 20 %
W Alarm Out BRORE Latch

@ Show Message [ Alarm Upload [_)Send Email

B Audio Broadcast [N/A [+] Output Port

(Bl Output Control

sec.

Talk Port |

Save Cancel

El





Figure 4‑21
4.6 Arm/Disarm 
From main menu->Alarm->Arm/Disarm, you can go to the arm/disarm interface. 

The arm/disarm interface is shown as below. See Figure 4‑22. Here you can arm or disarm a channel/all channels. 

· Arm all: Check the box here to arm all protection zones. 

· Disarm all: Check the box here to disarm all protection zones. `
· Normal/bypass: Check the box, the status is shown as normal. Cancel the box, the status is shown as bypass. 

[image: image32.png]® Arm Al O Disarm All

v/ Status No. Name Type Entry Delay Exit Delay
v/ Normal 1 Zone1 Realtime - =
v|Normal 2 Zone2 Realtime - =
v|Normal 3 Zone3 Realtime - =
v|Normal 4 Zone4 Realtime - =
v|Normal 5 Zone5 Realime - =
v|Normal 6 Zone6 Realime - =
v|Normal 7 Zone7 Realtime - =
v|Normal 8 Zone8 Realtime - =





Figure 4‑22
4.7 Bell Output
From main menu->Alarm->Bell output, you can go to the siren output interface. 

Here is for you to set siren output. See Figure 4‑23.
· Bell output: Select siren output channel. 

· Alarm switch: Check the box here to enable alarm output function.

· Status: The corresponding channel is highlighted if there is any alarm. 
[image: image33.png]BELL OUTPUT All 1

Alarm Switch

Status

( Enable | Stop ) [ Cancel |





Figure 4‑23
4.8 Zone Status 
From main menu->Alarm->Zone Status, you can go to the zone status interface. 

Here is for you to view protection zone status. See Figure 4‑24.
· For the first dropdown list, you can select module. It includes local module, extension module and all.

· For the second dropdown list, you can select corresponding channel. 

· Confirm: Select a channel and then click it to cancel an alarm. 
[image: image34.png]Index 12345¢86738
Local Module SEEEEEEm
Ext Module1
Ext Module2
Ext Module3
Ext Module4
Ext Module5
Ext Module6
Ext Module7
Ext Module8

Local Module | ~| [1 ~| [ Confirm |





Figure 4‑24
4.9 Broadcast 
From main menu->Video->Broadcast, you can go to the broadcast interface. 

The broadcast interface is shown as in Figure 4‑25. Here you can set audio file volume. 

[image: image35.png]File Name Size Audition Delete

Left Space/Total Space 8.00 MB/ 8.00 MB Volume el

Schedule Playback





Figure 4‑25
Click Schedule playback button, you can see an interface shown as below. See Figure 4‑26.

Here you can set schedule playback period and file. 

[image: image36.png]Period Audio Broadcast Interval Output Port
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/





Figure 4‑26
4.10  MCU 
From main menu->Info->MCU, you can go to the MCU information interface. 

Here is for you to view micro chip unit information. See Figure 4‑27.

[image: image37.png]MCU Software Version V/1.000.0000.0
Online Alarm Keyboard No. 0





Figure 4‑27
5 WEB OPERATION 
5.1  Network Connection 

Before web operation, please check the following items:

· Network connection is right

· Alarm panel and PC network setup is right. Please refer to network setup(Setup ->Network)
· Use order ping ***.***.***.***(* alarm panel IP address) to check connection is OK or not. Usually the return TTL value should be less than 255.
· Open the IE and then input alarm panel IP address.

· System can automatically download latest web control and the new version can overwrite the previous one.

· If you want to un-install the web control, please run uninstall webrec3.0.bat. Or you can go to C:\Program Files\webrec to remove single folder. Please note, before you un-install, please close all web pages, otherwise the un-installation might result in error.
· Current series product supports various browsers such as Safari, firebox browser, Google browser. Device only support 1-channel monitor on the Apple PC. 
5.2 Login 

Open IE and input alarm panel address in the address column. For example, if your alarm panel IP is 10.10.3.16, then please input http:// 10.10.3.16 in IE address column. See Figure 5‑1.
[image: image38.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 5‑1
System pops up warning information to ask you whether install webrec.cab control or not. Please click yes button.
If you can’t download the ActiveX file, please modify your settings as follows. See Figure 5‑2.
[image: image39.png]Internet Options

General | Security |Privacy || Content | Connections | Programs | Advanced |

Select a zone to view or change security sektings.

@ ) v e

oot oca Tustedstes ¥
< ] 8
eret =
‘This zone is for Internet websites, =5
ks e ratios o
e

Security level for tis zone
Allowed levels For this zone: Medum ta Hih

Medium-high
- Appropriate for most websites

- Prompts before dowrloading potentialy unsafe
cantent.

~Unsigned Actve controls il not be downloaded

Custom level Default fevel

Reset all zones to defaul evel

m





Figure 5‑2
After installation, the interface is shown as below. See Figure 5‑3.
Please input your user name and password. 
Default factory name is admin and password is admin. 
Then you can select the login mode: LAN and WAN. 
Note: For security reasons, please modify your password after you first login.
[image: image40.png]



Figure 5‑3
5.3 LAN Mode 

For the LAN mode, after you logged in, you can see the main window.  See Figure 5‑4. 
[image: image41.png]WEB szve: T m Tl TR

4 CAM 1

gaona

Exzrmn
[T T
A d

DEEOEEEEEHEZ =

Copyright 2013,All Rights Reserved.





Figure 5‑4
5.4 WAN Login 

In WAN mode, after you logged in, the interface is shown as below. See Figure 5‑5.
[image: image42.png]WEB scrvee . [T ST SN ST ST

1-69Kbps-M QaGo
=acami L
oz 2013-12-30 15:98:02

Copyright 2013,All Rights Reserved.





Figure 5‑5
For detailed operation information, please refer to the User’s Manual included in the resources CD. 

Appendix Toxic or Hazardous Materials or Elements
	Component Name
	Toxic or Hazardous Materials or Elements

	
	Pb
	Hg
	Cd
	Cr VI
	PBB
	PBDE

	Sheet Metal(Case)
	○
	○
	○
	○
	○
	○

	Plastic Parts（Panel）
	○
	○
	○
	○
	○
	○

	Circuit Board
	○
	○
	○
	○
	○
	○

	Fastener
	○
	○
	○
	○
	○
	○

	Wire and Cable/AC Adapter
	○
	○
	○
	○
	○
	○

	Packing Material
	○
	○
	○
	○
	○
	○

	Accessories
	○
	○
	○
	○
	○
	○


Note

O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.
X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. During the environmental-friendly use period (EFUP) period, the toxic or hazardous substance or elements contained in products will not leak or mutate so that the use of these (substances or elements) will not result in any severe environmental pollution, any bodily injury or damage to any assets.  The consumer is not authorized to process such kind of substances or elements, please return to the corresponding local authorities to process according to your local government statutes 

Note

· For detailed operation introduction, please refer to our resource CD included in your package for electronic version of the User’s Manual.
· Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice.
· All trademarks and registered trademarks mentioned are the properties of their respective owners.
· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website for more information.

















































































































































































































































































































































































Input your IP address here.














PAGE  
13

