
Network Video Alarm Panel

User’s Manual
                                                                                                              Version 1.0.0
Welcome
Thank you for purchasing our network video alarm panel!

This user’s manual is designed to be a reference tool for the installation and operation of your system. 

Here you can find information about this series alarm panel features and functions.

Before installation and operation please read the following safeguards and warnings carefully!

Important Safeguards and Warnings 

1．Electrical safety

All installation and operation here should conform to your local electrical safety codes. 

The product must be grounded to reduce the risk of electric shock.
We assume no liability or responsibility for all the fires or electric shock caused by improper handling or installation. 

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Keep upwards. Handle with care. 

Do not apply power to the alarm panel before completing installation. 

Do not place objects on the alarm panel.
4．Qualified engineers needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

The alarm panel should be installed in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

This series product shall be transported, storage and used in the environment ranging from -10℃ to 55 ℃.
6.  Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components are included.

Contact your local retailer ASAP if something is broken in your package.
Table of Contents

11
FEATURES AND SPECIFICATIONS


11.1
Overview


11.2
Features


11.3
Specifications


52
Installation


52.1
Project Installation


52.2
Tamper Switch Installation


73
Structure


73.1
Mainboard


83.2
Connection Sample


93.2.1
Detectors Cable Connection


93.2.2
Alarm Output Connection


114
Overview of Navigation and Controls


114.1
Login, Logout & Main Menu


114.1.1
Login


124.1.2
Logout


124.2
Live Viewing


144.3
Right-Click Menu


144.3.1
Window Switch


144.3.2
PTZ Control


194.3.3
Color


194.3.4
Alarm Clear


194.3.5
Arm/Disarm


194.3.6
Search


204.3.7
Record Control


204.3.8
Alarm Output


204.3.9
Main Menu


204.4
Main Menu


204.4.1
Alarm


214.4.1.1
Zone


234.4.1.2
Alarm


264.4.1.3
Arm/Disarm


274.4.1.4
Profile


274.4.1.5
Remote


284.4.1.6
Call Centre


294.4.1.7
Alarm Output


304.4.1.8
Bell Output


304.4.1.9
Abnormality


324.4.1.10
Zone Status


324.4.2
Video


334.4.2.1
Encode


354.4.2.2
Detect


404.4.2.3
Search


474.4.2.4
Schedule


524.4.2.5
Record


554.4.2.6
Display


584.4.2.7
Broadcast


594.4.2.8
D/A Switch


594.4.3
Information


604.4.3.1
HDD


614.4.3.2
BPS


614.4.3.3
Log


624.4.3.4
Version


634.4.3.5
Online Users


644.4.3.6
Network


654.4.3.7
MCU


654.4.4
System


664.4.4.1
General


694.4.4.2
Network


874.4.4.3
Account


904.4.4.4
HDD Manager


944.4.4.5
RS485


954.4.4.6
Auto Maintenance


954.4.4.7
Backup


974.4.4.8
Config Backup


984.4.4.9
Text Overlay


1014.4.4.10
Default


1014.4.4.11
Shutdown


1035
WEB OPERATION


1035.1
Network Connection


1035.2
Login


1045.3
LAN Mode


1055.4
WAN Login


106Appendix H Toxic or Hazardous Materials or Elements




1 FEATURES AND SPECIFICATIONS
1.1 Overview

This series product is an alarm panel of multiple functions. It integrates the on-off alarm signal input and output, and video process. It is mainly used in the bank, school, market and residential district and etc. 
1.2 Features
This series product has the following features:
· Support IP network, 3G network, upload alarm information via PSTN network 

· Support arm/disarm settings, support 8-channel alarm input and 4-channel alarm output at the same time. 

· Support 4-channel video input and 1-channel video output, 4-channel audio input and 1-channel audio output. 

· Support 1-channel VGA output, 1-channel HDMI video output, and 1-channel BNC video output. 

· Support analog video input and IP video input. 

· Support SATA HDD, support video local storage and playback. 

· Support 1-channel bidirectional talk and broadcast. 

· Support wireless module. 

1.3 Specifications
	
	Parameter
	Wireless Module Series 
	Wireless Module+3G Module Series 

	System
	Main Processor
	High-performance industrial embedded micro controller

	
	OS
	Embedded LINUX

	
	System Resources
	Multiplex operations: Multiple-channel record, multiple-channel  playback and network operation simultaneously

	
	Interface
	User-friendly  graphical user interface  

	
	Input Devices
	USB mouse, special keyboard

	
	Input Method
	Arabic number, English  character, donation and extension Chinese (optional)

	
	Shortcut Function 
	Copy/paste operation, USB mouse right-key shortcut menu, double click USB mouse to switch screen.

	Alarm 
	Alamr Input 
	8-ch alarm input（NO/NC. ）
Alarm can activate local alarm output or extension alarm output, screen message prompt, alarm output, email, record channel, PTZ and etc. .

	
	Alarm Output
	4-channel relay output. (24VDC, 2A)                                                                                                                                                                                                                                                                                                                       

	
	Alarm Input/Output  Extension 
	64-channel extension alarm input/extension alarm output module
BUS extension alarm module

	
	Manual Alarm Control 
	Enable or disable alarm input channel

Simulate alarm signal to specific alarm output channel.

	
	Wireless Remote Control 
	433MHz wireless remote to arm/disarm/home/out mode.

	
	Programming Keyboard 
	Support programming keyboard to arm/disarm/search/cancel alarm function and etc. 

	Compression 

Standard 
	Video Compression
	H.264

	
	Audio Compression
	G.711A

	Video monitor 
	Video Input
	4-channel composite video input: (NTSC/PAL) BNC (1.0VBP- P, B75Ω) or 
3-channel composite video input: (NTSC/PAL) BNC (1.0VBP- P, B75Ω)+1-channel 720P digital channel or 
2-channel composite video input: (NTSC/PAL) BNC (1.0VBP- P, B75Ω) +2-channel 720P digital channel.
The digital channel max supports 1080P.

	
	Video Output
	1-channel PAL/NTSC, BNC (1.0VP- P, 75Ω) composite video signal output.  

1-channel VGA output.

1-channel HDMI output.

	
	Video Standard
	Support PAL/NTSC.

	
	Record Speed
	Real-time Mode: PAL 1f/s to 25f/s per channel and NTSC 1f/s to 30f/s per channel

	
	Video Partition
	1/4 windows(Optional)

	
	Monitor Touring 
	Support monitor tour functions such as alarm, motion detection, and schedule auto control. 

	
	Resolution

(PAL/NTSC)
	PAL(700TVL, 50f/s)/NTSC(700TVL, 60f/s)
Real-time monitor：

960H    960 ×576/960×480

	
	
	Playback: 
960H  960×576/960×480,

D1   704×576/704×480, 

HD1  352×576/352×480, 

2CIF  704×288/704×240, 

CIF   352×288/ 352×240, 

QCIF  176×144/176×120

	
	
	Support dual streams.

Extra stream resolution: 

CIF  352×288/ 352×240

QCIF 176×144/176×120   

	
	Image Quality
	6-level image quality (Adjustable)

	
	Privacy mask
	Support one privacy mask of user-defined size in full screen.  

Support max 4 zones.

	
	Image Information
	Channel information, time information and privacy mask zone.

	
	TV Adjust
	Adjust TV output zone suitable to anamorphic video.

	
	Channel Lock 
	Cover secret channel with black screen though system is encoding normally.

Screen-lock function to prevent unauthorized user seeing secret video.

	
	Channel Information
	Channel name, recording status, screen lock status, video loss status and motion detection status are shown on the bottom left of display screen.

	
	Color Configuration
	Hue, brightness, contrast, saturation and gain setup for each channel.

	Audio 
	Audio Input
	4-channel 200-2000mv 10KΩ(BNC)

	
	Audio Output
	1-channel audio output  200-3000mv  5KΩ(BNC)

	
	Bidirectional Audio 
	1-channel audio talk input 200-3000mv  10KΩ（BNC）

1-channel audio talk output 200-3000mv  5KΩ(BNC)

	Hard disk
	Hard Disk
	1 built-in SATA port. Support 1 HDD.

	
	Hard Disk Occupation
	Audio：PCM　28.8MByte/h

Video：56-900MByte/h

	Record and playback
	Recording Mode
	Manual recording, motion detection recording, schedule recording and alarm recording

Priority: Manual recording> alarm recording>motion detection recording>schedule recording.

	
	Recording Length
	1 to 120 minutes single record duration (Default setup is 60 minutes)

	
	Playback Repeat Way


	When hard disk is full, system can overwrite previous video file.

	
	Record Search
	Various search engines such as time, type and channel.

	
	Playback Mode 
	Various fast play, slow play speeds, manual frame by frame playback and reverse play mode.

	
	Various File Switch Ways 
	Can switch to previous or next file or any file in current play list.

Can switch to file on other channel of the same time. (If there is a file)

Support file continuous play, when a file is end system auto plays the next file in the current channel 

	
	Multi-channel Playback
	There is 1/4-channel playback mode. 


	
	Window  Zoom 
	Switch between self-adaptive screen/full screen when playback

	
	Partial Enlargement
	When in one-window full-screen playback mode, you can select any zone to activate partial enlargement function.

	Backup function 
	Backup Mode 
	HDD backup

	
	
	Support peripheral USB backup device. (Flash disk, portable disk, USB burner and etc.)

	
	
	Support peripheral eSATA backup device. 

	
	
	Support network download and save

	Network Function 
	Network control
	View monitor channel remotely.

	
	
	Alarm panel configuration through client-end and web browser

	
	
	Upgrade via client or browser to realize remote maintenance.

	
	
	View alarm information such as external alarm, motion detection and video loss via client.

	
	
	Support network PTZ lens control 

	
	
	File download backup and playback

	
	
	Multiple devices share information via corresponding software such as professional surveillance software (PSS) 

	
	
	Duplex transparent COM

	
	
	Network alarm input and output

	
	
	Bidirectional audio.

	
	3G Network 
	Support CDMA/3G network 

	Motion Detection and Alarm 
	Motion Detection
	Zone setup: support 396/330((PAL 22×18, NTSC 22×15)) detection zones. 
Various sensitivity levels. 
Alarm can activate record or external alarm or screen message prompt. 

	
	Video Loss 
	Alarm can activate external alarm or screen message prompt.

	
	Camera masking 
	Alarm can activate external alarm or screen message prompt.

	Interface
	USB Interface
	1 USB 2.0 port.

	
	Network connection
	1 RJ45 10M/100M self-adaptable Ethernet port 

	
	RS485
	3 RS485 ports 

A1B1:PTZ control port. Support various protocols.

A2B2；Extension module port. Support DH-Alarm protocol.

A3B3: Programming keyboard port. Support DH-Alarm protocol.

	
	Phone port 
	1 RJ11 port. 

	
	DC +12V
	1-channel +12V DC

	System

Information 
	Hard Disk Information 
	Display HDD current status

	
	Data Stream Statistics
	Data stream statistics for each channel (in wave mode)



	
	Log statistics
	Backup to 1024 log files. 

Support various search engines such as time and type.

	
	Version 
	Display version information: channel amount, alarm input and output amount, system version and release date.

	
	On-line user 
	Display current on-line user

	User Management 
	User Management 
	Multi-lever user management; various management modes

Integrated management for local user, serial port user and network user.

Configurable user power.

	
	
	Support user /group and its corresponding rights modification.

No limit to the user or group amount.

	
	Password Authentication
	Password modification 

Administrator can modify other user’s password.

	
	
	Account lock strategy 

Five times login failure in thirty minutes may result in account lock.

	Upgrade 
	Web browser, client-end and update tool.

	Login, Logout and Shutdown 
	Password login protection to guarantee safety 

	
	User-friendly interface when login. Provide the following options: Logout /shutdown/ restart.

	
	Right authentication when shut down to make sure only those proper people can turn off alarm panel

	General Parameter
	Power 
	AC220V / 1A  50-60 Hz

	
	UPS
	+12V 7AH lea-acid battery. Rechargeable

	
	Power Consumption (+3W)
	15-22W (Exclude HDD)

	
	Working Temperature
	-10℃－＋55℃

	
	Working Humidity
	10%－90%

	
	Air Pressure 
	86kpa－106kpa

	
	Dimension （mm）
	350(L)*350(W)*93(H)

	
	Weight 
	5.5KG(Exclude HDD)

	
	Installation Mode 
	Wall mount 


2 Installation 
2.1 Project Installation 

This series product supports wall mount installation. Please refer to the steps listed below. 

a) Open the device package and then take out the plastic expansion bolts and self-tapping screw. 

b) Please refer to Figure 2‑1 to dig five holes in the wall. 

c) Input four expansion bolts and then insert four self-tapping screws. DO NOT INSERT THE BOLT TO THE MIDDLE HOLE. 
d) Put the device on the screws and then insert the last self-tapping screw at the middle hole. 

[image: image1.png]245.8

200. 0

270.0

214.5

163. 0





Figure 2‑1
2.2 Tamper Switch Installation 
Before the installation, please prepare the accessories. It includes: tamper switch, three screws and three nuts. 

Open the cover of the device; secure the tamper switch on the device. See Figure 2‑2.

[image: image2.jpg]



Figure 2‑2
Insert the tamper switch to the main board. See Figure 2‑3.

[image: image3.png]04460
94V-0



  [image: image4.png]



Figure 2‑3
3 Structure 
3.1 Mainboard 
The interface is shown as in Figure 3‑1.
[image: image5.png]23 2 2‘II 20 19 18

ard Slot | N

T — eoal ] wonna| [ — 17
3G Card Slot i KO 0S8 VGA - P e 3 T
S wera| | — 16

WQMMUU

fllqauuoyfl @Eﬁsl’mtdownt?ed;v‘ue and unplug T — Pover suppling e 15
e power cable before you operate! here shaibe operated by the
qualified service engineer! Improper
1 2 3 cable connection may resultin 100 poverspptigron | © || —— 4 4
it Input Poit d‘m — system malfunction or damage!
4 utpu Pon -

R

Y — 13
| Audio Output Port

SATAHDDPort

CU(‘

[—]

C\. e o
= TEoSReYSeEReg B g Za=o
== .\f\m
NE AN 3/\ TTTTTT TTT hiahdhdhd TT habdhdhd habdbdbi imididid

Video QQutput Port ideo Input Port All

1 2 AN

1l
LWL LL LL o 1
| E¢

EOLare 2. ZR )hm Siten Siren Yren Siren Sifen

4 5 6 7 8 9 10 1112





Figure 3‑1
Please refer to the following sheet for detailed information.
	SN
	Name 
	Function 

	1
	Audio input port 
	4-channel audio input port 

	2
	Bidirectional talk output port 
	Bidirectional talk 

	3
	Audio output port 
	BNC audio output. Bidirectional talk output 

	4
	Video output port 
	BNC video output 

	5
	Video input port 
	4-channel video input 

	6
	LAN
	Ethernet port 

	7
	Alarm input port
	8-channel alarm input. 

G: the public end. 

Z1-Z8: Protection zone input.

	8
	Alarm output port 
	4-channel alarm output.

	9
	RS485 port 
	A1B1: Connect to the PTZ. 

A2B2: Connect to protection zone extension module. 

A3B3 +12V G: Connect to alarm programming keyboard. 

	10
	Siren port 
	Connect to siren.

	11
	Accumulator 
	Lead-acid battery 12V DC 7AH.

	12
	User cable port 
Phone port 
	Connect to user cable or phone. 

	13
	SATA HDD port 
	Support SATA HDD

	14
	HDD power supplying port 
	Provide power to the HDD. 

	15
	Power supplying port 
	Power supplying. 

	16
	User cable port 
	RJ11 user cable port. 

	17
	Phone port 
	RJ11 phone port. 

	18
	Wireless alarm module
	Connect to wireless alarm module. 

	19
	VGA port 
	Connect to VGA displayer. 

	20
	USB port 
	Connect to USB mouse. 

	21
	HDMI port
	HDMI port. Connect to the LCD and etc.

	22
	SIM card slot 
	Support SIM card

	23
	3G card slot
	Support 3G module. 


3.2 Connection Sample
Please refer to the following figures for detailed information. See Figure 3‑2 and Figure 3‑3.
[image: image6.png]Network Video Alarm Panel

FOMI USB

3G Card Slot

‘Caution! Plasa shu dor

SIM Card Skt

S—

o

the povve cable before ot

AudiolnputPort  Bidirectional Talk

e
OOOOBG

DOOOO

(2
(o5 —udio Ouputort

‘Warning! All e csbl
here el

Fonasgsrng o

HOD P

=3 X

HMT i

Tz 3 7

H"{

Viden Output Port  Video Input Port

1 1
AL LLTLLTT,

AIEOLare 2.2K Ohm,

- ’





Figure 3‑2
[image: image7.png]HDMI Output VG Outpu‘wircless Alarm
3G Card Slot  SIM Card Slot USB Port Ma‘dule
L [ P gjﬂ !@ \\’Arelesl Alarm @)‘ Ll j:] —User Cable Port
3G Card Slot HOMI LS5 VGA t Medile J
R Phone Por ::]7 Phone Port
' “— — Power Supplying Port
Caution! Please shut down the devide and unplug Wezning! Al the cable connections Poversuppiying Por ||
; the power cable before you operate! here shall be operated by the .
Audio Output qualified service engineer! Improper B
cable connection may resultin yop ey | © || DD Power
system malfunction or damage! o | Supplying Port
g a0 S o
Bidirectional 4 2 3 4 =
TalkOutput |~ N 2N (O (D (D (D iR E, SATA HDD Port
( (o) o)(¢) (e > i
g NZANANAY \/Q Satnazfaen.s 85888805z atn 2nT.B.B. 2000 I
3 ( H.‘C.\G /9@ g j
\I/\\/ = W M DA S A b DDA TR b bR I b Us‘lerCable
T k| FowLL L L] = ) “f————— Phone Cable

!

FrocetonZone o Siren Lead-Acid Battery

Extension Modul

If ‘\/—\‘

Video Output Video Input 4 Alarm Tnput Alarm Output RS485 Programming
Network Switch B1Z Keyboard
Control

Online User Onlinie User Network Keyboard

Online User




Figure 3‑3
3.2.1 Detectors Cable Connection 
Please refer to the following figure for detailed information. See Figure 3‑4.
[image: image8.png]Alarm Alarm

Peripheral Detectors Peripheral Detectors

Panel Panel
CCT T T T T, [
| | : |
z N | i £ NO 1
12.2kQ [ ! I
! | I 2.2kQ |
| | ! |
@ I ] : 6l— G !
| | ! |
| | ! |
| | | |





Figure 3‑4
3.2.2 Alarm Output Connection 
Please refer to the following figure for detailed information. See Figure 3‑5.
[image: image9.png]DC 12V

NO/NC

Circuit Breaker
COM Peripheral Devices

O ®

Peripheral DC Load





Figure 3‑5
4 Overview of Navigation and Controls 

Before operation, please make sure:

· You have properly installed HDD and connect all the necessary cables. 

· The provided input power and the device power are matched.

· Always use the stable current, if necessary UPS is a best alternative measure.  
4.1  Login, Logout & Main Menu 

4.1.1 Login 

After system booted up, system pops up the startup wizard. 

Click the Cancel button; you can go to the system login interface. 

Click the Next Step button; you can go to the startup wizard interface. Here you can set the system basic information. See Figure 4‑1.

[image: image10.png]Startup Wizard

Startup Wizard:General, Encode, Schedule,
Record Control, Network.

B Startup





Figure 4‑1
The system login interface is shown as in Figure 4‑2.
System consists of four accounts: 
· Username: admin.  Password: admin. (administrator, local and network)

· Username: 888888. Password: 888888. (administrator, local only)

· Username: 666666. Password: 666666(Lower authority user who can only monitor, playback, backup and etc.)

· Username: default. Password: default(hidden user)

You can use USB mouse, front panel, or keyboard to input. About input method: Click [image: image11.png]


 to switch between numeral, English character (small/capitalized) and denotation.

Note: 

For security reason, please modify password after you first login. 

Continuous three times login failure will result in system alarm and five times login failure will result in account lock!

[image: image12.jpg]SYSTEM LOGIN

User Name (888888 5

Password 123}

3
6
9 ncel
=





Figure 4‑2
4.1.2 Logout 

In the main menu, click shutdown button, you can see an interface is shown as below.  See Figure 4‑3.
[image: image13.png]UTDOWN

Logout menu user g
Password is needed to re-enter the menu
after logout.

OK Cancel





Figure 4‑3
There are several options for you. See Figure 4‑4.

[image: image14.png]Logout menu user
Shutdown

Restart system
Switch user





Figure 4‑4
4.2 Live Viewing
After you logged in, the system is in live viewing mode. You can see system date, time and channel name. If you want to change system date and time, you can refer to general settings (Main Menu->Setting->General). If you want to modify the channel name, please refer to the display settings (Main Menu->Video->Display).
	1
	[image: image15.png]8]




	Recording status
	3
	[image: image16.png]



	Video loss 

	2
	[image: image17.png]



	Motion detection 
	4
	[image: image18.png]



	Camera lock


Tips

· Preview drag: If you want to change position of channel 1 and channel 16 when you are previewing, you can left click mouse in the channel 1 and then drag to channel 16, release mouse you can switch channel 1 and channel 16 positions. 
· Use mouse middle button to control window split: You can use mouse middle button to switch window split amount. 
Preview Control 
The preview control function has the following features.

· Support preview playback. 

· In the preview desktop, system can playback previous 5-60 minutes record of current channel. Please go to the Main Menu->General to set real-time playback time. 
· Support drag and play function. You can use your mouse to select any playback start time. 

· Support playback, pause and exit function. 

· Right now, system does not support slow playback and backward playback function. 

· Support digital zoom function.

· Support real-time backup function. 

You can follow the contents listed below for the operation instruction. 

Preview control interface
Move you mouse to the top centre of the video of current channel, you can see system pops up the preview control interface. See Figure 4‑5. If your mouse stays in this area for more than 6 seconds and has no operation, the control bar automatically hides. 
[image: image206.png]Security Setting:

nternet Zone

Settings

O Disable
© Enable

9] Download signed ActiveX controls (not secure)
O Disable
© Enable (not secure)
O Prompt (recommended)

9] Download unsigned Active controls (not secure)
© Disable (recommended)
© Enable (not secure)
O Prompt

19 Intiaize and scrpt ActiveX contros ok merked as sae for s
O Disable (recommended)
© Enable (not secure)
O Prompt

4] Run ctiveX contros and pug-ins
3 bbb e

< I

*Takes effect after you restart Internet Explorer

Reset custom settings





[image: image207.png]ST



[image: image19.png]



Figure 4‑5
You can refer to the following sheet for detailed information. 

	SN 
	Name 
	Function 

	1
	Realtime playback 
	It is to playback the previous 5-60 minutes record of current channel. 

Please go to the Main Menu->General to set real-time playback time.

System may pop up a dialogue box if there is no such record in current channel. 

	2
	Digital zoom 
	It is to zoom in specified zone of current channel. It supports zoom in function of multiple-channel. 

The selected area has an icon as [image: image20.png]


 and the free area is shown as an icon as[image: image21.png]


.

	3
	Real-time backup function 
	It is to backup the video of current channel to the USB device. System can not   backup the video of multiple-channel at the same time. 

Current selected backup channel has an icon as [image: image22.jpg]


 and the free channel is shown as an icon as[image: image23.png]


. .
Once the backup started, you can see the free channel is shown as an icon as[image: image24.png]


.

	4
	Exit
	


Playback control 
The playback control has the following features.

· Support play, pause, and exit and drag function. 

· During the preview playback process, you can not see the channel title and record status of current channel. It will display the channel title and the record status once you exit the preview playback. 

· During the preview playback, you can not switch the displayed channel or change current window-display mode.
· Please note the tour function has the higher priority than the preview playback. You can not control the preview playback until the tour function ended. 

4.3 Right-Click Menu

On the preview interface, right click mouse, you can view menu interface shown as in Figure 4‑6.  

[image: image25.png]View 1
View 4

Pan/TiltZoom
Color Setting

Alarm Clear
Arm/Diarm

Search
Record
Alarm Output
Main Menu




Figure 4‑6
4.3.1 Window Switch 

System supports 1/4-window. You can select from the dropdown list. See Figure 4‑7.

[image: image26.png]Vi

Pan/TiltZoom
Color Setting

Alarm Clear
Arm/Diarm

Search
Record
Alarm Output
Main Menu

M k— CAM 1
View 4

CAM 2
CAM 3
CAM 4




Figure 4‑7
4.3.2 PTZ Control
Note: All the operations here are based on PELCOD protocol. For other protocols, there might be a little difference.  

Cable Connection

 Please follow the procedures below to go on cable connection

· Connect the dome RS485 port to ATM 485 port.

· Connect dome video output cable to ATM video input port.

· Connect power adapter to the dome.

PTZ Setup

Note: The camera video should be in the current screen. Before setup, please check the following connections are right:

· PTZ and decoder connection is right. Decoder address setup is right.

· Decoder A (B) line connects with ATM A (B) line.

Boot up the ATM, input user name and password.

In the main menu, click setting, and then click Pan/Tilt Control button. The interface is shown as in Figure 4‑8. Here you can set the following items:

· Device: Please select from the dropdown list. It includes: PTZ/Alarm keyboard/alarm module. 

· Channel: Select the current camera channel.

· Protocol: Select corresponding PTZ protocol(such as PELCOD)

· Address: Default address is 1.

· Baud rate: Select corresponding baud rate. Default value is 9600.

· Data bit: Select corresponding data bits. Default value is 8.

· Stop bit: Select corresponding stop bits. Default value is 1.

· Parity: There are three options: odd/even/none. Default setup is none.

[image: image27.png]Devices [PTZ <]
Channel |1 7
Protocol [NONE |
Address (1

Baudrate (9600 |
Data Bits |8 7
Stop Bits |1 -
Parity None ~]





Figure 4‑8
After completing all the setting please click save button. 

The PTZ setup is shown as in Figure 4‑9.

Please note the commend name is grey once device does not support this function.

Double click the title to hide the PTZ menu interface. 

Here you can control PTZ direction, speed, zoom, focus, iris, preset, tour, scan, pattern aux function, light and wiper, rotation and etc. 

Speed is to control PTZ movement speed. The value ranges from 1 to 8.The speed 8 is faster than speed 1. You can use the remote control to click the small keyboard to set. 

You can click [image: image28.png]


 and [image: image29.png]


 of the zoom, focus and iris to zoom in/out, definition and brightness.  
The PTZ rotation supports 8 directions.
[image: image30.png]Page Switch

5
Zoom @
Focus @
s @
Set

Ptz Trace





Figure 4‑9
In the middle of the eight direction arrows, there is a 3D intelligent positioning key. See Figure 4‑10.X Please make sure your protocol supports this function and you need to use mouse to control.

Click this key, system goes back to the single screen mode. Drag the mouse in the screen to adjust section size. The dragged zone supports 4X to 16X speeds. Drag the mouse from top to the bottom to zoom in. Drag the mouse from bottom to up to zoom out.  It can realize PTZ automatically. The smaller zone you dragged, the higher the speed.

[image: image208.png]Mark Time

2013-12-26 14:08:58

Mark Name

[ Default

)oK )

Cancel





Figure 4‑10
Here is a sheet for you reference.

	Name 
	Function

key 
	function
	Function

 key
	function

	Zoom
	[image: image31.bmp]
	Near 
	[image: image32.bmp]
	Far 

	Focus
	[image: image33.bmp]
	Near 
	[image: image34.bmp]
	Far 

	Iris
	[image: image35.bmp]
	close 
	[image: image36.bmp]
	Open 


Preset/ Patrol/Pattern/Scan 

In Figure 4‑9, please click the “set” button. The interface is shown as below. See Figure 4‑11.

Here you can set the following items:

· Preset

· Tour
· Pattern

· Border

[image: image37.png]Function
Preset
Tour
Pattern
Border

Preset 1
Patrol No. |0
Set

Del Preset





Figure 4‑11
In X328H328H328H Figure 4‑9, click page switch button, the interface is shown as in Figure 4‑12.

Here you can activate the following functions:

· Preset

· Tour
· Pattern

· Auto scan

· Auto pan

· Flip

· Reset

· Page switch
           [image: image38.png]No. 0 (

Preset

Tour

AutoScan AutoPan

Flip

Page Switch





Figure 4‑12
Note: 
· Preset, tour and pattern all need the value to be the control parameter. You can define it as you require.

· You need to refer to your speed dome user’s manual for Aux definition. In some cases, it can be used for special process.

· The following setups are usually operated in the Figure 4‑9, Figure 4‑11 and Figure 4‑12.

Preset Setup

In Figure 4‑9, use eight direction arrows to adjust camera to the proper position.

In Figure 4‑11, click preset button and input preset number. The interface is shown as in Figure 4‑13.

Now you can add this preset to one tour.

[image: image39.png]Function
Preset
Tour
Pattern
Border

Preset 1
Patrol No. |0
Set

Del Preset





Figure 4‑13
Activate Preset 

In Figure 4‑12, please input preset number in the No. blank, and click preset button.
Patrol setup (Tour Setup)
In Figure 4‑11, click patrol button. The interface is shown as in Figure 4‑14. Input preset number and add this preset to a patrol (tour). For each patrol (tour), you can input max 80 presets.  

[image: image40.png]PAN/TILT/ZOOM

Preset
Patrol
Pattern Add Preset

Border Del Preset





Figure 4‑14
Activate Patrol (tour)
In X341H341H341HFigure 4‑11, input patrol (tour) number in the No. blank and click patrol button 

Pattern Setup

In Figure 4‑11, click pattern button and then click “begin” button. The interface is shown as in Figure 4‑15. Then you can go to Figure 4‑9  to modify zoom, focus, and iris.  

Go back to Figure 4‑15 and click “end” button. You can memorize all these operations as pattern 1.

[image: image41.png]PAN/TILT/ZOOM

Preset
Patrol

Pattern
Border





Figure 4‑15
Activate Pattern Function 

In Figure 4‑12, input mode value in the No. blank, and click pattern button.
Auto Scan Setup

In Figure 4‑13, click border button. The interface is shown as in Figure 4‑16.

Please go to Figure 4‑9, use direction arrows to select camera left limit 

Then please go to Figure 4‑16 and click left limit button 

Repeat the above procedures to set right limit.

[image: image42.png]PAN/TILT/ZOOM

Preset
Patrol

Pattern
Border





Figure 4‑16
Activate Auto Scan
In Figure 4‑12, click “Auto Scan” button, the system begins auto scan. Correspondingly, the auto scan button becomes Stop button. Click stop button to terminate scan operation.
Flip

In Figure 4‑12, click page switch button, you can see an interface is shown as below. See Figure 4‑17. Here you can set auxiliary function. The aux value has relation ship with the Aux button of the decoder.
Click page switch button again, system goes back to Figure 4‑9.

[image: image43.png]Direct Aux Oper

Light ~] (on) (of)

Aux Num Oper

i)





Figure 4‑17
4.3.3 Color

Here you can set hue, brightness, contrast, saturation, gain, white level, color mode and etc. See Figure 4‑18.
[image: image44.png]Period W (00 :00 -24 :00 00 :00 -24 :00
Hue . 50 . 50
Brightness : 50 : 50
Contrast : 50 : 50
Saturation : 50 : 50
Gain . 50 . 50
White Level low 7 low 7
Color mode Standard N Standard N
(Customized ) [ Default | ( [e]4 ] [ Cancel |





Figure 4‑18
4.3.4 Alarm Clear
Please refer to chapter 4.4.1.10 for detailed information.
4.3.5 Arm/Disarm
Please refer to chapter 4.4.1.3 for detailed information.
4.3.6 Search 

Please refer to chapter 4.4.2.3 for detailed information.
4.3.7 Record Control
Please refer to chapter 4.4.2.5 for detailed information.
4.3.8 Alarm Output

Please refer to chapter 4.4.1.7 for detailed information.
4.3.9 Main Menu 

Click it to go to the main menu interface. 

Please refer to chapter 4.4 for detailed information.
4.4 Main Menu 
On the preview interface, right click mouse and then select main menu, you can go to the following interface. See Figure 4‑19. It includes four buttons: Alarm, video, info, system. 

Tips

Right click mouse to go back to the previous interface. 
[image: image45.png]ALARM

€

INFO

VIDEO

SYSTEM





Figure 4‑19
4.4.1 Alarm 

The alarm interface includes the following items. See Figure 4‑20.
It includes ten buttons: zone, alarm, arm/disarm, profile, remote, call centre, alarm output, bell output, abnormality, zone status. 
[image: image46.png]OB me

ALARM ARM/DISARM PROFILE REMOTE

B & m & %

CALL CENTRE ALARM OUTPUT BELL OUTPUT ABNORMALITY ZONE STATUS

“®:Setting of exterior alarm output and responding record functions.





Figure 4‑20
4.4.1.1 Zone
It is to set protection zone and its corresponding alarm trigger operation. Please refer to chapter 4.4.1.2 for detailed parameter information. 

· Zone No. Please select a zone number from the dropdown list. 

· Zone name: Please select a zone name from the dropdown list. Support customized setup. 

· Enable: Check the box here to enable zone function. 

· Type: There are three types: realtime zone (Figure 4‑22), delay zone (Figure 4‑23), 24-H zone (Figure 4‑51). 

· Realtime zone: On the arm status, once the detector connected to the zone is trigged, the alarm controller can generate an alarm immediately. There is no delay time. 

· Delay zone: Please set two parameters: Enter delay and Exit delay.

a) In the entry delay mode, there is no trigger operation after the alarm is trigged for M seconds (M refers to the in delay value you set here). You can disarm during the M seconds. If you do not disarm, the protection zone is activated after M seconds. For example: after you enabled arm status and then set the in delay is 10s. System only enables link alarm after the alarm occurred for 10 seconds no matter the alarm has disappeared or still exists. 

b)  Exit delay: After you enable arm status, the arm operation becomes valid after N seconds (N refers to the exit delay value you set here). During the N seconds period, the alarm link operation is null. For example, after you enable out delay time 15s, there is no alarm link event during the first 15 seconds until the 15-second countdown ends. 

· 24-H zone: The detector of current protection zone is armed for all the 24-H, no matter it is armed or disarmed. But you can arm/disarm manually. There is no arm period setup and the arm button on the front panel is null. You can only set on the menu interface. 
· Call centre: System can upload the alarm to the call centre when an alarm occurs. 
[image: image47.png]No. 1 ~| Name
Enable . Local |~
Type Settings

Period E

W Alarm Out BRRE Ext Alarm Out
@ Show Message [ Alarm Upload [_)Send Email
[WRecord Channel B(2)(3)(2

WPTZ Activaton  (__Setup ) Delay

W Tour B2RE

@ Snapshot BRORE

WBuzzer

B Audio Broadcast [N/A ~| OutputPort

(Bl Output Control
W Call Centre 12

Zone 1

El

Talk Port





Figure 4‑21
[image: image48.png]Type Realtime N

Zone Source Type |Gas Sensor |~





Figure 4‑22
[image: image49.png]Type
Zone Source Type

Entry Delay
Exit Delay

Delay v
Gas Sensor |~/
0 sec.
0 sec.





Figure 4‑23
[image: image50.png]Type 24H 7

Zone Source Type |Gas Sensor |~





Figure 4‑24
4.4.1.2 Alarm

Before operation, please make sure you have properly connected alarm devices such as buzzer.

In the main menu, from Setting to Alarm, you can see alarm setup interface. See Figure 4‑25.

Alarm interface is shown as below. See Figure 4‑25.

· Alarm in: Here is for you to select channel number.

· Event type: There are two types. One is local input and the other is network input. 

· Type: normal open or normal close.

· PTZ activation: When an alarm occurred, system can activate the PTZ operation. The PTZ activation lasts an anti-dither period. 

· In the Pan/Tilt/Zoom interface (Main menu->Setting-> Pan/Tilt/Zoom), please set video channel, speed dome protocol and etc. 

· Select the channel of current speed dome as current monitor video and the right click mouse to select Pan/Tilt/Zoom item. Now you can set preset, tour pattern.

· In Figure 4‑25, click “select” button, you can see an interface is shown as in Figure 4‑26. Here you can set the activation operation such as preset tour, pattern and enable. 
· Period: Click set button, you can see an interface is shown as in Figure 4‑27. Here you can set for business day and non-business day. In Figure 4‑27, click set button, you can see an interface is shown as in Figure 4‑28. Here you can set your own setup for business day and non-business day.

· Anti-dither: Here you can set anti-dither time. The value ranges from 5 to 600s. The anti-dither time refers to the alarm signal lasts time. It can be seem as the alarm signal activation stays such as the buzzer, tour, PTZ activation, snapshot, channel record.  The stay time here does not include the latch time. During the alarm process, the alarm signal can begin an anti-dither time if system detects the local alarm again. The screen prompt, alarm upload, email and etc will not be activated. For example, if you set the anti-dither time as 10 second, you can see the each activation may last 10s if the local alarm is activated. During the process, if system detects another local alarm signal at the fifth second, the buzzer, tour, PTZ　activation, snapshot, record channel will begin another 10s while the screen prompt, alarm upload, email will not be activated again. After 10s, if system detects another alarm signal, it can generate an alarm since the anti-dither time is out.  
· Show message: System can pop up a message to alarm you in the local host screen if you enabled this function.
· Alarm upload: System can upload the alarm signal to the network (including alarm centre) if you enabled current function.
· Send email: System can send out email to alert you when an alarm occurs. 

· Record channel: you can select proper channel to record alarm video (Multiple choices). 
· You need to set alarm record mode as Schedule in Record interface (Main Menu->Advanced->Record). Please note the manual record has the highest priority. System record all the time no matter there is an alarm or not if you select Manual mode. 

· Now you can go to the Schedule interface (Main Menu->Setting->Schedule) to set the record type, corresponding channel number, week and date. You can select the record type:Regular/MD/Alarm/MD&Alarm. Please note, you can not select the MD&Alarm and MD(or Alarm) at the same time. 

· Now you can go to the Encode interface to select the alarm record and set the encode parameter (Main Menu->Setting->Encode). 

· Finally, you can set the alarm input as the local alarm and then select the record channel. The select channel begins alarm record when an alarm occurred. Please note system begins the alarm record instead of the MD record if the local alarm and MD event occurred at the same time. 
· Latch: When the anti-dither time ended, the channel alarm you select in the alarm output may last the specified period. The value ranges from 1 to 300 seconds. This function is not for other alarm activation operations. The latch is still valid even you disable the alarm event function directly.  .

· Tour: Here you can enable tour function when an alarm occurs.  System supports one-window tour. Please go to chapter 4.4.2.6 Display for tour interval setup. 
· Snapshot: You can enable this function to snapshoot image when an alarm occurs.
· Video matrix: Check the box to enable this function. The video matrix output can display the alarm video of current channel when there is an alarm signal. 
· Buzzer: Highlight the icon to enable this function. The buzzer beeps when an alarm occurs.
· Snapshot: For snapshot operation, please refer to chapter 4.4.2.4.2.
· Audio broadcast: System triggers audio function when generating an alarm. 

· Output port: It is to set output port: speaker/MIC output port. 
· Siren output control: System triggers siren when an alarm occurs. 

Note: The network alarm means the alarm signal from the TCP/IP. You can enable the network alarm function via the net SDK. The network alarm does not have the device type; anti-dither and alarm upload function. The rest items are the same. 
Please highlight icon [image: image51.jpg]


 to select the corresponding function. After setting all the setups please click save button, system goes back to the previous menu.

[image: image52.png]Event Type NEE ~| Alarm In 1 9
Enable [ ]

Period E

W Alarm Out BRORE Latch 10 sec.
B Show Message Send Email

[WRecord Channel B(2)(3)(2

WPTZ Activaton  (__Setup ) Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~
(Bl Output Control





Figure 4‑25
[image: image53.png]CAM 1
CAM 3

None

0 CAM 2

None

0 CAM 4

o)

None

None





Figure 4‑26
[image: image54.png]Work Day ~| [ Setwp | x
W (0000 24 :00 0000 24 00
00 :00  -24 :00 00 :00 24 :00
00 :00  -24 :00 00 :00 24 :00
0.3 6 9 12 15 18 21 24
Sun S
Mon I ——
Tue 1 ——
Wed I ——
Thu 1 ——
Fri I ——
Sat ]
0 3 6 9 12 15 18 21 24





Figure 4‑27
[image: image55.png]Sun  Mon Tue Wed Thu Fri Sat
Work Day ® L L L L (o] (o]
FreeDay O O O O O L] L]





Figure 4‑28
4.4.1.3 Arm/Disarm

The arm/disarm interface is shown as below. See Figure 4‑29. Here you can arm or disarm a channel/all channels. 

· Arm all: Check the box here to arm all protection zones. 

· Disarm all: Check the box here to disarm all protection zones. 

· Normal/bypass: Check the box, the status is shown as normal. Cancel the box, the status is shown as bypass. 

[image: image56.png]® Arm Al O Disarm All

v/ Status No. Name Type Entry Delay Exit Delay
v/ Normal 1 Zone1 Realtime - =
v|Normal 2 Zone2 Realtime - =
v|Normal 3 Zone3 Realtime - =
v|Normal 4 Zone4 Realtime - =
v|Normal 5 Zone5 Realime - =
v|Normal 6 Zone6 Realime - =
v|Normal 7 Zone7 Realtime - =
v|Normal 8 Zone8 Realtime - =





Figure 4‑29
4.4.1.4 Profile
The profile interface is shown as below. See Figure 4‑30.
Please highlight the corresponding box to select a mode and then select profile mode form the dropdown list.

Click Save button to save current setup. 
[image: image57.png]Enable
Outdoor
Home

PROFILE

12345867328

SEEEEES
mimie] | ] ] )

Outdoor ]

Outdoor

Home

( Defaut ) ( Save ) ( Cancel )





Figure 4‑30
4.4.1.5 Remote
The alarm remote interface is shown as below. See Figure 4‑31.

· Wireless ID: It refers to the wireless ID of the remote control. 

· User name: It is the user name of the remote control. 

· Modify: It is to change remote control status. Y means current remote control is valid. N means it is null now. 

· Delete: It is to delete the registered remote control. 

· Status: It is to display remote control status. Y means current remote control is valid. N means it is null now. 

Click wireless code button at the bottom of the interface to search remote controls. Follow the dialogue box to click any button of the remote control to connect. After successful connection, you can change the registered remote control information. 

[image: image58.png]0 |WirelessID UserName Modify Delete Status

Wireless Code





Figure 4‑31
4.4.1.6 Call Centre

The interface is shown as in Figure 4‑32.
· Phone groups: System max supports 2 groups.

· Centre name: Input customized centre name.

· Centre No.: Input call centre number. 

· Protocol type: The default setup is CID.

· Signal transmission mode: The default setup is DTMF5/S.

· Dial time: Once the call centre did not pick up the phone, you can set dial times here. The value ranges from 1 to 5.

· Dial delay: It is the dial interval between two times. 

· Account: It is the user code provided by the call centre. The default setup is 0000.
[image: image59.png]Phone Groups

Centre Name
Centre No.

Protocol Type

MobileCentre1

057188883888

CID

Signal Transmission Mode [DTMF 5/S N
Dial Times 3

Dial Delay 10

Account 0000

[ Default | Save





Figure 4‑32
4.4.1.7 Alarm Output

Here is for you to set proper alarm output.
Please highlight icon [image: image60.jpg]


 to select the corresponding alarm output.

After all the setups please click OK button, system goes back to the previous menu. See Figure 4‑33.
Click settings button, the interface is shown as in Figure 4‑34. Here you can set alarm output module, output channel, output name and type. 
[image: image61.png]"
>

Alarm Type
Schedule
Manual
Stop

Status

Al 123 4
®e0O0
OCee
0000

0000

O 00

Local alarm output channel

(Setings ) OK ) ( Cancel )





Figure 4‑33
[image: image62.png]Alarm Output Module |Local Module | ~|

Alarm Out 1 N

Alarm Output Name

Alarm Output Type

[ Default | ( Save |[( Cancel |





Figure 4‑34
4.4.1.8 Bell Output 
Here is for you to set siren output. See Figure 4‑35.
· Bell output: Select siren output channel. 

· Alarm switch: Check the box here to enable alarm output function.

· Status: The corresponding channel is highlighted if there is any alarm. 
[image: image63.png]BELL OUTPUT All 1

Alarm Switch

Status

( Enable | Stop ) [ Cancel |





Figure 4‑35
4.4.1.9 Abnormality 

Abnormality interface is shown as in Figure 4‑36.

· Event type: There are several options for you such as disk error, no disk, disconnection, IP conflict, MAC conflict and etc. 
· Less than: Please input HDD space threshold value here (For disk no space type only). 
· Alarm output: Please select alarm activation output port (multiple choices). 

· Latch: Here you can set corresponding delaying time. The value ranges from 10s-300s. System automatically delays specified seconds in turning off alarm and activated output after external alarm cancelled.
· Show message: system can pop up the message in the local screen to alert you when an alarm occurs. 

· Alarm upload: System can upload the alarm signal to the network (including alarm centre) if you enabled current function.

· Send email: System can send out email to alert you when an alarm occurs. 

· Buzzer: Highlight the icon to enable this function. The buzzer beeps when an alarm occurs.
· Audio broadcast: System triggers audio function when generating an alarm. 

· Output port: It is to set output port: speaker/MIC output port. 
· Siren output control: System triggers siren when an alarm occurs. 

[image: image64.png]Event Type No Disk 9
Enable
W Alarm Out BRORE Latch

@ Show Message [ Alarm Upload [_)Send Email

WBuzzer

B Audio Broadcast [N/A [+] Output Port

(Bl Output Control

sec.

Talk Port |

Save Cancel

El





Figure 4‑36
[image: image65.png]WBuzzer

Event Type Disk No Space |
Enable

Less Than 20 %
W Alarm Out BRORE Latch

@ Show Message [ Alarm Upload [_)Send Email

B Audio Broadcast [N/A [+] Output Port

(Bl Output Control

sec.

Talk Port |

Save Cancel

El





Figure 4‑37
4.4.1.10 Zone Status 
Here is for you to view protection zone status. See Figure 4‑84.
· For the first dropdown list, you can select module. It includes local module, extension module and all.

· For the second dropdown list, you can select corresponding channel. 

· Confirm: Select a channel and then click it to cancel an alarm. 
[image: image66.png]Index 12345¢86738
Local Module SEEEEEEm
Ext Module1
Ext Module2
Ext Module3
Ext Module4
Ext Module5
Ext Module6
Ext Module7
Ext Module8

Local Module | ~| [1 ~| [ Confirm |





Figure 4‑38
4.4.2 Video 
The video interface is shown as below. See Figure 4‑39.

It has 8 buttons: encode, detect, search, schedule, record, display, broadcast, D/A switch. 
[image: image67.png]C

ENCODE

DISPLAY

0

DETECT

®

BROADCAST

& w B

SEARCH SCHEDULE RECORD

N

D/A SWITCH





Figure 4‑39
4.4.2.1 Encode

Encode setting includes the following items. See Figure 4‑40.

Please note some series do not support extra stream. 

· Channel: Select the channel you want.
· Type: Please select from the dropdown list. There are three options: regular/motion detect/alarm. You can set the various encode parameters for different record types. 
· Compression: System supports H.264. 
· Resolution: For analog channel, system supports various resolutions, you can select from the dropdown list. The main stream supports 960H/D1/HD1/2CIF/CIF/QCIF and the extra stream supports CIF/QCIF. For digital channel, it is the resolution of the connected network camera. 

· Bit rate type: System supports two types: CBR and VBR. In VBR mode, you can set video quality.

· Quality: There are six levels ranging from 1 to 6. The sixth level has the highest image quality.
· Video/audio: You can enable or disable the video/audio.
· Overlay: Click overlay button, you can see an interface is shown in Figure 4‑41.
· Cover area (Privacy mask): Here is for you to set privacy mask section. You can drag you mouse to set proper section size. In one channel video, system max supports 4 zones in one channel.

· Preview/monitor: privacy mask has two types. Preview and Monitor. Preview means the privacy mask zone can not be viewed by user when system is in preview status. Monitor means the privacy mask zone can not be view by the user when system is in monitor status.
· Time display: You can select system displays time or not when you playback. Please click set button and then drag the title to the corresponding position in the screen.

· Channel display: You can select system displays channel number or not when you playback. Please click set button and then drag the title to the corresponding position in the screen.
· Copy：After you complete the setup, you can click Copy button to copy current setup to other channel(s). You can see an interface is shown as in Figure 4‑42. You can see current channel number is grey. Please check the number to select the channel or you can check the box ALL. Please click the OK button in Figure 4‑42 and Figure 4‑40 respectively to complete the setup. Please note, once you check the All box, you set same encode setup for all channels. Audio/video enable box, overlay button and the copy button is shield. See Figure 4‑43.  
Please highlight icon [image: image68.jpg]


 to select the corresponding function.

[image: image69.png]Channel 1 N
Type Regular ~| |Extra Stream1| |
Compression H.264 v |H.264 N
Resolution 960H - |CIF N
Frame Rate(FPS) (25 M N
Bit Rate Type CBR ~| |CBR N
| Frame Interval 1S ML 9
Bit Rate(Kb/S) 2048 |~ 160 |~
Reference Bit Rate 1024-3072Kb/S 40-256Kb/S
Audio/Video [ ]
Audio Format G711a 9

(ovERAY)

(SnRpSHoT)





Figure 4‑40
[image: image70.png]Cover-Area Preview [_|Monitor| Set
Time Display W Monitor( Set
Channel Display W Monitor( Set

Save Cancel





Figure 4‑41
[image: image71.png]



Figure 4‑42
[image: image72.png]Channel All 9
Type Regular ~| |Extra Stream1| |
Compression H.264 v |H.264 N
Resolution 960H - |CIF N
Frame Rate(FPS) (25 M N
Bit Rate Type CBR ~| |CBR N
| Frame Interval 1S ML 9
Bit Rate(Kb/S) 2048 |~ 160 |~
Reference Bit Rate 1024-3072Kb/S 40-256Kb/S
Audio/Video [ ]
Audio Format G711a 9

(SAPSHOT)





Figure 4‑43
4.4.2.2 Detect

In the main menu, from Setting to Detect, you can see motion detect interface. See Figure 4‑44.There is three detection types: motion detection, video loss, camera masking.

· The video loss has no detection region and sensitivity setup and camera masking has no detection region setup. 

· You can see motion detect icon if current channel has enabled motion detect alarm. 

· You can drag you mouse to set motion detect region without Fn button. Please click OK button to save current region setup. Right click mouse to exit current interface. 

4.4.2.2.1 Motion Detect
Detection menu is shown as below. See Figure 4‑44
· Event type: From the dropdown list you can select motion detection type.
· Channel: Select a channel from the dropdown list to set motion detect function. 
· Enable: Check the box here to enable motion detect function. 
· Region: Click select button, the interface is shown as in Figure 4‑45.  Here you can set motion detection zone. There are 396(PAL)/330(NTSC) small zones. Blue zone is the motion detection zone. White zone is the disarmed zone. System can only detect motion detect signal from the arm region. You can click Fn button to switch between the arm mode and disarm mode.  In arm mode, you can click the direction buttons to move the green rectangle to set the motion detection zone. After you completed the setup, please click ENTER button to exit current setup. Do remember click save button to save current setup. If you click ESC button to exit the region setup interface system will not save your zone setup.

· Sensitivity: System supports 6 levels. The sixth level has the highest sensitivity.
· Period: Click set button, you can see an interface is shown as in Figure 4‑47.Here you can set motion detect period. System only enables motion detect operation in the specified periods. It is not for video loss or the camera masking. Please select Sunday to Saturday from the dropdown list. You can see there are six periods for you to set. Please note you need to check the box before the period to enable the period setup valid. In Figure 4‑47, select work day or non-work day from the drop down list, and then click the Set button, you can set your own business day and non-business day setup. See Figure 4‑48. You can set Monday to Friday as the working day and Saturday and Sunday as non-business day. Click Save button to go back to previous interface. 

· Anti-dither: System only memorizes one event during the anti-dither period. The value ranges from 5s to 600s.
· Alarm output: when an alarm occurs, system enables peripheral alarm devices.

· Latch: when motion detection complete, system auto delays detecting for a specified time. The value ranges from 1-300(Unit: second)

· Show message: System can pop up a message to alarm you in the local host screen if you enabled this function.

· Alarm upload: System can upload the alarm signal to the network (including alarm centre) if you enabled current function.

· Send email: System can send out email to alert you when an alarm occurs. 
· Record channel: System auto activates motion detection channel(s) to record once an alarm occurs. Please make sure you have set MD record in Schedule interface(Main Menu->Setting->Schedule) and schedule record in manual record interface(Main Menu->Advanced->Manual Record) 
· PTZ activation: Here you can set PTZ movement when an alarm occurs. Such as go to preset, tour &pattern when there is an alarm. Click “select” button, you can see an interface is shown as in X305H305H305HFigure 4‑46X.

· Record Delay: System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.
· Tour: Here you can enable tour function when an alarm occurs. System one-window tour. Please go to chapter 4.4.2.6 Display for tour interval setup. 

· Buzzer: Highlight the icon to enable this function. The buzzer beeps when an alarm occurs.
· Snapshot: For snapshot operation, please refer to chapter 4.4.2.4.2.
· Audio broadcast: System triggers audio function when generating an alarm. 

· Output port: It is to set output port: speaker/MIC output port. 
· Siren output control: System triggers siren when an alarm occurs. 
· Test: Click it to test current motion detect setup (do not need to save). Click Select button after Region, you can set motion detect area. The default setup is whole region. The blue zones are the selected area. Please set sensitivity and click Test button, you can see system only display motion detect status of the blue area. The red flashing area is current motion detect. See Figure 4‑50.
Please highlight icon [image: image73.jpg]


 to select the corresponding function. After all the setups please click Save button, system goes back to the previous menu.
Note:

In motion detection mode, you can not use copy/paste to set channel setup since the video in each channel may not be the same.  

In Figure 4‑45, you can left click mouse and then drag it to set a region for motion detection. Click Fn to switch between arm/withdraw motion detection. After setting, click enter button to exit. 

[image: image74.png]Event Type Motion Detect | ~| Channel 1 S

Enable | ] Sensitivity 3 -

Region Setup
Period Setup Anti-dither 5 sec.

W Alarm Out BRORE Latch 10 sec.
@ Show Message [ Alarm Upload [_)Send Email
[WRecord Channel B(2)(3)(2

WPTZ Activation E Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~

(Bl Output Control

Toa ) (o) (G





Figure 4‑44
[image: image75.png]



Figure 4‑45
[image: image76.png]CAM 1
CAM 3

None

0 CAM 2

None

0 CAM 4

o)

None

None





Figure 4‑46
[image: image77.png]Thu M
W (0000 24 :00 0000 24 00
00 :00  -24 :00 0000  -24 :00
0000 -24 :00 0000  -24 :00
0_3 6 1215 138 21 24
Sun S
Mon I ——
Tue 1 ——
Wed I ——
Thu 1 ——
Fri I ——
Sat ]
0 3 6 12 15 18 21 24





Figure 4‑47
[image: image78.png]Work Day ~| [ Setwp |
W (0000 24 :00 0000 24 00
00 :00  -24 :00 00 :00 24 :00
0000 -24 :00 0000  -24 :00
0.3 6 9 12 15 18 21 24
Sun S
Mon I ——
Tue 1 ——
Wed I ——
Thu 1 ——
Fri I ——
Sat ]
0 3 6 9 12 15 18 21 24





Figure 4‑48
[image: image79.png]Work Day o L L L L] L] (o]
Free Day [ o o o (o] (o] L]

( Save | ( Cancel |





Figure 4‑49
[image: image80.png]



Figure 4‑50
4.4.2.2.2 Video Loss 

In Figure 4‑44, select video loss from the type list. You can see the interface is shown as in Figure 4‑51.This function allows you to be informed when video loss phenomenon occurred. You can enable alarm output channel and then enable show message function.

Tips:

· You can enable preset/tour/pattern activation operation when video loss occurs.

· Please refer to chapter 4.4.1.2 for alarm setup information, except there is no device type, anti-dither, video matrix function. 

· System only enables video loss function during the period you set here. It is null for motion detect or camera masking type. 
[image: image81.png]Event Type Video Loss | ~| Channel 1 S
Enable [ ]

Period Setup

W Alarm Out BRORE Latch 10 sec.
@ Show Message [ Alarm Upload [_)Send Email

[WRecord Channel B(2)(3)(2

WPTZ Activaton  (__Setup ) Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~

*Bell Output Control





Figure 4‑51
4.4.2.2.3 Camera Masking

When someone viciously masks the lens, or the output video is in one-color due to the environments light change, the system can alert you to guarantee video continuity. Camera masking interface is shown as in Figure 4‑52. You can enable “Alarm output “or “Show message” function when camera masking alarm occurs. 

· Sensitivity: The value ranges from 1 to 6. It mainly concerns the brightness. The level 6 has the higher sensitivity than level 1. The default setup is 3. 
Tips:

You can enable preset/tour/pattern activation operation when video loss occurs.

Please refer to chapter 4.4.2.2.1 motion detection for detailed information.

Note:

· In Detect interface, copy/paste function is only valid for the same type, which means you can not copy a channel setup in video loss mode to camera masking mode.

· About Default function. Since detection channel and detection type may not be the same, system can only restore default setup of current detect type. For example, if you click Default button at the camera masking interface, you can only restore default camera masking setup. It is null for other detect types. 

· System only enables camera masking function during the period you set here. It is null for motion detect or video loss type. 

[image: image82.png]Event Type Camera Maski|~| Channel 1 S

Enable | ] Sensitivity 3 -

Period Setup

W Alarm Out BRORE Latch 10 sec.
@ Show Message [ Alarm Upload [_)Send Email
[WRecord Channel B(2)(3)(2

WPTZ Activation E Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~

(Bl Output Control





Figure 4‑52
4.4.2.3 Search 
Click search button, search interface is shown as below. See Figure 4‑53.

Usually there are three file types:
· R: Regular recording file.

· A: External alarm recording file.
· M: Motion detection recording file. 
[image: image83.png](From ReadWrite Hdd [~
@® REC O PIC

Su Mo Tu We Th Fr Sa
OREMEEE

12 ; 14 1 16

AllRecord % Normal ~ + Alarm ¥ Motion

5 12




Figure 4‑53
Please refer to the following sheet for more information.

	SN
	Name 
	Function 

	1
	Display window 
	· Here is to display the searched picture or file.
· Support 1/4-window playback. 

	2
	Search type 
	· Here you can select to search the picture or the recorded file. 
· You can select to play from the read-write HDD, from peripheral device or from redundancy HDD. 

· Before you select to play from the peripheral device, please connect the corresponding peripheral device. You can view all record files of the root directory of the peripheral device. Click the Browse button; you can select the file you want to play. 

	3
	Calendar 
	· The blue highlighted date means there is picture or file. Otherwise, there is no picture or file. 
· In any play mode, click the date you want to see, you can see the corresponding record file trace in the time bar. 

	4
	Playback mode and channel selection pane. 
	· Playback mode：1/4-channel. 
· In 1-window playback mode: you can select 1-4 channels. 

· In 4-window playback mode: you can select 4 channels according to your requirement. 

· The time bar will change once you modify the playback mode or the channel option. 

	5
	Card number search 
	The card number search interface is shown as below. Here you can view card number/field setup bar. You cam implement advanced search. 

[image: image84.png]lcrro "B Ejohes B8 [G§ scad |- (D)

et i e )






	6
	File list switch button 
	· Double click it; you can view the picture/record file list of current day. 
· The file list is to display the first channel of the record file.   

· The system can display max 128 files in one time. Use the (│and │( or the mouse to view the file. Select one item, and then double click the mouse or click the ENTER button to playback. 
· You can input the period in the following interface to begin accurate search. 
· File type：R—regular record； A—external alarm record；M—Motion detect record.

[image: image85.png]00 ; 00 ; 00 « |




· Lock file. Click the file you want to lock and click the button [image: image86.jpg]


 to lock. The file you locked will not be overwritten.

· Search locked file: Click the button[image: image87.jpg]


 to view the locked file.

· Return: Click button[image: image88.jpg]


, system goes back to the calendar and channel setup interface. 

Please note:

· System max locks 16 files. The size of the locked file shall be less than the 1/4 of the HDD total space. The first 16G of each partition can not be locked. 

· System can only lock one file at one time and can not lock the extra stream. For the file that is writing or overwriting, it can not be locked. 

	7
	Mark file list button 
	Click it to go to mark file list interface. You can view all mark information of current channel by time. Please refer to chapter 4.4.2.3.3 for detailed information. 
Please note only the product of this icon supports mark function. 

	8

	Playback control pane. 
	►/(   
	Play/Pause
There are three ways for you to begin playback. 

· The play button

· Double click the valid period of the time bar.

· Double click the item in the file list. 

In slow play mode, click it to switch between play/pause. 

	
	
	■
	Stop 

	
	
	(
	Backward play 
In normal play mode, left click the button, the file begins backward play. Click it again to pause current play. 

In backward play mode, click ►/( to restore normal play.

	
	
	│(/(│
	In playback mode, click it to play the next or the previous section. You can click continuously when you are watching the files from the same channel. 
In normal play mode, when you pause current play, you can click (│ and │( to begin frame by frame playback. 

In frame by frame playback mode, click ►/( to restore normal playback.

	
	
	[image: image209.png]Channel (1 7

Start Time (2013 - 12- 26 |00: 00: 00

End Time (2013 - 12- 27 [00: 00: 00 [ Search

1 CH Mark Time Mark Name
1 1 2013-12-26 14:08:58 liar

Delete




►
	Slow play
In playback mode, click it to realize various slow play modes such as slow play 1, slow play 2, and etc.  

	
	
	(
	Fast forward 
In playback mode, click to realize various fast play modes such as  fast play 1,fast play 2 and etc. 

	
	
	Note: The actual play speed has relationship with the software version. 

	
	
	[image: image89.png]



	Smart search 

	
	
	[image: image90.jpg]



	The volume of the playback 

	
	
	[image: image91.jpg]



	Click the snapshot button in the full-screen mode, the system can snapshot 1 picture. 
System supports custom snap picture saved path. Please connect the peripheral device first, click snap button on the full-screen mode, you can select or create path. Click Start button, the snapshot picture can be saved to the specified path.

	
	
	[image: image92.jpg]



	Mark button. 

Please note this function is for some series product only. Please make sure there is a mark button in the playback control pane. 

You can refer to chapter 4.4.2.3.3 for detailed information. 

	9
	Time bar 
	· It is to display the record type and its period in current search criteria. 
· In 4-window playback mode, there are corresponding four time bars. In other playback mode, there is only one time bar.  

· Use the mouse to click one point of the color zone in the time bar, system begins playback. 

· The time bar is beginning with 0 o'clock when you are setting the configuration. The time bar zooms in the period of the current playback time when you are playing the file.

· The green color stands for the regular record file. The red color stands for the external alarm record file. The yellow stands for the motion detect record file. 

	10
	Time bar unit 
	●The option includes: 24H, 12H, 1H and 30M. The smaller the unit, the larger the zoom rate. You can accurately set the time in the time bar to playback the record. 

· The time bar is beginning with 0 o'clock when you are setting the configuration. The time bar zooms in the period of the current playback time when you are playing the file. 

	11
	Backup 
	· Select the file(s) you want to backup from the file list. You can check from the list. Then click the backup button, now you can see the backup menu. System supports customized path setup. After select or create new folder, click the Start button to begin the backup operation.  The record file(s) will be saved in the specified folder. 
· Check the file again you can cancel current selection. System max supports to display 32 files from one channel. 
· After you clip on record file, click Backup button you can save it. 

· For one device, if there is a backup in process, you can not start a new backup operation. 

	12
	Clip 
	· It is to edit the file. 
●Please play the file you want to edit and then click this button when you want to edit. You can see the corresponding slide bars such as [image: image93.png]


 or [image: image94.png]


 in the time bar of the corresponding channel. You can adjust the slide bar or input the accurate time to set the file end time.  
· After you set, you can click Clip button again to edit the second period. You can see the slide bar restore its previous position. 

· Click Backup button after clip, you can save current contents in a new file. 
· You can clip for one channel or multiple-channel. The multiple-channel click operation is similar with the one-channel operation. 
Please note:

· System max supports 1024 files backup at the same time. 

· You can not operate clip operation if there is any file has been checked in the file list.  

	13
	Record type 
	In any play mode, the time bar will change once you modify the search type. 

	Other Functions

	14
	Smart search 
	· When system is playing, you can select a zone in the window to begin smart search. Click the motion detect button to begin play.

· Once the motion detect play has begun, click button again will terminate current motion detect file play. 

· There is no motion detect zone by default. 

· If you select to play other file in the file list, system switches to motion detect play of other file. 

· During the motion detect play process, you can not implement operations such as change time bar, begin backward playback or frame by frame playback.

· Please refer to chapter 4.4.2.3.1 Smart Search for detailed operation.   

	15
	Other channel synchronization switch to play  when playback 
	When playing the file, click the number button, system can switch to the same period of the corresponding channel to play. 

	16
	Digital zoom 
	When the system is in full-screen playback mode, left click the mouse in the screen. Drag your mouse in the screen to select a section and then left click mouse to realize digital zoom. You can right click mouse to exit.

	17
	Manually switch channel when playback 
	During the file playback process, you can switch to other channel via the dropdown list or rolling the mouse. 

This function is null if there is no record file or system is in smart search process.


Note:

All the operations here (such as playback speed, channel, time and progress) have relationship with hardware version. Some series alarm panels do not support some functions or playback speeds. 

4.4.2.3.1 Smart Search 

During the multiple-channel playback mode, double click one channel and then click the [image: image95.png]


 button, system begins smart search. System supports 396(22*18 PAL) and 330(22*15 NTSC) zones. Please left click mouse to select smart search zones. See Figure 4‑54.

[image: image96.png](From ReadWrite Hdd [~
@® REC O PIC

® AllRecord & Normal ~ « Alarm % Motion




Figure 4‑54
Click the[image: image97.png]


, you can go to the smart search playback. Click it again, system stops smart search playback. 

Important 

· System does not support motion detect zone setup during the full-screen mode.
· During the multiple-channel playback, system stops playback of rest channels if you implement one-channel smart search.
4.4.2.3.2 Accurate playback by time
Select records from one day, click the list, you can go to the file list interface. You can input time at the top right corner to search records by time. For example, click time 06:00.00 and then click Search button, you can view all the record files after 06:00.00 (The records includes current time.). Click Play button, you can see system begins play from 06:00.00. See Figure 4‑55.
Note

· After you searched files, system implement accurate playback once you click Play for the first time. 

· System does not support accurate playback for picture.

· System supports synchronization playback and non-synchronous playback. The synchronization playback supports all channels and non-synchronous playback only supports accurately playback of current select channel. 
[image: image98.png]00 : 00 : 00 ©
[N21(s0(4)

StariTime Type

13:55:05 R

Start Time
13-12-26 13:42:32
End Time
13-12-26 13:54:06
Size(KB) 6007

® AllRecord & Normal ~ « Alarm % Motion




Figure 4‑55
4.4.2.3.3 Mark Playback 
Please make sure your purchased device support this function. You can use this function only if you can see the mark playback icon on the Search interface (Figure 4‑53).

When you are playback record, you can mark the record when there is important information. After playback, you can use time or the mark key words to search corresponding record and then play. It is very easy for you to get the important video information. 

· Add Mark 
When system is playback, click Mark button
[image: image99.jpg]


, you can go to the following interface. See Figure 4‑56.
[image: image210.png]Al -
Alarm

MD
Alarm/MD
Card

PIC




 
Figure 4‑56
· Playback Mark 

During 1-window playback mode, click mark file list button[image: image100.jpg]


 in Figure 4‑53, you can go to mark file list interface. Double click one mark file, you can begin playback from the mark time.  

· Play before mark time

Here you can set to begin playback from previous N seconds of the mark time.  

Note
Usually, system can playback previous N seconds record if there is such kind of record file. Otherwise, system playbacks from the previous X seconds when there is such as kind of record. 

· Mark Manager

Click the mark manager button [image: image101.jpg]


 on the Search interface (Figure 4‑53); you can go to Mark Manager interface. See Figure 4‑57. System can manage all the record mark information of current channel by default. You can view all mark information of current channel by time. 


Figure 4‑57
· Modify

Double click one mark information item, you can see system pops up a dialogue box for you to change mark information. You can only change mark name here. 
· Delete

Here you can check the mark information item you want to delete and then click Delete button, you can remove one mark item. . 

Note

· After you go to the mark management interface, system needs to pause current playback. System resume playback after you exit mark management interface. 

· If the mark file you want to playback has been removed, system begin playbacking from the first file in the list.  

4.4.2.4 Schedule 

After system booted up, it is in default 24-hour regular mode. You can set record type and time in schedule interface.

In the main menu, from setting to schedule, you can go to schedule menu. See Figure 4‑58.

· Channel: Please select the channel number first. You can select “all” if you want to set for the whole channels.

· Week day: There are eight options: ranges from Saturday to Sunday and all.
· Pre-record: System can pre-record the video before the event occurs into the file. The value ranges from 1 to 30 seconds depending on the bit stream.

· Redundancy: System supports redundancy backup function.  It allows you backup recorded file in two disks. You can highlight Redundancy button to activate this function. Please note, before enable this function, please set at least one HDD as redundant. (Main menu->Advanced->HDD Management)  Please note this function is null if there is only one HDD. 
· Snapshot: You can enable this function to snapshoot image when an alarm occurs.

· Record types: There are four types: regular, motion detection (MD), Alarm, MD & alarm.
· Holiday setting: Here you can set holiday date. Check the box, it means current channel shall record as your holiday setup. Please go to the Period interface to set the holiday date record setup. See Figure 4‑59. Please note you need to go to Chapter 4.4.4.1 to enable Holiday function first.

Please highlight icon [image: image102.jpg]


 to select the corresponding function. After completing all the setups please click save button, system goes back to the previous menu. 
At the bottom of the menu, there are color bars for your reference. Green color stands for regular recording, yellow color stands for motion detection and red color stands for alarm recording. The white means the MD and alarm record is valid. Once you have set to record when the MD and alarm occurs, system will not record neither motion detect occurs nor the alarm occurs.

[image: image103.png]sec. Redundancy(_)Snapshot [l Holiday [l

Channel (1 [~]PreRecord(4

Period  [Thu 3 Record Type  Regular MD
Period 1 [00 :00 24 :00 | ] O
Period 2 [00 :00 24 :00 @] O
Period 3 [00 :00 24 :00 @] O
Period4 [00 :00 24 :00 @] O
Period 5 [00 :00 24 :00 @] O
Period 6 [00 :00 24 :00 @] O

1" Regular I MD || Alarm

MD&Alarm

Alarm
(]
(]
(]
(]
(]
(]

00o0o0ooo

I MD&Alarm

6

9

12

15

18 21 24





Figure 4‑58
[image: image104.png]Channel

~|PreRecord[4 |sec.Redundancy[]Snapshot[_]Holiday il

Period [Holiday || Record Type  Regular MD Alarm  MD&Alarm
Period 1 (00 :00 -24 :00
Period 2 (00 :00 -24 :00
Period 3 (00 :00 -24 :00
Period 4 (00 :00 -24 :00
Period 5 (00 :00 -24 :00
Period 6 (00 :00 -24 :00

1" Regular I MD || Alarm W MD&Alarm

3 6 9 12 15 18 21 24





Figure 4‑59
4.4.2.4.1 Quick Setup

Copy function allows you to copy one channel setup to another. After setting in channel 1, click Copy button, you can go to interface Figure 4‑60. You can see current channel name is grey such as channel 1. Now you can select the channel you wan to paste such as channel 5/6/7. If you wan to save current setup of channel 1 to all channels, you can click the first box “ALL”. Click the OK button to save current copy setup. Click the OK button in the Record interface, the copy function succeeded. 

Please note, if you select ALL in Figure 4‑60, the record setup of all channels are the same and the Copy button becomes hidden. 
[image: image105.png]



Figure 4‑60
4.4.2.4.2 Snapshot Operation

· Schedule Snapshot 

In Encode interface, click snapshot button to select snapshot mode as timing, and then set size, quality and frequency. See Figure 4‑61.

In Schedule interface, please enable snapshot function. See Figure 4‑62.
[image: image106.png]Mode

Image Size
Image Quality

Snapshot Frequency

Timing

CIF

4

1SPL





Figure 4‑61
[image: image107.png]sec. Redundancy(_)Snapshot [l Holiday [l

Channel (1 [~]PreRecord(4

Period  [Thu 3 Record Type  Regular MD
Period 1 [00 :00 24 :00 | ] O
Period 2 [00 :00 24 :00 @] O
Period 3 [00 :00 24 :00 @] O
Period4 [00 :00 24 :00 @] O
Period 5 [00 :00 24 :00 @] O
Period 6 [00 :00 24 :00 @] O

1" Regular I MD || Alarm

MD&Alarm

Alarm
(]
(]
(]
(]
(]
(]

00o0o0ooo

I MD&Alarm

6

9

12

15

18 21 24





Figure 4‑62
· Activation Snapshot

Please follow the steps listed below to enable the activation snapshot function. After you enabled this function, system can snapshot when the corresponding alarm occurred. 

· In Encode interface, click snapshot button to select snapshot mode as trigger, and then set size, quality and frequency. See Figure 4‑63.
· In Detect interface please enable snapshot function for specified channels. See Figure 4‑64. Or in alarm interface please enable snapshot function for specified channels. See Figure 4‑65.
Please refer to the following figure for detailed information. See Figure 4‑65.
[image: image108.png]Mode

Image Size
Image Quality

Snapshot Frequency

Trigger

CIF

4

1SPL





Figure 4‑63
[image: image109.png]Event Type Motion Detect | ~| Channel 1 S

Enable | ] Sensitivity 3 -

Region Setup
Period Setup Anti-dither 5 sec.

W Alarm Out BRORE Latch 10 sec.
@ Show Message [ Alarm Upload [_)Send Email
[WRecord Channel B(2)(3)(2

WPTZ Activation E Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~

(Bl Output Control

Toa ) (o) (G





Figure 4‑64
[image: image110.png]Event Type NEE ~| Alarm In 1 9
Enable [ ]

Period E

W Alarm Out BRORE Latch 10 sec.
B Show Message Send Email

[WRecord Channel B(2)(3)(2

WPTZ Activaton  (__Setup ) Delay 10 sec.
WTour BRORE

@ Snapshot BRORE

WBuzzer

W Audio Broadcast [N/A ~| OutputPort  [Talk Port ~
(Bl Output Control





Figure 4‑65
4.4.2.4.3 Priority
Please note the activation snapshot has the higher priority than schedule snapshot. If you have enabled these two types at the same time, system can activate the activation snapshot when an alarm occurs, and otherwise system just operates the schedule snapshot. 

4.4.2.5 Record 
Note: 

You need to have proper rights to implement the following operations. Please make sure the HDD has been properly installed.

Tips

On the preview interface, right click mouse you can go to the manual record interface. 

4.4.2.5.1 Basic operation
There are two modes: main stream and extra stream. There are three statuses: schedule/manual/stop. Please highlight icon“○” to select corresponding channel. See Figure 4‑66.
· Manual: The highest priority. After manual setup, all selected channels will begin ordinary recording.

· Schedule: Channel records as you have set in recording setup (Main Menu->Setting->Schedule) 
· Stop: All channels stop recording.
[image: image111.png]RecordMode Al 1 2 3 4

Schedule O @00
Manual O OOCee
Stop O 0000
Extra Stream.

Schedule O 0000
Manual O 0000
Stop O eeee





Figure 4‑66
4.4.2.5.2 Enable/disable record

Please check current channel status: “○” means it is not in recording status, “●” means it is in recording status. 

You can use mouse or direction key to highlight channel number. See Figure 4‑67.
[image: image112.png]RecordMode Al 1 2 3 4

Schedule O @00
Manual O OOCee
Stop O 0000
Extra Stream.

Schedule O 0000
Manual O 0000
Stop O eeee





Figure 4‑67
4.4.2.5.3 Enable all channel recording 

Highlight ○ below All, you can enable all channel recording. 

· All channel schedule record

Please highlight “ALL” after “Schedule”. See Figure 4‑68.

When system is in schedule recording, all channels will record as you have previously set (Main menu->Setting->Schedule). 

The corresponding indication light in front panel will turn on.

[image: image113.png]RecordMode Al 1 2 3 4

Schedule ® o000
Manual O 0000
Stop O 0000
Extra Stream.

Schedule ® o000
Manual O 0000
Stop O 0000





Figure 4‑68
· All channel manual record

Please highlight “ALL” after “Manual.” See Figure 4‑69.

When system is in manual recording, all scheduled set up you have set in will be null ((Main menu->Setting->Schedule)). 

You can see indication light in front panel turns on, system begins manual record now. 

[image: image114.png]RecordMode Al 1 2 3 4

Schedule O 0000
Manual ® o000
Stop O 0000
Extra Stream.

Schedule O 0000
Manual ® o000
Stop O 0000





Figure 4‑69
· Stop all channel recording 

Please highlight “ALL” after “Stop”. See Figure 4‑70. 

System stops all channel recording no matter what mode you have set in the menu (Main menu->Setting->Schedule) 

[image: image115.png]RecordMode Al 1 2 3 4

Schedule O 0000
Manual O 0000
Stop ® 9000
Extra Stream.

Schedule O 0000
Manual O 0000
Stop ® 9000





Figure 4‑70
4.4.2.6 Display

Display setup interface is shown as below. See Figure 4‑71.


· Transparency: Here is for you to adjust transparency. The value ranges from 128 to 255.
· Channel name: Here is for you to modify channel name. System max support 25-digit (The value may vary due to different series). Please note all your modification here only applies to alarm panel local end. You need to open web or client end to refresh channel name.
· Time display: You can select to display time or not when system is playback. 

· Channel display: You can select to channel name or not when system is playback.
· Resolution: There are four options: 1920×1080, 1280×1024(default),1280×720,1024×768. Please note the system needs to reboot to activate current setup. 
· Image enhance: Check the box; you can optimize the margin of the preview video. 
· Tour setup: Here you can activate tour function. Click Setup button, you can see an interface shown as in Figure 4‑73.
· Enable tour: Highlight box here to enable this function. 
· Interval: System supports 1/4-window tour. Input proper interval value here. The value ranges from 5-120 seconds. It is for schedule tour/alarm/motion detect tour. 

· Split: You can select window split mode from the dropdown list. 

· Channel group: It is for you to view channel names under current window split mode. You can add/delete channel here. Double click one item, you can change channel group setup to edit it. Now system max supports 24 channels.   

· Add: Under specified window split mode, click it to add channel group.

· Delete: Click it to remove selected channel group. 

· Move up: Click it to move current selected channel up.

· Move down: Click it to move current selected channel down.

· Default: Click it to restore default setup.

Please highlight icon [image: image116.jpg]


 to select the corresponding function.

After completing all the setups please click save button, system goes back to the previous menu.
 [image: image117.png]GUI

Transparency 200 Channel Name
Time Display | ] Channel Display [l

Resolution 1280x1024__|~| Image Enhance [l

Tour Setup

Motion Tour Type |View 1 ~| Alarm Tour Type |View 1 v

Zero-Ch Encode





Figure 4‑71
In Figure 4‑71, click modify button after channel. You can see an interface is shown as in Figure 4‑72. Please note all your modification here applies to local end only. You need to refresh web or client-end to get the latest channel name. System max support 25-digital character. 

[image: image118.png]CAM 1 CAM 1| v 2 CAM 2
CAM 3 CAM 3 CAM 4 CAM 4





Figure 4‑72
[image: image119.png]Enable Tour [l Interval (5 sec.
Window Split [View 4 N
1 v Channel Group

1 |v 1234

< ]! >

Add ) ( Delete | Up J[ Down |

Default Cancel





Figure 4‑73
· Zero-Channel Encoding: Click Setup button, you can go to the following interface.  Figure 4‑74. Here you can enable and set zero-channel encoding function so that you can view several video sources at one channel. 

· Enable: This function is disabled by default. Check the box here to enable this function so that you can control the zero-channel encoding function at the WEB.

· Compression: System default setup is H.264. You can set according to device capability. 

· Resolution: The resolution value may vary due to different device capabilities. Please select from the dropdown list.

· Frame rate: The frame rate value may vary due to different device capabilities. Please select from the dropdown list.

· Bit Rate: The bit rate value may vary due to different device capabilities and frame rate setups. Please select from the dropdown list.

· Save: Click the Save button to save current setup. If this function is disabled, you can not operate zero-channel encoding function at the WEB, the video is black or null even you operate when the function is disabled. After you enabled this function, login the Web and you can select zero-channel encoding mode at the right corner of the interface[image: image120.png]


. Select a mode; you can view the local preview video. 

[image: image121.png]Enable

Compression
Resolution
Frame Rate(FPS)
Bit Rate(Kb/S)

H.264

CIF

25

1024

K1





Figure 4‑74
4.4.2.7 Broadcast 

The broadcast interface is shown as in Figure 4‑75. Here you can set audio file volume. 

[image: image122.png]File Name Size Audition Delete

Left Space/Total Space 8.00 MB/ 8.00 MB Volume el

Schedule Playback





Figure 4‑75
Click Schedule playback button, you can see an interface shown as below. See Figure 4‑76.

Here you can set schedule playback period and file. 

[image: image123.png]Period Audio Broadcast Interval Output Port
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/
00 :00 -24 :00 N/A - 60 Minute Talk Port| ~/





Figure 4‑76
4.4.2.8 D/A Switch  

Please select corresponding mode according to your actual situation. See Figure 4‑77.
· Analog channel: It is the max analog channel amount system supports.

· Digital channel: It is the max network camera channel amount system supports. 

After you selected a mode, you can see its max encode capability. 
[image: image124.png]Analog Channel Digital Channel

® Mode 1: 4 0
O Mode 2: 3 1
O Mode 3: 2 2

Mode 1: Max Capability
Analog Channel: 4@960H(25FPS)

Save Cancel





Figure 4‑77
4.4.3 Information 

Here is for you to view system information. See Figure 4‑78.

It has seven buttons: HDD, BPS, log, version, online user, network, MCU. 
[image: image125.png]HDD INFO

o

VERSION

®
&

ONLINE USERS

NETWORK





Figure 4‑78
4.4.3.1 HDD 

Here is to list hard disk type, total space, free space, video start time and status. See X391H391H391H See Figure 4‑79.○ means current HDD is normal. X means there is error. - means there is no HDD. 

If disk is damaged, system shows as “?”. Please remove the broken hard disk before you add a new one.

Once there is a hard disk confliction, please check hard disk time and system time is the same or not. Please go to setting then general to modify system time.  At last, reboot the system to solve this problem. 
After system booted up, if there is any confliction, system goes to HDD information interface directly. Please note, system does not ask you to deal with it forcedly.

When HDD confliction occurs, you can check system time and HDD time are identical or not. If they are not identical, please go to General (Chapter 5.3.1) to adjust system time or go to HDD Management (Chapter 5.5.1) to format HDD and then reboot the alarm panel.
[image: image126.png]SATA 1
o
1* Type Total Space Free Space Status SMAR.T.
All - 232.79 GB 0.00 MB - -
1 Read/Write 232.79 GB 0.00 MB Normal Normal
< ]! >
4 Page Up ™! Page Down Fr [ View recording times





Figure 4‑79
Tips:

Please click Fn button or left click mouse to view HDD record time and HDD type and time.
Double click one HDD information; you can see the HDD SMART information. 

4.4.3.2 BPS

Here is for you to view current video data stream (KB/s) and occupied hard disk storage (MB/h). See Figure 4‑80.

[image: image127.png]Channel Kb/S MB/MH Wave

1 64 31 ] [
2 67 32 ] [
3 67 32 ] [
4 65 31 ] [





Figure 4‑80
4.4.3.3 Log 

Here is for you to view system log file. System lists the following information. See Figure 4‑81.

Log types include system operation, system configuration, data management, alarm event, record operation, log clear, file operation and etc. 

· Start time/end time: Pleased select start time and end time, then click search button. You can view the log files in a list. System max displays 100 logs in one page. It can max save 1024 log files.  Please use page up/down button on the interface or the front panel to view more. 

· Backup: Please select a folder you want to save; you can click the backup button to save the log files. After the backup, you can see there is a folder named Log_time on the backup path. Double click the folder, you can see the log file 

· Details: Click the Details button or double click the log item, you can view the detailed information. See Figure 4‑82. Here you can use rolling bar to view information, or you can use Page up/Page down to view other log information. For the alarm event log such as video loss, you can click the Playback button at the bottom right corner to playback.  
[image: image128.png]Type All M

Start Time (2013 - 12- 26 |00: 00: 00
End Time (2013 - 12- 27 [00: 00: 00 [ Details ) ( Search

38 Log Time Event e
27 |2013-12-26 13:55:17 <Local Zone Event: 2>
28 |2013-12-26 13:55:17 <Local Zone Event: 3>
29 |2013-12-26 13:55:17 <Local Zone Event: 4>
30 |2013-12-26 13:55:17 <Local Zone Event: 5>
31 |2013-12-26 13:55:17 <Local Zone Event: 6>
32 |2013-12-26 13:55:17 <Local Zone Event: 7>
33 |2013-12-26 13:55:17 <Local Zone Event: 8>
34 |2013-12-26 13:56:37 User logged in<888888> —
35 |2013-12-26 13:59:57 Playback file[2013-12-26 13:59:57]
36 |2013-12-26 14:10:57 User logged out<888888>

37 |2013-12-26 14:21:57 User logged in<888888>

38 [2013-12-26 14:25:37 Playback file[2013-12-26 14:25:37] N
14 Page Up M Page Down Clear

PrePage 1/1(Curent Page/Total Page) Jump to

(Go





Figure 4‑81
[image: image129.png]Log Time 2013-12-26 14:25:37

Log Type File operation-->Record Playback

Type Record Playback e

Type HardDisk —

IP Address Login Local

User 888888 |

Channel 1 -

File Name 2013-12-26 13:55:05---2013-12-26 14:22:02 R

Channel 2

File Name 2013-12-26 13:55:05---2013-12-26 14:22:04 R N
Previous Next OK





Figure 4‑82
4.4.3.4 Version 

Here is for you to view hardware features, software version, built date, release SN information and etc. You can also update system here. See Figure 4‑83.
· Start: Please insert the USB device that have the update file to the device and then click the Start button to begin the update. 

Important 

Please make sure the upgrade file name shall be update.bin.

[image: image130.png]Device Model Alarm Panel
Channels 4
Alarm In 8
Alarm Out 4

Hardware Version V1.0
System Version  2.616.0000.4

Build Date 2013-12-25

Web 3.0.0.1

Serial No. YPQ3KQ144D00024
UPGRADE

If you need to upgrade system now,please insert USB upgrade disk,then press the start
button to start upgrade.Don't shut down the power during upgrade!





Figure 4‑83
4.4.3.5 Online Users 

Here is for you manage online users. See Figure 4‑84.

You can disconnect one user or block one user if you have proper system right. Max disconnection setup is 65535 seconds.
System refreshes current interface every five seconds to detect there is any newly added or deleted user. 
[image: image131.png]User Name IP
admin 10.10.6.5
admin 10.10.6.5

3

Disconnect) [ Block for | [60 sec.





Figure 4‑84
4.4.3.6 Network 

In this interface, you can see network test and network load information. 

4.4.3.6.1 Network Test 

Network test interface is shown as in Figure 4‑85.

· Destination IP: Please input valid IPV4 address and domain name. 

· Test: Click it to test the connection with the destination IP address. The test results can display average delay and packet loss rate and you can also view the network status as OK, bad, no connection and etc. 

· Network Sniffer backup: Please insert USB2.0 device and click the Refresh button, you can view the device on the following column. You can use the dropdown list to select peripheral device. Click Browse button to select the snap path. The steps here are same as preview backup operation. 

You can view all connected network adapter names (including Ethernet, PPPoE, WIFI, and 3G), you can click the button [image: image132.png]


 on the right panel to begin Sniffer. Click the grey stop button to stop. Please note system can not Sniffer several network adapters at the same time. 
After Sniffer began, you can exit to implement corresponding network operation such as login WEB, monitor. Please go back to Sniffer interface to click [image: image133.png]


 stop Sniffer. System can save the packets to the specified path. The file is named after “Network adapter name+time”. You can use software such as Wireshark to open the packets on the PC for the professional engineer to solve complicated problems.  

[image: image134.png]Network Test.

Desinsion P (10364786

TestRosul  Average Delay:3 630ms PacketLoss Rate:0% 3G Stte:OK.

Network Snifer Packe! Backup.

[ —

L ——

[Name. 3 Sfer Packer e Snfle Packet Backup |
TN 043363 oe >





Figure 4‑85
4.4.3.6.2 Network Load 

Network load is shown as in Figure 4‑86. Here you can view the follow statistics of the device network adapter. 

Here you can view information of all connected network adapters. The connection status is shown as offline if connection is disconnected. Click one network adapter, you can view the flow statistics such as send rate and receive rate at the top panel

[image: image135.png]:

I
T
ISR

LA SendSpeed 367KBS Receive Spead  USMBS

[[Name  WAC Address  Saws P Address __Type _ MTU
LANT 9002a9.9442f1 Succeed 1043363  Eemer 1500





Figure 4‑86
4.4.3.7 MCU 

Here is for you to view micro chip unit information. See Figure 4‑84.

[image: image136.png]MCU Software Version V/1.000.0000.0
Online Alarm Keyboard No. 0





Figure 4‑87
4.4.4 System
In main menu, highlight system icon and double click mouse. System interface is shown as below. See Figure 4‑88.
It has eleven buttons: general, network, account, HDD manager, RS485, auto maintain, backup, config backup, text overlay, default and shutdown. 
[image: image137.png]E &5 &

GENERAL NETWORK ACCOUNT HDD MANAGE RS485

2 P @ >

AUTO MAINTAIN BACKUP CONFIG BACKUP TEXT OVERLAY  DEFAULT

‘ X

SHUTDOWN

»





Figure 4‑88
4.4.4.1 General 

General setting includes the following items. See Figure 5‑3. 

· System time: Here is for you to set system time 

· Date format: There are three types: YYYYY-MM-DD: MM-DD-YYYYY or DD-MM-YYYY.

· Date separator: There are three denotations to separate date: dot, beeline and solidus.
· DST: Here you can set DST time and date. Please enable DST function and then click set button. You can see an interface is shown as in Figure 4‑90. Here you can set start time and end time by setting corresponding week setup. In Figure 4‑90, enable date button, you can see an interface is shown as in Figure 4‑91. Here you can set start time and end time by setting corresponding date setup.
· Time format: There are two types: 24-hour mode or 12-hour mode. 

· Language: System supports various languages: Chinese (simplified), Chinese (Traditional), English, Italian, Japanese, French, Spanish (All languages listed here are optional. Slight difference maybe found in various series.)

· HDD full: Here is for you to select working mode when hard disk is full. There are two options: stop recording or rewrite. If current working HDD is overwritten or the current HDD is full while the next HDD is no empty, then system stops recording, If the current HDD is full and then next HDD is not empty, then system overwrites the previous files.
· Video standard: There are two formats: NTSC and PAL.

· Pack duration: Here is for you to specify record duration. The value ranges from 1 to 120 minutes. Default value is 60 minutes.
· Real-time playback: It is to set playback time you can view in the preview interface (chapter 4.2 Live Viewing). The value ranges from 5 to 60 minutes. 
· Device ID: Please input a corresponding device name here. 
· Holiday setup: Click it you can see an interface shown as in Figure 4‑92. Here you can set holiday date. Please go to chapter 4.4.2.4 Schedule to view or set the holiday date record setup.
· When you enable Holiday settings and schedule setup at the same time, holiday setting has the priority. If the selected day is a holiday, then system records as you set in holiday setting. If it is not a holiday, system records as you set in Schedule interface. Please note you need to go to chapter 4.4.2.4 Schedule to enable Holiday setup. Otherwise you can not enable holiday record setup. 
· Please note, there is no year setup on the holiday setup. For example, if you set 30th Oct, 2012 as a holiday, then the date of 30th Oct in each year will be set as a holiday. So, general speaking, your holiday setup in other year may also affect the holiday setup in 2012. 

· Auto logout: Here is for you to set auto logout interval once login user remains inactive for a specified time. Value ranges from 0 to 60 minutes.
· Mouse property: Click mouse setup button, you can go to mouse setup interface. See Figure 4‑93. You can set double click speed via dragging the slide bard. You can Click Default button to restore default setup. 

· Startup wizard: Check the box here, system goes to the startup wizard when the device boots up the next time. Otherwise, it goes to the system login interface directly. 

· Navigation bar: Check the box here, system displays the navigation bar on the interface. 

Note:
Since system time is very important, do not modify time casually unless there is a must!

Before your time modification, please stop record operation first!

After completing all the setups please click save button, system goes back to the previous menu.

[image: image138.png]System Time (2013 - 12-26 |14: 38: 46 [ save )

Basic Settings.

Date Format  [YYYY MM DD|~| DST Set

Date Separator |- v Time Format 24-HOUR v
Device Settings.

Language ENGLISH | HDD Full Overwrite 7
Video Standard [PAL N Pack Duration (60 min.
Device No. 8 Realtime Play (5 min.
Device ID Alarm

Other Settings.

B Holiday Mouse Property E)

B Navigation Bar Auto Logout 10 min.
B Startup Wizard





Figure 4‑89
[image: image139.png]@ Dayof Week O Date

Start: [Jan > |1st ~| [Su_ [~/ |00

1 00

End: [Jan MELED ~| [Su_ [~/ |00

1 00





Figure 4‑90
[image: image140.png]O Dayof Week ® Date

Start: [@]2014 - 01

01: 00

End:  [@]2014 - 01

01: 00





Figure 4‑91
[image: image141.png]a

S

Fr

6] 7]

5]

12)[(13][ 14

19)(20](21])

26](27])( 28]

~|(2013
Th

Tu We

Mo

Su

2](3](4]

1]

9] (10 [(11]

8]

=
=S
~
-
©|l
bk
0| ley
RN

H
u





Figure 4‑92
[image: image142.png]Double-click Speed.

Slow | Fast

[ Default | ( [e]4 ] [ Cancel |





Figure 4‑93
4.4.4.2 Network 

Here is for you to input network information.
The IPv4 interface is shown as in Figure 4‑94 and the IPv6 interface is shown as in Figure 4‑95.

· IP Version: There are two options: IPv4 and IPv6. Right now, system supports these two IP address format and you can access via them. 

· MAC address: The host in the LAN can get a unique MAC address. It is for you to access in the LAN. It is read-only. 

· IP address: Here you can use up/down button ((() or input the corresponding number to input IP address. Then you can set the corresponding subnet mask the default gateway. 

· Subnet prefix: The input value ranges from 0 to 128. It is to mark a specified network MAC address. Usually it includes an organization of multiple-level. 

· Default gateway: Here you can input the default gateway. Please note system needs to check the validity of all IPv6 addresses. The IP address and the default gateway shall be in the same IP section. That is to say, the specified length of the subnet prefix shall have the same string. 

· DHCP: It is to auto search IP. When enable DHCP function, you can not modify IP/Subnet mask /Gateway. These values are from DHCP function. If you have not enabled DHCP function, IP/Subnet mask/Gateway display as zero. You need to disable DHCP function to view current IP information.  Besides, when PPPoE is operating, you can not modify IP/Subnet mask /Gateway. 
· TCP port: Default value is 37777. You can change if necessary.
· UDP port: Default value is 37778. You can change if necessary.
· HTTP port: Default value is 80.

· RTSP port: Default value is 554.

Important: System needs to reboot after you changed and saved any setup of the above four ports. Please make sure the port values here do not conflict.
· Max connection: system support maximal 128 users. 0 means there is no connection limit.
· MTU: It is to set MTU value of the network adapter. The value ranges from 1280-7200 bytes. The default setup is 1500 bytes. Please note MTU modification may result in network adapter reboot and network becomes off. That is to say, MTU modification can affect current network service. System may pop up dialog box for you to confirm setup when you want to change MTU setup. Click OK button to confirm current reboot, or you can click Cancel button to terminate current modification. Before the modification, you can check the MTU of the gateway; the MTU of the alarm panel shall be the same as or is lower than the MTU of the gateway. In this way, you can reduce packets and enhance network transmission efficiency. 

The following MTU value is for reference only. 

· 1500: Ethernet information packet max value and it is also the default value. It is the typical setup when there is no PPPoE or VPN. It is the default setup of some router, switch or the network adapter. 

· 1492: Recommend value for PPPoE.

· 1468: Recommend value for DHCP.

· Preferred DNS server: DNS server IP address.

· Alternate DNS server: DNS server alternate address.

· Transfer mode: Here you can select the priority between fluency/video qualities.

· LAN download: System can process the downloaded data first if you enable this function. The download speed is 1.5X or 2.0X of the normal speed.
Important

For the IP address of IPv6 version, default gateway, preferred DNS and alternate DNS, the input value shall be 128-digit. It shall not be left in blank.

After completing all the setups please click save button, system goes back to the previous menu.

[image: image143.png]IP Version IPv4

IP Address 10 . 15 - 144 DHCP

Subnet Mask 255 . 255 0

Default Gateway [10 . 15 1

TCP Port 37777 HTTP Port 80

UDP Port 37778 HTTPS PORT (443

RTSP Port 554 MTU 1500

Preferred DNS [ 10 1 - 80

Alternate DNS 10 1 - 81

Max Connection (128 LAN Download





Figure 4‑94
[image: image144.png]IP Version IPvé 9
LinkAddr
IP Address /|64

Default Gateway

TCP Port

HTTP Port 80
HTTPS PORT (443
MTU 1500

UDP Port

RTSP Port

Preferred DNS

Alternate DNS

Max Connection (128 LAN Download

NETWORK SETTING





Figure 4‑95
4.4.4.2.1 Network Setting 
Network setting interface is shown as in Figure 4‑96. Please check the box to enable corresponding function and then double click current item to go to setup interface. 

[image: image145.png]IP FILTER

NTP time.windows.com : 60
MULTICAST 239.255.42.42

PPPoE

DDNS No Available DDNS Setup
UPnP Port Forwarding

3G

WIFI No Connection

EMAIL MailServer : 25

FTP Record FTP : 0.0.0.0
ALARM CENTRE Private : 10.1.0.2
SNMP

NETWORK PRIORITY LAN

REGISTER 0.0.0.0: 8000

Default Cancel





Figure 4‑96
4.4.4.2.1.1 IP Filter 

IP filter interface is shown as in Figure 4‑97. You can add IP in the following list. The list supports max 64 IP addresses. System supports valid address of IPv4 and IPv6. Please note system needs to check the validity of all IPv6 addresses.
After you enabled trusted sites function, only the IP listed below can access current alarm panel. 

If you enable blocked sites function, the following listed IP addresses can not access current alarm panel.

· Enable: Highlight the box here, you can check the trusted site function and blocked sites function. You can not see these two modes if the Enable button is grey. 

· Type: You can select trusted site and blacklist from the dropdown list. You can view the IP address on the following column. 

· Start address/end address: Select one type from the dropdown list, you can input IP address in the start address and end address. Now you can click Add IP address or Add IP section to add.

a) For the newly added IP address, it is in enable status by default. Remove the √ before the item, and then current item is not in the list. 

b) System max supports 64 items. 

c) Address column supports IPv4 or IPv6 format. If it is IPv6 address, system can optimize it. For example, system can optimize aa:0000: 00: 00aa: 00aa: 00aa: 00aa: 00aa as aa:: aa: aa: aa: aa: aa: aa. 

d) System automatically removes space if there is any space before or after the newly added IP address. 

e) System only checks start address if you add IP address. System check start address and end address if you add IP section and the end address shall be larger than the start address.

f) System may check newly added IP address exists or not. System does not add if input IP address does not exist.  

· Delete: Click it to remove specified item. 

· Edit: Click it to edit start address and end address. See Figure 4‑99. System can check the IP address validity after the edit operation and implement IPv6 optimization. 
· Default: Click it to restore default setup. In this case, the trusted sites and blocked sites are both null.

Note:

· If you enabled trusted sites, only the IP in the trusted sites list can access the device.

· If you enabled blocked sites, the IP in the blocked sites can not access the device. 

· System supports add MAC address. 
[image: image146.png]M Enable @ Trusted Sites

O Blocked Sites

Type

Trusted Sites |~/

Start Address  [10.10.6.10
End Address [10.10.6.18

(Add IP Address |

(Add IP Section )

Start Address End Address Edit Delete
/| 10.10.6.10 10.10.6.18 7/ x
3
Default Cancel





Figure 4‑97
[image: image147.png]Restricted Type |Trusted Sites |~

IP Version IPv6 9

IP Address 2001:255::258:bc52 [ AddIP )
12247

2001:255::258:bc52

Delete IP

Default Cancel





Figure 4‑98
[image: image148.png]Start Address

End Address

10.10.6.21

10.10.6.30





Figure 4‑99
4.4.4.2.1.2 NTP Setup 

You need to install SNTP server (Such as Absolute Time Server) in your PC first. In Windows XP OS, you can use command “net start w32time” to boot up NTP service.
NTP setup interface is shown as in Figure 4‑100.

· Host IP: Input your PC address.

· Port:  This series alarm panel supports TCP transmission only. Port default value is 123.

· Update interval: minimum value is 1. Max value is 65535. (Unit: minute)
· Time zone: select your corresponding time zone here.
· Manual update: It allows you to synchronize the time with the server manually. 
Here is a sheet for your time zone setup.

	City /Region Name
	Time Zone

	London
	GMT+0

	Berlin
	GMT+1

	Cairo
	GMT+2

	Moscow
	GMT+3

	New Deli
	GMT+5

	Bangkok
	GMT+7

	Beijing (Hong Kong)
	GMT+8

	Tokyo
	GMT+9

	Sydney
	GMT+10

	Hawaii
	GMT-10

	Alaska
	GMT-9

	Pacific Time(P.T)
	GMT-8

	American  Mountain Time(M.T)
	GMT-7

	American Central Time(C.T)
	GMT-6

	American Eastern Time(E.T)
	GMT-5

	Atlantic Time 
	GMT-4

	Brazil
	GMT-3

	Middle Atlantic Time 
	GMT-2


[image: image149.png]Server IP

Port

Time Zone GMT+08:00 9

Update Period (60 min.

( Default | [ [e]4 ) [ Cancel ) (Manual Update |





Figure 4‑100
4.4.4.2.1.3 Multicast 

Multicast setup interface is shown as in Figure 4‑101. 

[image: image150.png]IP Address [239 . 255 . 42 . 42
Port 36666

[ Default | ( [e]4 ] [ Cancel |





Figure 4‑101
Here you can set a multiple cast group. Please refer to the following sheet for detailed information. 
· IP multiple cast group address 

-224.0.0.0-239.255.255.255

-“D” address space

· The higher four-bit of the first byte=”1110”
· Reserved local multiple cast group address

-224.0.0.0-224.0.0.255

-TTL=1 When sending out telegraph
-For example

224.0.0.1 All systems in the sub-net

224.0.0.2 All routers in the sub-net

224.0.0.4  DVMRP router

224.0.0.5  OSPF router

224.0.0.13 PIMv2 router 

· Administrative scoped addressees
-239.0.0.0-239.255.255.255

-Private address space

· Like the single broadcast address of RFC1918 

· Can not be used in Internet transmission 

· Used for multiple cast broadcast in limited space. 

Except the above mentioned addresses of special meaning, you can use other addresses. For example:

Multiple cast IP: 235.8.8.36
Multiple cast PORT: 3666.

After you logged in the Web, the Web can automatically get multiple cast address and add it to the multiple cast groups. You can enable real-time monitor function to view the view.

Please note multiple cast function applies to special series only.

4.4.4.2.1.4 PPPoE

PPPoE interface is shown as in Figure 4‑102. 

Input “PPPoE name” and “PPPoE password” you get from your ISP (Internet service provider). 

Click save button, you need to restart to activate your configuration. 

After rebooting, alarm panel will connect to internet automatically. The IP in the PPPoE is the alarm panel dynamic value. You can access this IP to visit the unit. 

[image: image151.png]User Name L3

Password

IPAddress [0 . 0 . 0 . O

[ Default ( [e]4 ] [ Cancel |





Figure 4‑102
4.4.4.2.1.5 DDNS Setup 

DDNS setup interface is shown as in Figure 4‑103.

You need a PC of fixed IP in the internet and there is the DDNS software running in this PC. In other words, this PC is a DNS (domain name server).

In network DDNS, please select DDNS type and highlight enable item. Then please input your PPPoE name you get from you IPS and server IP (PC with DDNS ) . Click save button and then reboot system.

Click save button, system prompts for rebooting to get all setup activated.

After rebooting, open IE and input as below: 

http：//(DDNS server IP)/(virtual directory name)/webtest.htm 

e.g.: http：//10.6.2.85/ alarm panel _DDNS/webtest.htm.) 

Now you can open DDNSServer web search page.
[image: image152.png]DDNS Type  [Quick DDNS ~| I Enable

Server IP www.quickddns.com

Port 12366

Domain Mode ® Default Domain O Custom Domain Name L3
Domain Name (52544CFA181C -quickddns.com

Email Address Please input email address.

Note: System will reclaim the domain that is idle for more than one year. You can geta
notification email one month before the reclaim if your email address setup is right.

[ Test [ [e]4 ] [ Cancel |





Figure 4‑103
Please note NNDS type includes: CN99 DDNS, NO-IP DDNS, Quick DDNS and Dyndns DDNS. All the DDNS can be valid at the same time, you can select as you requirement. 

Quick DDNS function shall work with special DDNS server and special Professional Surveillance Software (PSS).
Quick DDNS and Client-end Introduction 

1) Background Introduction 

Device IP is not fixed if you use ADSL to login the network. The DDNS function allows you to access the alarm panel via the registered domain name. Besides the general DDNS, the Quick DDNS works with the device from the manufacturer so that it can add the extension function. 

2) Function Introduction 

The quick DDNS client has the same function as other DDNS client end. It realizes the bonding of the domain name and the IP address. Right now, current DDNS server is for our own devices only. You need to refresh the bonding relationship of the domain and the IP regularly. There is no user name, password or the ID registration on the server. At the same time, each device has a default domain name (Generated by MAC address) for your option. You can also use customized valid domain name (has not registered.). 
3) Operation 

Before you use Quick DDNS, you need to enable this service and set proper server address, port value and domain name. 

· Server address：www.quickddns.com
· Port number：80
· Domain name：There are two modes: Default domain name and customized domain name. 
Except default domain name registration, you can also use customized domain name (You can input your self-defined domain name.)  After successful registration, you can use domain name to login installed of the device IP. 

· User name: It is optional. You can input your commonly used email address. 

Important 

· Do not register frequently. The interval between two registrations shall be more than 60 seconds. Too many registration requests may result in server attack.

· System may take back the domain name that is idle for one year. You can get a notification email before the cancel operation if your email address setup is OK. 

4.4.4.2.1.6 UPNP

The UPNP protocol is to establish a mapping relationship between the LAN and the WAN. Please input the router IP address in the LAN in Figure 4‑95. Double click the UPNP item in Figure 4‑96, you can see the following interface. See Figure 4‑104.

· UPNP  on/off ：Turn on or off the UPNP function of the device.

· Status:  When the UPNP is offline, it shows as “Unknown”. When the UPNP works it shows “Success”
· Router LAN IP: It is the router IP in the LAN.

· WAN IP: It is the router IP in the WAN.
· Port Mapping list： The port mapping list here is the one to one relationship with the router’s port mapping setting. 

· Enable Switch 
[image: image153.emf]: ：It shows that the function of port mapping is enabled in this port.

· List：

· Service name：Defined by user.
· Protocol： Protocol type

· Internal port：Port that has been mapped in the router.
· External port：Port that has been mapped locally.

· Default: UPNP default port setting is the HTTP, TCP and UDP of the alarm panel.

· Add to the list: Click it to add the mapping relationship. 

· Delete: Click it to remove one mapping item. 

Double click one item; you can change the corresponding mapping information. See Figure 4‑105.

Important:

When you are setting the router external port, please use 1024~5000 port. Do not use well-known port 1~255 and the system port 256~1023 to avoid conflict.

For the TCP and UDP, please make sure the internal port and external port are the same to guarantee the proper data transmission. 

[image: image154.png]PAT ON @ OFF O
UPNP Status  [unknow

Router LANIP [0 . 0 R 0 .0

WAN IP 0 .0 .0 .0

PAT Table
3 |Service Name Protocol Int.Port Ext.Port
1 |V HTTP TCP 80 80
2 || TCP TCP 37777 37777
3 |v/UDP UDP 37778 37778

Default ) (Addtothe List) [ Delete ) Cancel





Figure 4‑104
[image: image155.png]Service Name

Protocol
Int.Port
Ext.Port

TCP

TCP

37777

37777





Figure 4‑105
4.4.4.2.1.7 3G 

3G setup interface is shown as below. See Figure 4‑106. 
Please refer to the following contents for the parameter information. 

· Pane 1: Display 3G signal intensity after you enabled 3G function. 

· Pane 2: Display 3G module configuration information after you enabled 3G function. 

· Pane 3: Display 3G module status information after you enabled 3G function. 

It is to display current wireless network signal intensity such as EVDO, CDMA1x, WCDMA, WCDMA, EDGE and etc. 

· 3G module: It is to display current wireless network adapter name.

· 3G Enable/Disable: Check the box here to enable 3G module. 

· Network type: There are various network types for different 3G network modules. You can select according to your requirements.

· APN: It is the wireless connection server. It is to set you access the wireless network via which method. 

· AUTH: It is the authentication mode. It supports PAP/CHAP.

· Dial number: Please input 3G network dialup number you got from your ISP.
· User name: It is the user name for you to login the 3G network.

· Password: It is the password for you to login the 3G network.

· Pulse interval: You can set dialup duration. Once you disable the extra stream, the connection time begins. For example, if you input 5 seconds here, then 3G network connection period is 5 seconds. The device automatically disconnect when time is up.  If there is no extra stream, 3G network connection is valid all the time. If the alive time is 0, then the 3G network connection is valid all the time.

· Dial: Here you can enable or disable 3G network connection/disconnection manually. 

· 3G wireless network: Here is to display wireless network status, SIM card status, dial status. If the 3G connection is OK, then you can see the device IP address the wireless network automatically allocates. 

[image: image156.png]No signal

3G Network1
Network Type
APN

AUTH

Dial Number
User Name
User Password

Pulse Interval

3G Wireless Network.

Module State :
SIM State -
PPP State -

pPPS

Auto

ctnet

PAP

#7177

card

0

3G Enable/Disable

sec.

IP Address -
Subnet Mask -

Default Gateway -

[ oK

) ( Cancel )( Apply )





Figure 4‑106
4.4.4.2.1.8 WIFI 

You can view the WIFI connection status in the Network Setting interface. See Figure 4‑107.

You can view current connection status and IP address if there is a connection. 
[image: image157.png]| IP FILTER Trusted Sites : 0
/| NTP time.windows.com : 60
/| MULTICAST 239.255.42.42
/| PPPOE
/| DDNS Private DDNS :
| UPNP Port Forwarding
3G SETTING
- WIFI SETTING No connection
& EMAILE MailServer : 25
FTP Record FTP :
ALARM SERVER  Private : 10.1.0.2
SNMP
REGISTER 0.0.0.0: 8000
Default Cancel





Figure 4‑107
The WIFI interface is shown as below. See Figure 4‑108.

· Auto connect WIFI: Check the box here, system automatically connects to the previous WIFI hotspot. 

· Refresh: You can click it to search the hotspot list again. It can automatically add the information such as the password if you have set it before. 

· Disconnect: Here you can click it to turn off the connection. 

· Connect: Here you can click it to connect to the hotspot. System needs to turn off current connection and then connect to a new hotspot if there is connection of you selected one. See Figure 4‑109.

[image: image158.png]Auto Connect WIFI [l

7 SSID Signal Intensity | WIFI Working Information
1 |TP-LINK_52019C 1
2 10539 Current Hotspot No connection
3 Xingjiaibn
4 |[TP-LINK_076ACA IP Address ...
5 14029
6 10388 SubnetMask ...
7 10333
Gateway ...

Refresh | (Connection ) (DisConnect) | [e]4 ) ( Cancel )( Apply |





Figure 4‑108
[image: image159.png]Ssib kqy

Connection Status No connection

Verification Type WEP N

Encrypt Type AUTO N

Connection Password (esssssssse

IP Address 0 .0 .0 .0 |EDHCP
Subnet Mask 0 .0 .0 .0

Gateway 0 .0 .0 .0

(oK ) Cancel ) (Connection )





Figure 4‑109
After successfully connection, you can see the following interface. You can see it is connected now. See Figure 4‑109.

[image: image160.png]WDHCP

Ssib kqy
Connection Status Connected
Verification Type WEP N
Encrypt Type AUTO N
Connection Password (esssssssse
IP Address 192 . 168 1 . 100
Subnet Mask 255 . 255 . 255 0
Gateway 192 . 168 1 1
[ oK ) [ Cancel | (DisConnect )





Figure 4‑110
· WIFI working status: Here you can view current connection status. 
Please note:

· After successful connection, you can see WIFI connection icon at the top right corner of the preview interface. 

· When the hotspot verification type is WEP, system displays as AUTO since the device can not detect its encryption type. 

· System does not support verification type WPA and WPA2. The display may become abnormal for the verification type and encryption type. 
After device successfully connected to the WIFI, you can view the hotspot name, IP address, subnet mask, default gateway and etc. See Figure 4‑111.
[image: image161.png]Auto Connect WIFI [l

7 SSID Signal Intensity | WIFI Working Information

1 |TP-LINK_52019C 1

2 Xingjiaibn Current Hotspot kqy

3 14029

4 [TP-LINK_076ACA IP Address 192.168.1.100
5 kay

[ 10388 SubnetMask  255.255.255.0
7 10333

Gateway 192.168.1.1

Refresh | (Connection | (Disconnect) | OK ) ( Cancel )( Apply |





Figure 4‑111
4.4.4.2.1.9 Email

The email interface is shown as below. See Figure 4‑112.

· SMTP server: Please input your email SMTP server IP here.

· Port: Please input corresponding port value here.

· User name:  Please input the user name to login the sender email box.

· Password: Please input the corresponding password here. 

· Sender: Please input sender email box here.

· Title: Please input email subject here. System support English character and Arabic number. Max 32-digit.

· Receiver: Please input receiver email address here. System max supports 3 email boxes. System automatically filters same addresses if you input one receiver repeatedly. 
· SSL enable: System supports SSL encryption box. 

· Interval: The send interval ranges from 0 to 3600 seconds. 0 means there is no interval.

· Health email enable: Please check the box here to enable this function. This function allows the system to send out the test email to check the connection is OK or not.
· Interval: Please check the above box to enable this function and then set the corresponding interval. System can send out the email regularly as you set here. Click the Test button, you can see the corresponding dialogue box to see the email connection is OK or not.  See Figure 4‑113.

Please note system will not send out the email immediately when the alarm occurs. When the alarm, motion detection or the abnormity event activates the email, system sends out the email according to the interval you specified here. This function is very useful when there are too many emails activated by the abnormity events, which may result in heavy load for the email server.

[image: image162.png]SMTP Server |MailServer Port 25
Anonymous

User Name Password
Receiver

Sender

Title DVR ALERT [
Atachment [l

Encrypt Type [NONE 9

EventInterval (120 sec.

Health Enable

Interval 60 min.

( [e]4 J[ Cancel ) Test |





Figure 4‑112
[image: image163.png]Mail Test Error





Figure 4‑113
4.4.4.2.1.10 FTP

You need to download or buy FTP service tool (such as Ser-U FTP SERVER) to establish FTP service.

Please install Ser-U FTP SERVER first. From “start” -> “program” -> Serv-U FTP Server -> Serv-U Administator. Now you can set user password and FTP folder. Please note you need to grant write right to FTP upload user. See Figure 4‑114.

[image: image164.png]Local Server
View Window Help

L serv-
Fie Edt

User

WXBo n@ i=eeal?

50 Senl Servers
58 << LocalServer>>
° License
21 Settings
g Aty
=48 Domain
= shonghy
21 Settings

@ aciviy

€ soommn] 1 Ganera (B Ot pocess |TB o hoomss| 4 ULADL Pt

DDVR WD TRl

¥ wiite
¥ Append
¥ Delete

Ditectores:
¥ List
¥ Create

¥ Remove

rectres
% Inkeit

Add Delte Edt

i Apely

3 Restore

<< Local Server >> [System Admiistrator]

| Down: 0.000 kBps / Up: 0.000 kBps | 3 of 32767 Sockets |0 (0) Users |0fers

=lalx]





Figure 4‑114
You can use a PC or FTP login tool to test setup is right or not.

For example, you can login user ZHY to H140H140H140HTUFTP://10.10.7.7UTH   and then test it can modify or delete folder or not.  See Figure 4‑115. 

[image: image165.jpg]Internet Explorer

9 oottt o e o

FlPserver:  10.10.7.7
User name;
password:

After youlog on, you can add tis server to your Favories and return to t easiy.

[Jog on anonymousty





Figure 4‑115
System also supports upload multiple alarm panels to one FTP server. You can create multiple folders under this FTP. 

In Figure 4‑1049, select FTP and then double click mouse. You can see the following interface. See Figure 4‑116.

· Please highlight the icon [image: image166.jpg]


 in front of Enable to activate FTP function.

· Here you can input FTP server address, port and remote directory. When remote directory is null, system automatically create folders according to the IP, time and channel.

· User name and password is the account information for you to login the FTP. 

· Now you can set upload file length, picture upload interval, upload file channel, time and type. 

· File length is upload file length. When setup is larger than the actual file length, system will upload the whole file. When setup here is smaller than the actual file length, system only uploads the set length and auto ignore the left section. When interval value is 0, system uploads all corresponding files. 
· At the same time, you can set two periods, three record files for different channels. 
· Click the Test button, you can see the corresponding dialogue box to see the FTP connection is OK or not.  See Figure 4‑117.

[image: image167.png]Server IP 0 .0 .0 .0 Port (21
User Name
Password Anonymous
Remote Directory File Length [0 M
Snapshot 2 sec.
3
Channel 1 N
Weekday Tue v Alarm  Motion Regular
Time Period 1 00 :00 -24 :00
Time Period 2 00 :00 -24 :00

( [e]4 J[ Cancel ) Test |





Figure 4‑116
[image: image168.png]FTP Test Connect Failed





Figure 4‑117
4.4.4.2.1.11 Alarm Centre

This interface is reserved for you to develop. 

4.4.4.2.1.12 SNMP

SNMP is an abbreviation of Simple Network Management Protocol. It provides the basic network management frame of the network management system. The SNMP widely used in many environments. It is used in many network device, software and system.

You can set in the following interface. See Figure 4‑118.

[image: image169.png]SNMP Enable [l

] Vi
] V2
SNMP Port 161

Read-Community (public

Write-Community [private

Trap Address A

Trap Port 162





Figure 4‑118
Please enable the SNMP function. Use the corresponding software tool (MIB Builder and MG-SOFT MIB Browser. You still need two MIB file: BASE-SNMP-MIB, DVR-SNMP-MIB) to connect to the device. You can get the device corresponding configuration information after successfully connection. 

Please follow the steps listed below to configure. 

· In Figure 4‑118, check the box to enable the SNMP function. Input the IP address of the PC than is running the software in the Trap address. You can use default setup for the rest items. 

· Compile the above mentioned two MIB file via the software MIB Builder.

· Run MG-SOFT MIB Browser to load the file from the previous step to the software. 

· Input the device IP you want to manage in the MG-SOFT MIB Browser. Please set the corresponding version for your future reference. 

· Open the tree list on the MG-SOFT MIB Browser; you can get the device configuration. Here you can see the device has how many video channels, audio channels, application version and etc. 
Note

Port conflict occurs when SNMP port and Trap port are the same. 
4.4.4.2.1.13 Network Priority 

This interface is for you to set network priority. The default setup is PPPoE>WIFI>3G>LAN. See Figure 4‑119. You can change if necessary. 
Default gateway: It is to display current default gateway. It is to display PPPoE if PPPoE dial succeeded.

Priority: The value ranges from 0 to 3. 0 has the highest priority. 

Name: It is to display network name. Use [image: image170.png]


/[image: image171.png]


 to move current item up/down. 

Note:

PPPoE always has the highest priority. You can change the priority of the rest three items.

[image: image172.png]Default Gateway [LAN

Priority

Name

[

1
2
3

PPPOE
WIFI
3G
LAN

Default

) OK

Cancel





Figure 4‑119
4.4.4.2.1.14 Auto register 

This function allows the device to auto register to the proxy you specified. In this way, you can use the client-end to access the alarm panel and etc via the proxy. Here the proxy has a switch function. In the network service, device supports the server address of IPv4 or domain. 

Please follow the steps listed below to use this function. 

Please set proxy server address, port, and sub-device name at the device-end. Please enable the auto register function, the device can auto register to the proxy server. 

1) The setup interface is shown as in Figure 4‑120.
Important 

Do not input network default port such as TCP port number.

[image: image173.png]W Enable

No. 1 9
Server IP Address [0.0.0.0

Port 8000

ID 0





Figure 4‑120
2) The proxy server software developed from the SDK. Please open the software and input the global setup. Please make sure the auto connection port here is the same as the port you set in the previous step. 

3) Now you can add device. Please do not input default port number such as the TCP port in the mapping port number. The device ID here shall be the same with the ID you input in Figure 4‑120. Click Add button to complete the setup. 

4) Now you can boot up the proxy server. When you see the network status is Y, it means your registration is OK. You can view the proxy server when the device is online. 

Important 

The server IP address can also be domain. But you need to register a domain name before you run proxy device server. 

4.4.4.3 Account

Here is for you to implement account management.  See Figure 4‑121.  Here you can:

· Add new user

· Modify user

· Add group 

· Modify group

· Modify password.
For account management please note:

· For the user account name and the user group, the string max length is 6-byte. The backspace in front of or at the back of the string is invalid. There can be backspace in the middle. The string includes the valid character, letter, number, underline, subtraction sign, and dot.

· System account adopts two-level management: group and user. System max supports 64 users and 20 groups. 
· For group or user management, there are two levels: admin and user. One user should belong to one group. User right can not exceed group right.

· About reusable function: this function allows multiple users use the same account to login.
· About user account and MAC. When you add a new user, you can input the MAC address of current user. Only the user of the same MAC address can access the device remotely. (MAC address is for the device of the same LAN.) If you leave MAC address item in blank when you add a new user, the user of any MAC address can access the device remotely. You can set or change MAC address when you add or modify a user.  The MAC address function is also valid for PSS login. Please note current function does not support IPV6. 
After all the setups please click save button, system goes back to the previous menu.
[image: image174.png]4 |User Group Status User MAC
1 |888888 admin Login Local

2 |666666 user Normal

3 [admin admin Login Net

4 |default user Default User

Add User

Add Group | (Modify Group | Modify Password |





Figure 4‑121
4.4.4.3.1 Modify Password 
Click password button, the interface is shown as in Figure 4‑122.

Here you can modify account password. 

Please select the account from the dropdown list, input the old password and then input the new password twice. Click the Save button to confirm current modification. 
For the users of user account right, it can modify password of other users.  
[image: image175.png]User 666666
Old Password

El

New Password

Confirm

Save Cancel





Figure 4‑122
4.4.4.3.2 Add/Modify Group 

Click add group button, the interface is shown as below. See Figure 4‑123.

Here you can input group name and then input some memo information if necessary. 

There are total 60 rights such as control panel, shut down, real-time monitor, playback, record, record file backup, PTZ, user account, system information view, alarm input/output setup, system setup, log view, clear log, upgrade system, control device and etc. 

The modify group interface is similar to the Figure 4‑123.
[image: image176.png]Name

Memo
43 Authority e
1 Control Panel B
2 Shutdown the Device |
3 Monitor [
4 Monitor_CHO1 [
5 Monitor_CHO02
6 Monitor_CHO03
7 Monitor_CHO04
8 Playback
9 Playback_CHO1
10 Playback_CH02
1" Playback_CH03
12 Playback_CH04
13 Record =
14 Dol

14 PageUp M PageDown F Enable/Disable Authority

Save Cancel





Figure 4‑123
4.4.4.3.3 Add/Modify User 

Click add user button, the interface is shown as in Figure 4‑124.

Please input the user name, password, user MAC address and select the group it belongs to from the dropdown list. 

Then you can check the corresponding rights for current user. 
For convenient user management, usually we recommend the general user right is lower than the admin account. 
The modify user interface is similar to Figure 4‑124.
[image: image177.png]User Reuseable
Password Confirm
Memo
User MAC
Group admin N
43 Authority e
1 |~ Control Panel [l
2 |VI Shutdown the Device .
3 |vI Monitor
4 v Monitor_CHO01
5 v Monitor_CH02
6 v Monitor_CH03
7 v Monitor_CH04
8 |v| Playback
9 /| Playback_CHO1 v

14 PageUp M PageDown F Enable/Disable Authority

Save Cancel





Figure 4‑124
When you create a new user, you can input the corresponding MAC address of current user. If you leave this item in blank, any MAC address user can share this user account to login. Please note system needs to check the validity of MAC. Only the 12-digit 0-f format address can pass the validity verification. System only saves small character even you input capitalized one. You can see the corresponding prompt if there is any illegal input.

4.4.4.4 HDD Manager
Here is for you to view and implement hard disk management. See Figure 4‑125.

You can see current HDD type, status, capacity and record time. When HDD is working properly, system is shown as O. When HDD error occurred, system is shown as X.

· Alarm set: Click alarm set button, the interface is shown as below. See Figure 4‑126. (This interface is just like the abnormity setup). Please refer to chapter 5.5.2 for detailed information.

· HDD operation: You can select HDD mode from the dropdown list such as read-only or you can erase all data in the HDD. Please note system needs to reboot to get all the modification activated.
[image: image178.png]SATA 1 HDD Group [~
o HDD Setting | (HDD Channel
Alarm Set | [Alarm Release
HDD No. 1 ~| Setto [Read/Write |~| [ Execute |
Type Read/Write
Status Normal
Capacity 232.79 GB
Record Time 13-11-08 12:49:16 / 13-11-08 13:13:51 e
13-11-11 12:06:30 / 13-11-11 15:39:03
13-11-131 8/13-11-13 1 |
13-11-13 15:36:06 / 13-11-15 16:27:23 B
13-11-28 08:59:11/ 13-11-28 13:16:51
13-12-111 8/13-12-11 1
13-12-26 13:42:32/ 13-12-26 15: v





Figure 4‑125
Please highlight icon [image: image179.jpg]


 to select the corresponding function.

[image: image180.png]Event Type No Disk 9
Enable
W Alarm Out BRORE Latch

@ Show Message [ Alarm Upload [_)Send Email

WBuzzer

B Audio Broadcast [N/A [+] Output Port

(Bl Output Control

sec.

Talk Port |

Save Cancel

El





Figure 4‑126
For the HDD group setup operation, please note:

· Each channel’s records can be stored into the specified HDD Group.

· Each HDD Group is corresponding to several hard disks, while one hard disk is only included in one HDD Group.

· Each channel is only corresponding with one HDD Group, while one HDD Group can store records from several channels. 

· HDD Group is only available for read-write HDD and self-defined disks, other types of hard disks cannot be set as HDD Group.

Important:

· Current series software version can only set the HDD group operation of the read-write HDDs. It is not for the redundancy HDD or read-only HDD. 

HDD Setting

Click the button “HDD Settings” at the top right corner of the Figure 4‑125, system will pop up an interface as below. See Figure 4‑127.
· HDD: Here you can view the HDD amount the device can support. If there is a mark [image: image181.png]


 in the front of the number, it means current position has installed a HDD. 
· HDD Group: It lists the HDD Group number of current hard disk.
When you are setting the HDD Group, please check the box of the hard disk, and then choose the corresponding HDD Group number and save the settings.
Please note, one HDD is corresponding to one group, while one group can have many HDDs.  The HDD group No. is corresponding HDD port, the HDD group No. may vary if you change the HDD. 
In Figure 4‑127, you can see the system has two working hard disks at the first and second position, and the first hard disk belongs to HDD Group 1, the second hard disk belongs to HDD Group 2. 
Important
Once you change the HDD Group settings, system will reboot!
[image: image182.png]HDD No. HDD Group

1 1 <





Figure 4‑127
Channels Setting

Click the button named with “Channels Settings” at the top right corner of the Figure 4‑125, system will pop up an interface shown as in Figure 4‑128.

Channel: Here you can view the actual channel amount of the device. 

Group No.:It is the SN in the HDD group management. Here we can set HDD 1 and HDD 2 sets to as HDD group 1 and HDD group 2 respectively. You can see the HDD group option has only two options 1 and 2 when you set corresponding HDD group of the channel. 

When you set the channel setup, please select the corresponding channel such as 1 and 2, and then select the HDD group such as 1. Click the Save button, you can complete the setup. 

In the Figure 4‑128, you set channel 1 and channel 3 to the corresponding HDD group2 and set channel 2 and channel 4 to channel 16 to HDD group 1. In this setup, the records from channel 1 and channel 3 are both saved in HDD of the group 2. The records from channel 2 and channel 4 to channel 16 are all saved in HDD of the group 1. 

When you are setting the configurations of the channels setting, please select relevant channels first (such as channel 1 to 16), and then select the HDD Group NO. Please click the Save button to save current setup.  

Important
· Once you change the HDD Group settings, system will pack the records and then reboot!

Tips

There is an easy way for you to test whether the records from the corresponding channel is saved in the specified HDD. You can remove the HDD and then check the channel can record or not. You can see the channel does not record and you can not search the previous record now.  
[image: image183.png]channel HDD Group channel HDD Group

1 1 < 2 1 9
3 1 9 4 1 9





Figure 4‑128
Channel Quota

Please note only the product of the quota setup icon supports this function. 

Here you can set channel storage capacity. See Figure 4‑129.

[image: image184.png]SATA 1 Quota N
o] Quota Setup
Alarm Set | [Alarm Release
HDD No. 1 ~| Setto [Read/Write |~| [ Execute |
Type Read/Write
Status Normal
Capacity 232.79 GB
Record Time 13-11-08 12:49:16 / 13-11-08 13:13:51 e
13-11-11 12:06:30 / 13-11-11 15:39:03
13-11-131 8/13-11-13 1 7 |
13-11-13 15:36:06 / 13-11-15 16:27:23 B
13-11-28 08:59:11/ 13-11-28 13:16:51
13-12-111 8/13-12-11 1
13-12-26 13:42:32/ 13-12-26 15: v





Figure 4‑129
Click Quota Setup button, you can go to Figure 4‑130. Here you can set the each channel storage capacity in each HDD. 

[image: image185.png]Channel 1 3
HDD No. Quota Free Space
SATA1 0% 9 100%





Figure 4‑130
Click Quota Statistics, you can go to the following interface. You can view HDD capacity you set for each channel. See Figure 4‑131.
[image: image186.png]1 |Channel Quota
1 |Other Channels 232.79 GB





Figure 4‑131
4.4.4.5  RS485 
The pan/tilt/zoom setup includes the following items. See Figure 4‑132.

· Device: It includes PTZ/alarm keyboard/alarm module.
· Channel: Please select a channel from the dropdown list. 

· Protocol: Select corresponding PTZ protocol such as PELCOD.

· Address: Input corresponding PTZ address.

· Baud rate: Select baud rate.

· Data bit: Select data bit.

· Stop bit: Select stop bit.

· Parity: There are three choices: none/odd/even. 

After completed all the setups please click save button, system goes back to the previous menu.
For detailed setup, please refer to chapter 4.3.2 preset/patrol/pattern/scan.

[image: image187.png]Devices [PTZ <]
Channel |1 7
Protocol [NONE |
Address (1

Baudrate (9600 |
Data Bits |8 7
Stop Bits |1 -
Parity None ~]





Figure 4‑132
4.4.4.6 Auto Maintenance

Here you can set auto-reboot time and auto-delete old files setup. You can set to delete the files for the specified days. See Figure 4‑133.

You can select proper setup from dropdown list.

After all the setups please click save button, system goes back to the previous menu.
[image: image188.png]Auto-Reboot System

Every Tuesda|~| at |02:00 N
Auto-Delete Old Files
Customized |~/ 31 Day(s) Ago





Figure 4‑133
4.4.4.7 Backup 

The network video alarm panel support USB device backup and network download. Here we introduce USB backup. 

4.4.4.7.1 Detect Device

Click backup button, you can see an interface is shown as in Figure 4‑134. Here is for you to view devices information.  
You can view backup device name and its total space and free space. The device includes USB burner, flash disk, SD card, and portable HDD.

[image: image189.png]1 v Name(Type) Left Space/Total Space Device Status
1 || sdb1{USB DISK) 1.44 GB/1.96 GB Ready

14Page Up M Page Down - Select/Cancel backup device or file.

Detect | ( Backup ) ( Format |[ Stop |





Figure 4‑134
4.4.4.7.2 Backup

Select backup device and then set channel, file start time and end time. 

Click add button, system begins search. All matched files are listed below. System automatically calculates the capacity needed and remained. See Figure 4‑135.

system only backup files with a  √ before channel name. You can use Fn or cancel button to delete √ after file serial number.
Click backup button, you can backup selected files. There is a process bar for you reference.

When the system completes backup, you can see a dialogue box prompting successful backup. 

[image: image190.png]Selected Device [sdb1(USB DISK) 9
Type All ~| Channel |1 ~| File Format |DAV M
Start Time [@] (2011 - 04- 27 [00: 00: 00 (CRemove ) (_Add )
End Time [@] 2011 - 05- 25 [14: 20 50 (" Onekey Backup )
0 |[CH Type  Start Time End Time Size(KB)
L3
14PageUp ™ Page Down rn SelectCancel backup device or file.
Space Required / Space Remaining:0.00 KB/3.71 GB





Figure 4‑135
Click backup button, system begins burning. At the same time, the backup button becomes stop button. You can view the remaining time and process bar at the left bottom.
· File format: Click the file format; you can see there are two options: DAV/ASF.

· Picture backup: Please set the corresponding time, channel and then select the type as PIC from the dropdown list. See Figure 4‑136. Please click the Add button and then select the pictures. Click the Start button; you can copy the specified pictures to the selected portable devices. 

Figure 4‑136
· One key backup: It includes three steps: the search, select all, start the backup. You can skip the above three steps and then copy all the searched files directly. 

The file name format usually is: Channel number+Record type+Time. In the file name, the YDM format is Y+M+D+H+M+S. File extension name is .dav. 

Tips:

During backup process, you can click ESC to exit current interface for other operation. The system will not terminate backup process. 

Note:

When you click stop button during the burning process, the stop function becomes activated immediately. For example, if there are ten files, when you click stop system just backup five files, system only save the previous 5 files in the device (But you can view ten file names).
4.4.4.8 Config Backup 

The configuration file backup interface is shown as below. See Figure 4‑137.

This function allows you to import/export system configuration.  You can use this function when there are several devices need the same setup. 

· Export: Please connect the peripheral device first and then go to the following interface.  Click Export button, you can see there is a corresponding “Config_Time” folder. Double click the folder, you can view some backup files. 

· Import: Here you can import the configuration files from the peripheral device to current device.  You need to select a folder first. You can see a dialogue box asking you to select a folder if you are selecting a file. System pops up a dialogue box if there is no configuration file under current folder. After successfully import, system needs to reboot to activate new setup.

· Format: Click Format button, system pops up a dialogue box for you to confirm current operation. System begins format process after you click the OK button. 

Note:

· System can not open config backup interface again if there is backup operation in the process. 

· System refreshes device when you go to the config backup every time and set current directory as the root directory of the peripheral device. 

· If you go to the configuration backup interface first and then insert the peripheral device, please click Refresh button to see the newly added device. 
[image: image191.png]Device Name [sda1(USB DISK) |~| [ Refresh |
Total Space  |3.08 GB Free Space (248 GB
Address 123/
Name Size Type Delete
= Folder

New Folder | ([ Format | [ IMPORT | ([ EXPORT )





Figure 4‑137
4.4.4.9 Text Overlay 

The card overlay function is for financial areas. It includes Sniffer, information analysis and title overlay function. The Sniffer mode is network. 

The interface is shown as below. See Figure 4‑138.

· Sniffer mode: System supports net. 

· Protocol: Please select from the dropdown list. 

· Setting: Click ATM/POS setting button, the interface is shown as in Figure 4‑139. 
· Overlay channel: Please select the channel you want to overlay the card number. 

· Overlay mode: There are two options: preview and encode. Preview means overlay the card number in the local monitor video. Encode means overlay the card number in the record file. 

· Overlay Position: Here you can select the proper overlay position from the dropdown list. 

[image: image192.png]Sniffer Mode NET N
Protocol ATM/POS 9
Setting ATM/POS

Overlay Mode  [lPreview Encode
Overlay Position [Top Left N

Save Cancel





Figure 4‑138
Here we take the ATM/POS protocol to continue. 

There are two types: with or without the protocol according to client’s requirements.

With the protocol 

For ATM/POS with the protocol, you just need to set the source IP, destination IP (sometimes you need to input corresponding port number).

[image: image193.png]Sniffer Mode NET N
Protocol ATM/POS 9
Setting ATM/POS

Overlay Mode  [lPreview Encode
Overlay Position [Top Left N

Save Cancel





Figure 4‑139
Without the protocol 

For the ATM/POS without the protocol, the interface is shown as in Figure 4‑140.

Source IP refers to host IP address that sends out information (usually it is the device host.)

Destination IP refers to other systems that receive information. 

Usually you do not need to set source port and target port. 

There are total four groups IP. The record channel applies to one group (optional) only.

Six frame ID groups verification can guarantee information validity and legal.

[image: image194.png]Data Group Data Group1 |~/

Source IP 0 .0 .0 .0 Port [0

Destination IP 0 .0 .0 .0 Port [0

Record Channel (1](2][3](4
StartPosition Length Key

Frame ID1 |1 0 ( Data )
Frame ID2 |1 0 ( Data )
Frame ID3 |1 0 ( Data )
Frame ID4 [1{ 0 ( Data |
Frame ID5 |1 [] ( Data )
Frame ID6 |1 [] ( Data )

Save Cancel





Figure 4‑140
Click Data button you can see an interface is shown as in Figure 4‑141.

Here you can set offset value, length, title according to your communication protocol and data package. 
[image: image195.png]StartPosition Length  Title

Field1 [1 0
Field2 [1 0
Field3 [1 0
Field4 [1 0

Save Cancel





Figure 4‑141
4.4.4.10 Default 

Click default icon, system pops up a dialogue box. You can highlight [image: image196.jpg]


 to restore default factory setup. See X381H381H381HFigure 5-26.

· Select all

· General 

· Encode

· Schedule

· RS232

· Network

· Alarm

· Detect

· Pan/tilt/zoom

· Display

· Channel name
· Call centre 
Please highlight icon [image: image197.jpg]


 to select the corresponding function.

After all the setups please click save button, system goes back to the previous menu.
Warning!

System menu color, language, time display mode, video format, IP address, user account will not maintain previous setup after default operation!

[image: image198.png]Please select setting entries that you want to default.

Selectall [l

GENERAL W ENCODE [ ]
SCHEDULE [l RS232 [ ]
NETWORK [l ALARM [ ]
DETECT W PANTILT/ZOOM
DIsPLAY W CHANNEL NAME [l
ZONE CALLCENTRE W





Figure 4‑142
4.4.4.11 Shutdown
Double click shutdown button, system pops up a dialogue box for you to select. See Figure 4‑143.
· Logout menu user: log out menu. You need to input password when you login the next time.

· Shutdown: system shuts down and turns off power.

· Restart system: system begins rebooting.

· Switch user: you can use another account to log in.
[image: image199.png]Logout menu user

Logout menu user u
afidRestart application
Shutdown

Restart system
Switch user

#°Logout the OSD menu or resetrestart the system. After idle for
standby time (no inputs), the system will exit OSD automatically.





Figure 4‑143
5 WEB OPERATION 
There might be slightly difference in the interface due to different series.
5.1 Network Connection 

Before web operation, please check the following items:

· Network connection is right

· Alarm panel and PC network setup is right. Please refer to network setup(Setup ->Network)
· Use order ping ***.***.***.***(* alarm panel IP address) to check connection is OK or not. Usually the return TTL value should be less than 255.
· Open the IE and then input alarm panel IP address.

· System can automatically download latest web control and the new version can overwrite the previous one.

· If you want to un-install the web control, please run uninstall webrec3.0.bat. Or you can go to C:\Program Files\webrec to remove single folder. Please note, before you un-install, please close all web pages, otherwise the un-installation might result in error.
· Current series product supports various browsers such as Safari, firebox browser, Google browser. Device only support 1-channel monitor on the Apple PC. 
5.2 Login 

Open IE and input alarm panel address in the address column. For example, if your alarm panel IP is 10.10.3.16, then please input http:// 10.10.3.16 in IE address column. See Figure 5‑1.
[image: image200.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 5‑1
System pops up warning information to ask you whether install webrec.cab control or not. Please click yes button.
If you can’t download the ActiveX file, please modify your settings as follows. See Figure 5‑2.
[image: image201.png]Internet Options

General | Security |Privacy || Content | Connections | Programs | Advanced |

Select a zone to view or change security sektings.

@ ) v e

oot oca Tustedstes ¥
< ] 8
eret =
‘This zone is for Internet websites, =5
ks e ratios o
e

Security level for tis zone
Allowed levels For this zone: Medum ta Hih

Medium-high
- Appropriate for most websites

- Prompts before dowrloading potentialy unsafe
cantent.

~Unsigned Actve controls il not be downloaded

Custom level Default fevel

Reset all zones to defaul evel

m





Figure 5‑2
After installation, the interface is shown as below. See Figure 5‑3.
Please input your user name and password. 
Default factory name is admin and password is admin. 
Then you can select the login mode: LAN and WAN. 
Note: For security reasons, please modify your password after you first login.
[image: image202.png]



Figure 5‑3
5.3 LAN Mode 

For the LAN mode, after you logged in, you can see the main window.  See Figure 5‑4. 
[image: image203.png]WEB szve: T m Tl TR

4 CAM 1

gaona

Exzrmn
[T T
A d

DEEOEEEEEHEZ =

Copyright 2013,All Rights Reserved.





Figure 5‑4
5.4 WAN Login 

In WAN mode, after you logged in, the interface is shown as below. See Figure 5‑5.
[image: image204.png]WEB scrvee . [T ST SN ST ST

1-69Kbps-M QaGo
=acami L
oz 2013-12-30 15:98:02

Copyright 2013,All Rights Reserved.





Figure 5‑5
Appendix H Toxic or Hazardous Materials or Elements
	Component Name
	Toxic or Hazardous Materials or Elements

	
	Pb
	Hg
	Cd
	Cr VI
	PBB
	PBDE

	Sheet Metal(Case)
	○
	○
	○
	○
	○
	○

	Plastic Parts（Panel）
	○
	○
	○
	○
	○
	○

	Circuit Board
	○
	○
	○
	○
	○
	○

	Fastener
	○
	○
	○
	○
	○
	○

	Wire and Cable/Ac Adapter
	○
	○
	○
	○
	○
	○

	Packing Material
	○
	○
	○
	○
	○
	○

	Accessories
	○
	○
	○
	○
	○
	○


Note

O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.
X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. During the environmental-friendly use period (EFUP) period, the toxic or hazardous substance or elements contained in products will not leak or mutate so that the use of these (substances or elements) will not result in any severe environmental pollution, any bodily injury or damage to any assets.  The consumer is not authorized to process such kind of substances or elements, please return to the corresponding local authorities to process according to your local government statutes.
Note:

· This manual is for reference only. Slight difference may be found in the user interface.
· All the designs and software here are subject to change without prior written notice. 
· All trademarks and registered trademarks mentioned are the properties of their respective owners.
· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website or contact your local retailer for more information.

[image: image205.jpg]

















































































1                 2              3           4

















1











2




















3












































































































































Input your IP address here.








PAGE  
79

_1361772391

_1417267225

_1333259582.vsd
选项 1�

�


