[image: image55.jpg]E E sl onTrRoL”





                         Installation Guide  

ATM Standalone DVR Quick Start Guide 
Table of Contents

61
Hardware Installation and Connections


61.1
Check Unpacked DVR


61.2
HDD Installation


61.2.1
Choose HDDs


61.2.2
Calculate HDD Size


61.2.3
HDD Installation


71.3
Front Panel


101.4
Rear Panel


101.4.1
Overview


111.4.2
Connection Sample


121.5
Alarm Input and Relay Output


121.5.1
Alarm Input and Output Details


141.5.2
Relay Output Description


152
Overview of Navigation and Controls


152.1
Login, Logout & Main Menu


152.1.1
Login


152.1.2
Main Menu


162.1.3
Logout


162.1.4
Auto Resume after Power Failure


162.1.5
Replace Button Battery


162.2
Recording Operation


172.2.1
Live Viewing


172.3
Record Setup (Schedule)


172.3.1
Schedule Menu


172.3.2
Basic Operation


182.3.3
Manual record


182.3.4
Encode


202.3.5
Search and Playback


212.3.6
Basic Operation


232.4
Network Setup


242.5
PTZ Control and Color Setup


244.8.1 Cable Connection


244.8.2 PTZ Setup


264.8.3 3D Intelligent Positioning Key


273
Web Client Operation


273.1
Network Connection


273.2
Login


283.3
Main Window




Welcome
Thank you for purchasing our DVR!

This quick start guide will help you become familiar with our DVR in a very short time.

Here you can find hardware installation，cable connection information and general operations such as system setup, record, search, backup, alarm setup, also here you can find web operation instruction.
Before installation and operation please read the following safeguards and warnings carefully!

Important Safeguards and Warnings 

1．Electrical safety

All installation and operation here should conform to local electrical safety codes. 

We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation. 

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Keep upwards. Handle with care. 

Do not apply power to the DVR before completing installation. 

Do not place objects on the DVR
4．Qualified engineers needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

The DVR should be installed in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

6.  Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components listed below are included:

· One power cable

· One Ethernet cable 
· Four HDD cables

· Alarm & relay terminal blocks

· Extensional cable(for audio, loop & matrix)

· One remote control(including the battery)

· One USB mouse

· One CD(including DVR manual, client & small tools)
· Warranty card 
· A package of installation fittings 

Contact your local retailer ASAP if something is missing in your package.
Note: Any changes of this manual made to the actual product are subject to no further notification.
1 Hardware Installation and Connections

Note: All the installation and operations here should conform to your local electric safety rules.

1.1 Check Unpacked DVR

When you receive the DVR from the shipping agency, please check whether there is any visible damage to the DVR appearance. The protective materials used for the package of the DVR can protect most accidental clashes during transportation. Then you can open the box to check the accessories.  
Please check the items in accordance with the list on the warranty card. Finally you can remove the protective film of the DVR. 
1.2 HDD Installation  
1.2.1 Choose HDDs

We recommend Seagate HDD of 7200rpm or higher.

1.2.2 Calculate HDD Size

This series have no limit to HDD capacity. You can use 120G-750G HDD to guarantee higher stability.

The formula of total HDD size is:

Total Capacity (MB) = Camera Amount * Recording Hours * HDD Usage Per Hour (M/h)

H.264 compression is ideal for standalone DVRs. It can save more than 30% HDD capacity than MPEG4. When you calculate the total HD capacity, you should estimate the average HDD capacity per hour for each channel. 
For example, for a 4-ch DVR, the average capacity of HDD usage per hour per channel is 200M/h. Now if you hope the DVR can record the video 12 hours each day for 30 days, the total capacity of HDDs needed is: 4 channels * 30 days * 12 hours * 200 M/h = 288G. So you need to install one 300G HDD or two 160G HDDs.

1.2.3  HDD Installation 

Data ribbons, fastening screws and smart HDD shelf design are already provided in the accessories bag.

These series DVRs support two HDDs.
Please follow the instructions listed below to install SATA HDDs. 
[image: image1.png]


                                             [image: image2.png]



   1. Loosen the screws.                                                    2. Remove the unit cover.
[image: image3.png]


                                                            [image: image4.png]A~




 3. Dismantle the HDD bracket.                                    4. Install the first HDD. Note the HDD is placed    upside down. Please make sure bracket is in correct position.
           [image: image5.png]


                                                            [image: image6.png]



             5. Install the second HDD on the bracket.                           6. Fix the HDD bracket into the internal unit.
[image: image7.png]


                                                      [image: image8.png]



     7. Connect HDD cable to SATA port                                         8. Connect power cord to the   HDD.  
[image: image9.png]



9. Place the unit cover back   and screws firmly.
After HDD installation, please check connection of data ribbon and power cord.
1.3 Front Panel 

This section provides information about front panel and rear panel. When you install this series DVR for the first time, please refer to this part first. 

These series DVR has two types of front panels.  

One series front panel is shown as in Figure 1‑1.

[image: image10.jpg]



Figure 1‑1
1. Upper                        2.Left                              3. Esc 

4. Down                         5.Enter                           6. Right   

7. Previous                    8. Record                       9.Slow play 

10. Assistant                 11. Play/Pause              12.Fast forward 

13. Next                         14. On/off                      15.The first channel indication light  

16. The second channel indication light               17. The third channel indication light   

18 Power indications light 

The other front panel is shown as in Figure 1‑2.
[image: image11.png]



Figure 1‑2
1. Record indication light                                  2.Power indication light       

3. Remote control signal receiver                     4.Switch button 

5. Power button                                                 6. Direction buttons/1.2.3.4

7. Esc                                                                8.Enter 

9. Backward play/Pause/5                                10.Play/Pause/6

11. Slow play/7                                                 12.Forward/8

13. Previous/9                                                   14.Next/0

15. Record                                                         16.Assitant

Please refer to the following sheet for more information. 
	S/N
	Name
	Icon
	Function
	Remark

	13
	Play previous 
	I(

	When playback, click this button to 

display previous file. 

In menu operation, go to previous menu item,.


	In numeral input mode, click this button to input 1.



	11
	Slow play 
	[image: image55.jpg]►[image: image56.jpg]



	Various slow playback and normal 

playback. Speeds.


	In numeral input mode, click this button to input 2.



	10
	Play/Pause
	(II
	Play/Pause 
	In numeral input mode, click this button to input 3.



	
	
	
	In real-time monitor mode, click this 

button to go to search interface.
	

	12
	Fast forward 
	(     
	Various fast play and normal playback

Speeds.


	In numeral input mode, click this button to input 4.



	14
	Play next 
	(I
	In playback mode, play the next file. 

In menu operation, go to the next menu item.
	In numeral input mode, click this button to input 5.



	15
	Record 
	REC
	Working with direction keys to enable/disable record
	

	16
	Assistant 
	Fn[image: image57.jpg]



	In 1-window display mode, click this button to 

go to assistant menu: PTZ control and Video 

color.
	

	
	
	
	Working with direction keys to realize motion 

detection zone setup.


	

	
	
	
	Clear function: Press Fn about 1.5 seconds to clear all contents in current text box.


	

	
	
	
	In preview mode (There is no other menu available), press this button for 3 seconds to switch between TV/VGA. 

For HD1 series DVR, there are three modes: TV/VGA/VGA LCD

(60Hz LCD output)
	

	
	
	
	In text input mode, click this button 

continuously to switch between 

numeral/capitalized character and small character(expansible )
	

	
	
	
	Working with other keys to realize special functions in some menu items.
	

	7
	Cancel 
	ESC
	Cancel 
	In numeral input mode, 

Click this button to input 0.

	
	
	
	In playback mode, click this button to go back 

to real-time monitor mode. 
	

	8
	Confirm 
	Enter 
	Confirm 
	

	
	
	
	Go to the main menu 


	

	6
	Direction keys
	( (
	In real-time monitor mode, click left/right 

direction keys to switch between one-window 

and multiple-windows.
	In numeral input mode, 

Click ( to input 6.

	
	
	
	Increase/decrease numeral 
	In numeral input mode, 

Click ( to input 7.

	
	
	
	Modify setup 
	

	
	
	
	Switch PTZ control 
	

	
	
	( (
	In 1-window real-time monitor mode,

click up/down keys to switch monitor channel .
	In numeral input mode, 

Click ( to input 8.

	
	
	
	Switch PTZ control
	In numeral input mode, 

Click ( to input 8.


1.4 Rear Panel
1.4.1 Overview
Please refer to Figure 1‑3 for MB (D)/AL series DVR real panel information.
[image: image12.png]14 13 121110987654 3




Figure 1‑3
	1
	Power socket 
	2
	Power button 
	3
	The third video input channel 

	4
	The third audio input channel. (It is the fourth audio input channel for 4-ch series ATM DVR)
	5
	The second video input channel 
	6
	The second audio input channel 

	7
	The fitst video input channel 
	8
	The first audio input channel 
	9
	Video output 

	10
	Audio output 
	11
	Network port （RJ45）
	12
	USB Port 

	13
	VGA port 
	14
	RS232 port 
	15
	ALARM-RS485 port 

	16
	Fan
	
	
	
	


Please refer to Figure 1‑4 for AH/AG real panel information.
[image: image13.png]23456789 10




Figure 1‑4
	1
	Fan
	2
	The fourth video input  channel 
	3
	The third video input  channel 

	4
	The second audio input channel 
	5
	The second video inpu channel 
	6
	The first audio input  channel 

	7
	The first video input  channel
	8
	Audio output 
	9
	Video output 

	10
	VGA port 
	11
	RS232 port 
	12
	USB port 

	13
	Networkport（RJ45）
	14
	ALARM-RS485 port 
	15
	Power button 

	16
	Power socket 
	
	
	
	


1.4.2 Connection Sample
Here is a connection sample of DVR0404MB/0404MBD for your reference. See Figure 1‑5.

[image: image14.jpg]b Power Input
| - Fan

Alarm Input

L |

Power Switch

Video Output

Audio Output
— o
Alarm Output 1
Audio Input

= ‘ Video Iny ‘
= put

B RS485 - :‘ e\ /i” ’

PTZ Control

VGA Output

RS232
USB Port

Network Switch

Online User Online User Online User Network Keyboard Online User




Figure 1‑5
1.5 Alarm Input and Relay Output
These series DVRs have two alarm types. Normal open (NO) and normal close (NC).

1.5.1 Alarm Input and Output Details
Please refer to Figure 1‑6 for AG/AH series alarm input and output information. The alarm gets activated when the circuit is connected to the ground. 

[image: image15.jpg]



Figure 1‑6
· ALARMS1 in 1 to ALARM1 in 4 are corresponding to ALARM1 to ALARM4 respectively. The alarm gets activated in low voltage.
· In ALARM1, 1-NO is normal open activation output. In ALARM2, 2-NO C NC is a group of normal open/normal close activation output.
· +12V is external alarm input.  System needs external device to provide +12V power and you can connect it to the alarm device power below 1A.
· A/B is A cable and B cable to control 485 device. You can connect them to devices such as decoder. Please parallel connect a 120Ω resistance if there are too much PTZ decoders. 
· “[image: image16.png]


”：Ground cable.
You can refer to the following sheet and Figure 1‑7 for MB(D)/AL series alarm input and output information.
[image: image58.png]Security Setting:

nternet Zone

Settings

O Disable
© Enable

9] Download signed ActiveX controls (not secure)
O Disable
© Enable (not secure)
O Prompt (recommended)

9] Download unsigned Active controls (not secure)
© Disable (recommended)
© Enable (not secure)
O Prompt

19 Intiaize and scrpt ActiveX contros ok merked as sae for s
O Disable (recommended)
© Enable (not secure)
O Prompt

4] Run ctiveX contros and pug-ins
3 bbb e

< I

*Takes effect after you restart Internet Explorer

Reset custom settings





[image: image59.jpg]View 1 »
View 4 »

Pan/TiltZoom

Color Setting

Search
Record
Alarm Output
Alarm Input

Main Menu




[image: image60.png]



[image: image61.png]ST





Figure 1‑7
	Parameter


	Grounding Alarm  

	Ground
	Ground line

	Alarm in 1…4
	1, 2, …, 4. Become activated in low voltage, 

	Relay Output
	1,2,3,4: NO and C(Normally Open and Com)

5: NO,C and NC(Normally Open, Com, Normally Closed)

6: Ctrl 12V(This is used for reset the senor)

	485 A、B
	485 communication port. They are used to control devices such as PTZ.

	+12（C）
	This should input an external power input(Less than 1 A).


· 4-ch grounding alarm inputs. (Normal open or Normal close type)
· Please parallel connect COM end and GND end of the alarm detector (Provide external power to the alarm detector).

· Please parallel connect the Ground of the DVR and the ground of the alarm detector.

· Please connect the NC port of the alarm sensor to the DVR alarm input(ALARM)

· If you need to reset the touched-off alarm remotely, you can use DVR to supply controllable 12 V power to the alarm detector such as the smoke detector.

· Use the same ground with that of DVR if you use external power to the alarm device.
[image: image17.jpg]Alarm input public end should jump out with device power end
Alarm device connection terminal

12V _|SNo [CoM leg Alarm device
@ =) connection terminal
‘ | © ALARM

@ GND

12y GND





Figure 1‑8
1.5.2 Relay Output Description
· 2 ways relay alarm output. Provide external power to external alarm device.

· To avoid over loading, please read the following relay parameters sheet carefully. (See below table)

· The controllable +12v can be used to restore the smoke detector.
2 Overview of Navigation and Controls 

Before operation, please make sure you have properly installed HDDs and all the cable connections. 

2.1  Login, Logout & Main Menu 

2.1.1 Login 

When the system boots up, default video display is in multiple-window mode.

Click Enter or left click mouse, you can see the login interface. See Figure 2‑1. 

System consists of four accounts: 
· Username: admin.  Password: admin. (administrator, local and network)
· Username: 888888. Password: 888888. (administrator, local only)
· Username: 666666. Passwords: 666666(Lower authority user who can only monitor, playback, backup and etc.)

· Username: default. Password: default(hidden user)

For your system security, please modify you password after first login. 

You can use USB mouse, front panel, remote controller or keyboard to input. About input method: Click [image: image18.png]


 to switch between numeral, English character (small/capitalized) and denotation.
Note: Three times login failure within 30 minutes will result in system alarm and five times login failure will result in account lock!
[image: image19.jpg]SYSTEM LOGIN

User Name (888888 5

Password 123}

3
6
9 ncel
=





Figure 2‑1
2.1.2  Main Menu 

After you logged in, the system main menu is shown as below. See Figure 2‑2.  
There are total six icons: search, information, setting, backup, advanced and shutdown. 
You can move the cursor to highlight the icon, and then left click mouse to enter the sub-menu.
[image: image20.emf]
Figure 2‑2
2.1.3  Logout 
There are two ways for you to log out.
One is from menu option:
In the main menu, click shutdown button, you can see an interface is shown as below.  See Figure 2‑3.

[image: image21.jpg]UTDOWN

‘Logou( menu user
Password is needed to re-enter the menu
| after logout.

OK ] | Cancel |





Figure 2‑3
There are several options for you. See Figure 2‑4.

[image: image22.png]Logout menu user
Restart application
Shutdown

Restart system
Switch user





Figure 2‑4
The other ways is to press power button on the front panel for at least 3 seconds, system will stop all operations. Then you can click the power button in the rear panel to turn off the DVR.

2.1.4  Auto Resume after Power Failure 
The system can automatically backup video and resume previous working status after power failure.

2.1.5 Replace Button Battery 

Please make sure to use the same battery model if possible. 
We recommend replace battery regularly (such as one-year) to guarantee system time accuracy.

2.2 Recording Operation 

2.2.1  Live Viewing

When you login, the system is in live viewing mode. You can see system date, time and channel name. If you want to change system date and time, you can refer to general settings (Main Menu->Setting->General). If you want to modify the channel name, please refer to the display settings (Main Menu->Setting->Display)
The following icons stand for camera’s video and alarm status.

	1
	[image: image23.png]8]




	Recording status
	3
	[image: image24.png]



	Video loss 

	2
	[image: image25.png]



	Motion detection 
	4
	[image: image26.png]



	Camera lock


Note: Please refer to the following sheet for channel status. [image: image27.png]


 stands for opening switch function,  [image: image28.jpg]


 stands for closing switch function.

2.3 Record Setup (Schedule)

When the system boots up, it is in default 24-hour regular mode. You can set record type and time in schedule interface.

2.3.1 Schedule Menu
In the main menu, from setting to schedule, you can go to schedule menu. See Figure 2‑5.

There are three record types: R-Regular, MD-Motion detection, A- Alarm.  In some series, system also supports C-Card) 

[image: image29.png]Channel [1__[+]PreRecord[4 |sec.RedundancyllSnapshotll

Week Day ED Record Type  Regular MD  Alarm
Period 1 -08 :00 O @ 0
Period2 08 :00  -10 :00 8 0O O
Period3 (10 :00 O 0O =
Period4 (12 :00 0O @ O
Period5  [20 :00 O 0O =
Period 6 (00 :00 0O 0O O
1" Regular W MD || Alarm

o 3 6 s 1SS 21 24
I I

( Copy )( Pase )( Defaut ) ( Save [ Cancel )





Figure 2‑5
2.3.2 Basic Operation
There are totally six periods. See Figure 2‑5.
· Channel: Please select the channel number first. You can select “all” if you want to set for the whole channels.

· Week day: There are eight options: ranges from Saturday to Sunday and all.

· Redundancy: System supports redundancy backup function.  You can highlight Redundancy button to activate this function. Please note, before enable this function, please set at least one HDD as redundant.(Main menu->Advanced->HDD Management) 
· Snapshoot: You can enable this function to snapshoot image when alarm occurs.

· Record types: There are three types: regular, motion detection (MD) and Alarm.
Please highlight icon [image: image30.jpg]


 to select the corresponding function. After all the setups please click save button, system goes back to the previous menu. 

At the bottom of the menu, there is a color bar for your reference. Green stands for regular recording, yellow stands for motion detection and red stands for alarm recording.

2.3.3  Manual record

Note: 

You need to have proper rights to implement the following operations. Please make sure the HDDs have been properly installed.

2.3.3.1 Manual record menu

There are two ways for you to go to manual record menu.

· Right click mouse or in the main menu, Advanced->Manual Record. 

· In live viewing mode, click record button in the front panel or record button in the remote control.

There are three statuses: schedule/manual/stop. Highlight icon“○” to select corresponding channel. See Figure 2‑6.
· Manual: the highest priority. After manual setup, all selected channels will begin ordinary recording.

· Schedule: channel records as you have set in recording setup (Main Menu->Setting->Schedule) 
· Stop: all channels stop recording.
[image: image31.jpg]MANUAL RECORD

i! Record Mode Al %2
|| Schedule LR

OK Cancel





Figure 2‑6
2.3.4 Encode 
Encode setting includes the following items. See Figure 2‑7.

Please note some series do not support extra stream. 
· Channel: Select the channel you want.

· Compression: system supports H.264. Or you can select from the dropdown list.
· Resolution: System supports various resolutions, you can select from the dropdown list. For this model, we can support D1/HD1/CIF.
· Bit rate: system supports two types: CBR and VBR. In VBR mode, you can set video quality.
· Quality: There are six levels ranging from 1 to 6. The sixth level has the highest image quality.

· Frame rate: there are six levels: 1 f/s,2f/s,3f/s, 6f/s,12f/s,25f/s. (Some series DVRs only support PAL 25f/s )
· Video/audio: you can enable or disable the video/audio respectively for the main stream and extra stream.
· Overlay: click overlay button, you can see an interface is shown in Figure 2‑8.
· Cover area (Privacy mask): Here is for you to set window blanking section. You can drag you mouse to set proper section size.
· Preview/monitor: privacy mask has two types. Preview means the privacy mask zone can not be viewed by user when system is in preview status. Monitor means the privacy mask zone can not be view by the user when system is in monitor status.
· Time display: You can select system displays time or not when you playback.

· Channel display: You can select system displays channel number or not when you playback.
· Snapshoot: Click snapshoot button, you can see an interface is shown as in Figure 2‑9.
· Mode: There are two types: one is timing and the other is activation (trigger). 

· Image size: D1/HD1/BCIF/CIF.

· Image quality: level1 to level 6.

· Snapshoot frequency: Here you can set the snapshoot frequency. The value ranges from 1s/p to 7s/p.

System default setup is:

· Channel:1

· Compression:H.264

· Resolution: CIF/D1

· Bit rate: CBR

· Quality: 4
· Frame rate: 25f/s
Please highlight icon [image: image32.jpg]


 to select the corresponding function.

This series DVR is suitable for ATM use. System can overlay card number, transaction time and etc on the monitor or playback window (Need protocol support). System also supports search by transaction information and card number.

[image: image33.png]Channel 1 N

Compression H.264 ~| |Extra Stream1 |~

Resolution CIF ~| |QCIF N

Frame Rate(FPS) (25 M N

Bit Rate Type CBR ~| |[CBR N

Bit Rate(Kb/S) Customi; v| 620 |384 N

Reference Bit Rate 480-760Kb/S 256-512Kb/S

Audio/Video O m |0

( Copy )( Pase )( Defaut ) ( Save [ Cancel )





Figure 2‑7
[image: image34.png]Cover-Area () Preview [_] Monitor Set

Time Display B Monitor Set
Channel Display B Monitor Set





Figure 2‑8
[image: image35.png]{ Mode

Timing 9

Image Size D1 9

{ Image Quality 4 9
Snapshot Frequency [1 SP2L “





Figure 2‑9
2.3.5 Search and Playback 
There are two ways for you to go to search menu.

· Click Pause/Play button in the remote control.

· Click search in the main menu.

Search interface is shown as below. See Figure 2‑10. 
Usually there are three file types:

· R: regular recording file.

· A: external alarm recording file.

· M: motion detection recording file 
· C: card and POS test overlay recording file(For some special series only)
There are several playback windows. System supports 1/2-ch playback.

[image: image36.png]2008-06-11 14:03:08

ME 2000226150356

UMBER
PE

=

ml= =
=}
=

RROR

=
[©72007-06-2600: 00:00  [1=|Card B

CHD 0 - = rerommamm— 0(e )





Please refer to the following sheet for more information.
Figure 2‑10
Please refer to the following sheet for more information.

	Serial Number 
	Function

	1
	Play

	2
	Backward

	3
	Stop

	4
	Slow play

	5
	Fast play

	6
	Previous frame

	7
	Next frame

	8
	Volume

	9
	Previous file

	10
	Next channel 

	11
	Next file

	12
	Previous channel

	13
	Search

	14
	Backup


2.3.6 Basic Operation 

2.3.6.1 Playback 

There are various search modes: video type, channel number or time. The system can max display 128 files in one screen. You can use page up/down button to view if there are more than one page.

Select the file name and double click mouse (or click enter button), you can view file content.

2.3.6.2 Accurate playback

Input time (h/m/s) in the time column and then click playback button, system can operate accurate playback.

2.3.6.3 Synchronized playback function when playback

During playback process, click numeral key, system can switch to the corresponding channel video of the same time.
2.3.6.4 Digital zoom
When the system is in full-screen playback mode, drag your mouse in the screen to select a section and then left click mouse to realize digital zoom. You can right click mouse to exit. 

2.3.6.5 File backup

System supports backup operation during search. You can draw a √ before file name (multiple choices). Then click backup button (Button 14 in Figure 2‑9).
2.3.6.6 Slow playback and fast playback 

Please refer to the following sheet for slow play and fast playback function.

	Button 
	Illustration
	Remarks

	Fast play button (

	In playback mode, click this button to switch between various fast play modes such as fast play 1,fast play 2 and more.(Fast play 1 means fast play level 1 or not about speed)

	Frame rate may vary due to different versions.

	Slow play button  ► (Or you can turn the outer ring counter clockwise.)


	In playback mode, click this button to switch between various slow play modes such as slow play 1 or slow play 2.


	

	3、Play/Pause►          
	In slow playback mode, click this button to switch between play/pause modes.
	

	4、Previous/next 
	In playback mode, you can click  ( and (　to view previous or next video in current channel.
	


2.3.6.7 Fast forward/fast backward and frame by frame playback

	Special Functions of Shuttle and Jog
	Illustration 
	Remarks

	Fast forward(outer ring clockwise)
	When playback, turn the shuttle (outer ring) clockwise one round: you can view in fast level 1

Turn it two rounds you get fast level 2.  You can continue turning to get different speed.


	In forward or backward mode, double click Pause/Play button to get normal playback.

Frame rate may vary due to different version.

	Fast backward(outer ring counter clockwise)
	When playback, turn the shuttle (outer ring) counter clock-wise one round, you can view in backward level 1. Turn it two rounds, you get backward level 2. You can continue turning to get different speed.


	

	Manual playback frame by frame
	In playback mode, click play/pause button, slowly turn the jog (inner dial) clock-wise to view frame by frame, counter clock wise to view I frame playback.
	


2.3.6.8 Backward playback and frame by frame playback

	Button 
	Illustration
	Remarks

	Backward play

  ( in playback interface.
	In normal playback mode, left click backward play button, system begins backward playback.

Double click backward play button again, system goes to pause mode.


	When system is in backward play or frame by frame playback mode, you can click play button to go to normal playback.

	Manual playback frame by frame.
	Click pause button in normal playback mode, slowly turn the jog (inner dial) clock-wise to view frame by frame, counter clock wise to view I frame playback.
	


Note:

All the operations here (such as playback speed, channel, time and progress) have relationship with hardware version. Some series DVRs do not support some functions or playback speeds. 
2.4 Network Setup 

Here is for you to input network information. See Figure 2‑11. 
· IP address: Here you can input IP address.

· DHCP: It is auto search IP function. When enable DHCP function, you can not modify IP/Subnet mask /Gateway. These values are from DHCP function. If you have not enabled DHCP function, IP/Subnet mask/Gateway display as o. You need to disable DHCP function to view current IP information.  Besides, when PPPoE is operating, you can not modify IP/Subnet mask /Gateway. 
· TCP port: Default value is 37777. (System server port 37778 is reserved for network UDP use.)

· UDP port: Default value is 37778.
· HTTP port: Default value is 80.
· Max connection: system support maximal 10 users. 0 means there is no connection limit.

· Transfer mode: Here you can select the priority between fluency/video qualities.

· Network download: System can process the downloaded data first if you enable this function. 

After completing all the setups please click save button, system goes back to the previous menu.
[image: image37.png]IP Address 192 . 168 . 1 . 108 [J pHeP

Subnet Mask [255 . 255 . 255 . 0
Gateway 192 . 168 . 1 . 1

TCP Port 1 HTTP Port 1

UDP Port 0 Max Connection [0

()] Transfer Mode  [Fluency '

O Network Download

ADVANCED SETTING.
IP FILTER Trusted Sites:0 ]
NTP 0.0.0.0: 10 I
MULTICAST 239.255.42.42 B

| PPPOE username





Figure 2‑11 
2.4.1.1 Advanced Setup
Advanced setup interface is shown as in Figure 2‑12. Please draw a circle to enable corresponding function and then double click current item to go to setup interface. 
[image: image38.png]ADVANCED SETTING.

IP FILTER
NTP
MULTICAST
v PPPOE
DDNS

Trusted Sites:0
0.0.0.0: 10
239.255.42.42
username





Figure 2‑12
2.5 PTZ Control and Color Setup

Note: All the operation here is based on PELCOD protocol. , For other protocols, there might be a little difference.  
4.8.1 Cable Connection
 Please follow the procedures below to go on cable connection

· Connect the dome RS485 port to DVR 485 port.

· Connect dome video output cable to DVR video input port.

· Connect power adapter to the dome.
4.8.2 PTZ Setup
Note: The camera video should be in the current screen. Before setup, please check the following connections are right:

· PTZ and decoder connection is right. Decoder address setup is right.

· Decoder A (B) line connects with DVR A (B) line.

Boot up the DVR, input user name and password.

In the main menu, click setting, and then click Pan/Tilt Control button. The interface is shown as in Figure 2‑13. Here you can set the following items:

· Channel: select the current camera channel.
· Protocol: select corresponding PTZ protocol(such as PELCOD)
· Address: default address is 1.
· Baud rate: select corresponding baud rate. Default value is 9600.
· Data bits: select corresponding data bits. Default value is 8.
· Stop bits: select corresponding stop bits. Default value is 1.
· Parity: there are three options: odd/even/none. Default setup is none.
[image: image39.png]Channel |1 7
Protocol |PELCOD 7
Address (1

Baudrate (115200 7
Data Bits (8 7
Stop Bits [1 9
Parity None -

( Copy ) Pase |( Defaut ) ( Save [ Cancel |





Figure 2‑13
After all the setting please click save button. 
In one window display mode, right click mouse (click “Fn” Button in the front panel or click “Fn” key in the remote control). The interface is shown as in Figure 2‑14.

Figure 2‑14
Click Pan/Tilt/Zoom, the interface is shown as below. See Figure 2‑15.

Here you can set the following items:
· Speed: value ranges fro 1 to 8.

· Zoom 

· Focus 

· Iris

Click icon [image: image40.bmp] and [image: image41.bmp] to adjust zoom, focus and iris.

[image: image42.png]Speed 5
® zoom @

a
P . » ® Focus @
(] Iris ®
[N 4
v

et

II

Page Switch EiAliace





Figure 2‑15
In Figure 2‑15, please click direction arrows (See Figure 2‑16) to adjust PTZ position. There are total 8 direction arrows. 

Figure 2‑16
4.8.3 3D Intelligent Positioning Key

In the middle of the eight direction arrows, there is a 3D intelligent positioning key. See Figure 2‑17.

Click this key, system goes back to the single screen mode. Drag the mouse in the screen to adjust section size. It can realize PTZ automatically.

Figure 2‑17
Here is a sheet for you reference.

	Name 
	Function

key 
	function
	Shortcut key 
	Function

 key
	function
	Shortcut

 Key

	Zoom
	[image: image43.bmp]
	Near 
	►
	[image: image44.bmp]
	Far 
	(

	Focus
	[image: image45.bmp]
	Near 
	(
	[image: image46.bmp]
	Far 
	►

	Iris
	[image: image47.bmp]
	close 
	((
	[image: image48.bmp]
	Open 
	((


3 Web Client Operation
There might be slightly difference in the interface due to different series.
3.1 Network Connection 

Before web client operation, please check the following items:

· Network connection is right

· DVR and PC network setup is right. Please refer to network setup(main menu->setting->network)
· Use order ping ***.***.***.***(* DVR IP address) to check connection is OK or not. Usually the return TTL value should be less than 255.
· System is compatible with WIN VISTA web control right now. But you need to disable user account control function. Double click user account and then disable user account control. After completing setup, please reboot the PC. 

· System can automatically download latest web control and the new version can overwrite the previous one.

· If you want to un-install the web control, please run uninstall web.bat to un-install. Please note, before you un-install, please close all web pages, otherwise the un-installation might result in error.

3.2 Login 
Open IE and input DVR address in the address column. For example, if your DVR IP is 10.10.3.16, then please input http:// 10.10.3.16 in IE address column. See Figure 3‑1
[image: image49.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 3‑1
System pops up warning information to ask you whether install webrec.cab control or not. Please click yes button.
If you can’t download the ActiveX file, please modify your settings as follows. See Figure 3‑2.
[image: image50.png]Internet Options

General | Security |Privacy || Content | Connections | Programs | Advanced |

Select a zone to view or change security sektings.

@ ) v e

oot oca Tustedstes ¥
< ] 8
eret =
‘This zone is for Internet websites, =5
ks e ratios o
e

Security level for tis zone
Allowed levels For this zone: Medum ta Hih

Medium-high
- Appropriate for most websites

- Prompts before dowrloading potentialy unsafe
cantent.

~Unsigned Actve controls il not be downloaded

Custom level Default fevel

Reset all zones to defaul evel

m





Figure 3‑2
After installation, the interface is shown as below. See Figure 3‑3.
Please input your user name and password. 
Default factory name is admin and password is admin. 

Note: For security reasons, please modify your password after you first login.
[image: image51.png]WEBservice - Windows Internet Explorer (x]

OO - [ %] (x] [ g
& & @i = i@ @ - [ page - 3 Took - @+

bone T [ | | (g @ e | wnon -





Figure 3‑3
3.3 Main Window 

After you logged in, you can see the main window is shown as in Figure 3‑6. 
This main window can be divided into the following sections. 

· Section 1: there are five function buttons: configuration, search, alarm, about, log out. 

· Section 2: there are channel number and three function buttons: refresh, start dialog and local play.

· Section3: there are PTZ, color button and you can also select picture path and record path. 

· Section 4:real-time monitor window. Please note current preview window is circled by a green rectangle zone.
· Section 5: Here you can view window switch button.  You can also select video priority between fluency or real-time.

· System monitor window switch supports full screen/1-window/4-window/6-window/8-window/9-window/13-window/16-window/20-window/25-window/36-window. See Figure 3‑4.

[image: image52.png]BEOEPEEEEEE@®E




Figure 3‑4
· Preview window switch. System support 1/4/8/9/16-window real-time preview. Please you need to have the proper rights to implement preview operation. You can not preview if you have no right to preview the either channel. See Figure 3‑5.

[image: image53.png](=05::00-- 08 -0 |





Figure 3‑5

[image: image54.jpg]WEB service— | siawci ALaRm conrs AsouT wosout

S Saeaex
=

DEODEEEDEEDE






Figure 3‑6
Note
For security reason, only the user of high-level can shut down the DVR.

Do remember logging out the system in case there is vicious operation under your name!

For detailed operation introduction, please refer to our resource CD included in your package for electronic version of the user’s manual.
Slight difference may be found in user interface.

All the designs and software here are subject to change without prior written notice.

Please visit our website for more information.





























































































































































































































































































1   2 4 5  6  7     8                                         9 10 11





File List





3





1-window Playback


2-window Playback








Playback Window





File Information





14




















12





13
































Section 5





Section 2





Section 4





Section 3





Section1 





Input your IP address here.





     Terminal 





    Screw








[image: image56.jpg]
PAGE  
                                                                                                                                      27     


