[image: image247.jpg]E E sl onTrRoL”





                         Installation Guide  

Forensic DVR User’s Manual
[image: image257.png]



Table of Contents

91
Features and Specifications


91.1
Features


91.2
Specifications


121.3
Special Features


142
Overview and Controls


142.1
Front Panel


142.2
Rear Panel


152.3
Remote Control


162.4
Mouse Control


172.5
Virtual Keyboard


183
Installation and Connections


183.1
Check Unpacked DVR


183.2
HDD Installation


183.2.1
Choose HDDs


183.2.2
Calculate HDD Size


183.2.3
HDD Installation


193.3
CD/DVD Burner Installation


193.4
Desktop and Rack Mounting


193.4.1
Desktop Mounting


193.4.2
Rack Mounting


193.5
Connecting Power Supply


193.6
Connecting Video Input and Output Devices


193.6.1
Connecting Video Input


203.6.2
Connecting Video Output


203.7
Connecting Audio Input & Output, Bidirectional Audio, Looping Video


203.7.1
Audio Input/One Audio Output


213.7.2
Alarm Input and Relay Output


213.7.3
Alarm Input


223.7.4
Alarm Output


223.7.5
Alarm Input and Output Details


243.7.6
Relay Output Description


253.8
RS232


263.9
RS485


263.10
Other Interfaces


274
Overview of Navigation and Controls


274.1
Login, Logout & Main Menu


274.1.1
Login


274.1.2
Main Menu


284.1.3
Logout


284.1.4
Auto Resume after Power Failure


284.1.5
Replace button battery


284.2
Recording operation


284.2.1
Live Viewing


284.2.2
Manual Record


304.3
Search & Playback


304.3.1
Search Menu


324.3.2
Basic Operation


344.3.3
Backup in search interface


344.3.4
Calendar


344.4
Record setup (Schedule)


354.4.2 Basic Operation


364.5
Motion Detect


364.5.1 Go to Detect Menu


364.5.2 Motion Detect


394.5.3 Video Loss


394.5.4 Camera Masking


404.6
Alarm Setup and Alarm Activation


414.6.1
Go to alarm setup interface


414.6.2
Alarm setup


434.7
Real-time Backup and Burn


434.7.1
Real-time Backup


454.7.2
Backup


484.8
CD playback


494.9
PTZ Control and Color Setup


494.9.1
Cable Connection


504.10
PTZ Setup


514.11
Temperature/Humidity Overlay


514.12
Color Setting


535
Understanding of Menu Operations and Controls


535.1
Menu Tree


535.2
Main Menu


545.3
Setting


545.3.1
General


565.3.2
Encode


575.3.3
Schedule


575.3.4
RS232


585.3.5
Network


655.3.6
Alarm


655.3.7
Detect


655.3.8
Pan/Tilt/Zoom


665.3.9
Display


675.3.10
Default


685.4
Search


685.5
Advanced


695.5.1
HDD Management


705.5.2
Abnormity


705.5.3
Alarm Output


715.5.4
Manual Record


715.5.5
Account


725.5.6
Auto Maintain


725.5.7
TV Adjust


725.6
Information


735.6.1
HDD Information


735.6.2
BPS


745.6.3
Log


745.6.4
Version


755.6.5
Online Users


755.7
Backup


755.8
Exit


776
About Auxiliary Menu


776.1
Go to Pan/Tilt/Zoom Menu


776.1.1
3D Intelligent Positioning Key


786.2
Preset  /Patrol / Pattern /Border  Function


796.2.1
Preset Setup


796.2.2
Activate Preset


796.2.3
Patrol Setup


796.2.4
Activate Patrol


796.2.5
Pattern Setup


806.2.6
Activate Pattern Function


806.2.7
Border Setup


806.2.8
Activate Border Function


806.2.9
Flip


827
WEB CLINET OPERATION


827.1
Network Connection


827.2
Login and logout


857.2.1
Real-time Monitor


867.2.2
Burning


877.2.3
PTZ


907.2.4
Color


907.2.5
Picture Path and Record Path


917.2.6
Menu Interface Switch


927.3
Configure


927.3.1
System Information


947.3.2
Setting


1137.4
Search


1147.4.1
Download


1157.5
Alarm


1157.6
About


1167.7
Log out


1167.8
Un-install Web Control


1178
Enterprise Professional Surveillance System


1178.1
Log in


1188.2
Enable Monitor


1198.3
Add New Device


1219
Player Operation


1219.1
General Introduction


1229.2
Play File


1229.3
Right Mouse Menu


1239.3.1
Convert to AVI


1249.3.2
Parameter Setting


12610
FAQ


130Appendix A
HDD Capacity Calculation


131Appendix B
Compatible USB Drive List


132Appendix C
Compatible CD/DVD Burner List


133Appendix D       Compatible CD/DVD CD List


134Appendix E       Compatible SATA HDD List




Welcome

Thank you for purchasing our DVR!

This operating manual is designed to be a reference tool for the installation and operation of your system. 

Here you can find information about this series DVR features and functions, as well as a detailed menu tree.

Before installation and operation please read the following safeguards and warnings carefully!

Important Safeguard and Warnings 

1．Electrical Safety

All installation and operation here should conform to local electrical safety codes. 

We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation. 

2．Transportation Security

No heavy stress, violent vibration or water splash are allowed during transportation, storage and installation.

3．Installation

Keep upwards. Handle with care. 

Do not apply power to the DVR before completing installation. 

Do not drop objects on the DVR
4．Qualified Engineers Needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

The DVR should be installed in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

6.  Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components listed below are included:

· One power cable

· One Ethernet cable

· A package of installation fittings 

· DVR user’s manual

· One CD

· Product warranty card 
· One mouse
Contact your local retailer ASAP if something is missing in your package.
1  Features and Specifications
1.1 Features

This series DVR has the following features:
· H.264 compression algorithm ideal for standalone DVR

· Real-time live display up to 4 cameras, 100/120 fps recording for 4CIF

· Pentaplex function: live, recording, playback, backup & remote access

· 2 HDDs supported & CD-RW/DVD-RW supported
· Video files are stored in the HDD and backed up by the burner simultaneously
· Picture in picture live viewing as well as playback

· CD-RW/DVD-RW automatically switch and CD/DVD change promotion

· Forensic information input &display during playback and data encryption for police use
· Multiple control methods: IR remote control, keyboard, USB mouse and network keyboard.

· Smart video detection: motion detection, camera masking, video loss.

· Smart camera settings: privacy masking, camera lock, color setting, and title display

· Pan Tilt Zoom and Speed Dome Control: more than 60 protocols supported, preset, scan, auto pan, auto tour, pattern, auxiliary function supported. And with Speed Dome, 3D intelligent positioning function supported.

· 2-channel audio input and bidirectional talk supported

· Easy backup methods: USB devices, CD-RW/DVD-RW & network download

· Alarm triggering screen tips, buzzer, PTZ preset, e-mail, FTP upload.

· Smart HDDs Management: non-working HDD hibernation, HDD faulty alarm, Raid function.

· Powerful network software: built-in web server, EPSS (Enterprise professional surveillance software). Networking access for remote live viewing, recording, playback, setting, system status, event log, e-mail & ftp function.
1.2 Specifications

System

Main Processor 
          High performance embedded microprocessor

Operating System 
          Embedded LINUX

System Resources            Pentaplex function: live, recording, playback, backup & 
remote access

User Interface
          GUI, on-screen menu tips. 

Control Device                   USB mouse, keyboard, IR remote 
control, network keyboard,.

Input Method
                     Numeral/Character/Denotation

System Status                   HDD status, data stream statistics, log record, BIOS 
version, on-line user and etc.
Peripheral device              LTM8230C overlay temperature and humidity to current video
Video

Video Input                      4-Channel composite video(NTSC/PAL) BNC（1.0VP-P ，  75Ω）
Video Output                    1-ch PAL/NTSC，BNC（1.0VP-P ，  75Ω） composite signal

1-ch VGA output
Support switch between TV and VGA.
Video Standards                PAL（625Line，50f/s），NTSC（525Line，60f/s）

Video Compression           H.264

Video Resolution                Format

NTSC


PAL

                                           D1(4CIF)                  704*480               704 * 576
                                           2CIF                         704*240               704*240
                                           CIF                            352 *240             352 *288
                                           QCIF                          176*120             176*144

Video Recording                PAL 1f/s~25f/s  NTSC 1f/s~30f/s
Video Display Split             Full and multiple screen display, 1 / 4 PIP
Tour Display                      Support 

Image Quality                    1~6 level (level 6 is the best)

Privacy Masking                Eight self-defined four-sided zones for privacy masking for each camera 
Camera Lock                     Camera locked for users

Camera Adjustment           Adjust color according to different time period

Video Information 
Camera title, time, video loss, camera lock, motion detection, recording

TV Output Adjustment        Adjust TV output color & display zone 

Audio

Audio Input 
 2-channel input 200-1000mv 1KΩ(BNC)
                                             The first channel shall short circuit with the pickup.

                                             Active pickup (active Vpp=2V)

MIC audio input (Line in)
Audio Output 
             1-channel output, 200-2000mv 4Ω(BNC)
                                              Local speaker output/earphone output 
Audio Compression              ADPCM

Video Detection & Alarm 

Motion Detection 
   PAL: 22X18/NTSC: 22X15 detection zones
                                              Sensitivity: 1~6 (level 6 is highest)

                                              Trigger recording, PTZ movement, tour, alarm, e-mail & FTP

Video Loss 
Trigger recording, PTZ movement, tour, alarm, e-mail & FTP

Camera masking                    Trigger recording, PTZ movement, tour, alarm, e-mail & FTP

Alarm Input                            4-channel, programmable, ground, manual open/closed

                                              Trigger recording, PTZ movement, tour, alarm, e-mail & FTP

Relay output                          3-channel including one controllable +12V (do not need to connect external +12V), 30VDC, 1A, 125VAC, 0.5A (activation output)
Hard Disk

Hard Disk            
2 SATA ports   2 HDDs supported. 
Space Occupation                 Audio ：ADPCM28.8MB/H   Video ：56～900MB/H

HDD Management 
Hard disk hibernation technology, HDD faulty alarm & Raid (Redundancy)

Record, Playback & Backup

Recording Mode      
Manual, continuous, video detection (including motion detection, camera masking, video loss), Alarm

Recording Priority                    Manual >Alarm >Video Detection >Continuous.

Recording Interval                   1 to 120 minutes (default: 60 minutes)

Overwrite Mode
  Support

Raid Function                           Support

Search Mode 
Time/Date, Alarm, Motion Detection & exact search (accurate to second)

Playback
 2-channel playback simultaneously. PIP play, pause, stop, rewind, fast play, slow play, next file, previous file, next camera, previous camera, full screen, repeat, shuffle, backup selection.
Digital Zoom                            Selected zone can zoom into full screen during playback

Backup Mode                           Flash disk/ USB HDD/ USB CD-RW/DVD-RW/ network download

Network

Interface                                  RJ-45 Port (10/100M)

Network Functions                  TCP/IP, DHCP, DDNS, PPPoE, E-mail, FTP

Remote operation                    Monitor, PTZ control, playback, system setting, file download, log information

Auxiliary Interface

USB Interface                           2 ports, 1 for mouse control, 1 for backup.
RS232                                      Keyboard, PC communication

RS485                                      PTZ control

Environmental

Power Supply                           220V+10%  50+10%Hz / 110V 60Hz

Power Consumption
 27W (excluding HDD)
Working Temperature               0℃~＋55℃
Working Humidity                      10%~90%

Atmosphere Pressure                86kpa~106kpa
Dimension 
  360mmx275mmx132mm (W*D*H)

Weight                                        6.5 KG

Mounting                                    Desktop or rack
1.3 Special Features
· Time information overlay, channel information overlay, temperature and humidity information overlay
Time information is on the top right corner by default. Channel information is on left down corner by default.  Temperature and humidity information is on top left corner by default. Time information position and channel information position can be modified. Please note, before you overlaying temperature and humidity information, please go to pan/tilt zoom setup interface.  Then set as: LTM8230C, baudrate: 9600, data bit: 7 and stop bit: 2. finally, input corresponding address.
· Good audio effect 

System adopts ADPCM compression algorithm. It can lower the noise and enhance definition  
· Preview PIP
Support PIP(picture in picture) function so that you can view the specified position in detail and at the same time observe the whole situation.

· Real-time backup 
System supports synchronization backup, alternative backup and circle backup.
You can backup current monitor data to the specified DVD (one or several DVDs) and at the same time the specified channel can be memorized in PIP mode.

· Synchronization backup: The specified device can burn the same data at the same time.

· Alternative backup:  system support several burners to backup in turn.  You can insert the newt CD to continue backup once the current one is full. Please note to guarantee there is no data loss between these two CDs, please make sure current record function is normal.

· Circle backup: This function is based on alternative backup. When the first CD is full, burner pop up the CD and you can input a new one. Once the alternative backup is full, system goes back to the first CD to begin burning. 
· Switch CD function

When the CD free space reaches a threshold, system can alarm you. (You can specify a value here. Minimum value is 50M) After you insert the new CD, system reads the previous three to five minutes data to current CD and then start burning. Please note during the whole process, you need to make sure manual record is working properly and record is successfully.

· CD playback

You can use this function to verify the real-time burning data or backup data.  System supports CD playback only. For burning or backup operation, if you have selected multiple channels manually, playback operation is distinguished by channel and the image is the video effect of one specified channel. Sometimes, the specified channel is a PIP mode.
· Title information

To burn the necessary case information, you can set case information via client-end and then send it to the DVR. System can display the title information first and then playback the data.
· PC playback

The burned CD can playback in any PC (support PIP mode). System supports channel switch and watermark search. Please note you need to burn the CD player in current CD. If you have selected multiple channels manually and the playback is in PIP mode, when you play the burning or backup CD you can see system automatically overlays all these channels together and play in PIP mode. 
· Data encryption 
You need to input password to begin CD playback if you have input password when     burning. 

System also support switch encryption data to current popular storage format such as *.WMV、*.AVI、*.ASF、*.MPEG、*.MOV、*.RM so that you can edit the audio if necessary. 
2 Overview and Controls
This section provides information about front panel and rear panel. When you install this series DVR for the first time, please refer to this part first. 

2.1 Front Panel
Front panel is shown as in Figure 2‑1.
[image: image1.png]


[image: image247.jpg][image: image248.jpg]


[image: image249.jpg]


[image: image250.jpg]


[image: image251.png]


[image: image252.jpg]


[image: image2.png]



[image: image253.png]Security Setting:

nternet Zone

Settings

O Disable
© Enable

9] Download signed ActiveX controls (not secure)
O Disable
© Enable (not secure)
O Prompt (recommended)

9] Download unsigned Active controls (not secure)
© Disable (recommended)
© Enable (not secure)
O Prompt

19 Intiaize and scrpt ActiveX contros ok merked as sae for s
O Disable (recommended)
© Enable (not secure)
O Prompt

4] Run ctiveX contros and pug-ins
3 bbb e

< I

*Takes effect after you restart Internet Explorer

Reset custom settings




[image: image254.png]ST




[image: image255.png]



Figure 2‑1
	1
	Burner  

	2
	LCD

	3
	input/output volume  

USB port

IN：audio talk input

OUT：earphone output

	4
	Burning indication light

Power indication light

Burner/stop  button

Power button.  


2.2 Rear Panel

This series DVR real panel is shown as in Figure 2‑2.
[image: image256.png]


[image: image3.jpg]




Figure 2‑2
	1
	Power button

Power socket

Fan

	2
	AUDIO OUT

AUDIO IN

VEDIO OUT

VEDIO IN



	3
	Alarm-RS485 port

RS232 port

VGA port

USB port

NET port（RJ45）


	4
	Pickup port 

Four 12V power sockets


2.3 Remote Control
The remote control interface is shown as in Figure 2‑3.

[image: image4.jpg]1 5 7 8 9 1115 16

2 3 4 6 1012 13 14





Figure 2‑3
	Serial Number 
	Function 

	1
	Remote switch 

	2
	Multiple-window switch 

	3
	0-9 number key 

	4
	Record 

	5
	Auxiliary  key 

	6
	Confirm /menu key 

	7
	Cancel

	8
	Direction key 

	9
	Forward

	10
	Previous

	11
	Back  

	12
	Next

	13
	Slow play

	14
	Stop

	15
	Fast play

	16
	Play/Pause


2.4 Mouse Control
	Left click mouse 
	System pops up password input dialogue box if you have not logged in.

In real-time monitor mode, you can go to the main menu. 

	
	When you have selected one menu item, left click mouse to view menu content.

	
	Implement the control operation.

	
	Modify checkbox or motion detection status.

	
	Click combo box to pop up drop down list

	
	In input box, you can select input methods. Left click the corresponding button on the panel you can input numeral/English character (small/capitalized). Here ← stands for backspace button. ＿ stands for space button.

In English input mode: ​_stands for input a backspace icon and ← stands for deleting the previous character.

                                                  

[image: image5.png]



In numeral input mode:  _ stands for clear and   ← stands for deleting the previous numeral.

When input special sign, you can click corresponding numeral in the front panel to input. For example, click numeral 1 you can input“/” , or you can click the numeral in the  on-screen keyboard directly.




	Double left click mouse 
	Implement special control operation such as double click one item in the file list to playback the video.

	
	In multiple-window mode, double left click one channel to view in full-window. 

Double left click current video again to go back to previous multiple-window mode.

	Right click mouse 
	In real-time monitor mode, pops up shortcut menu: one-window, four-window, Pan/Tilt/Zoom, color setting, search, record, alarm input, alarm output, main menu.

Among which, Pan/Tilt/Zoom and color setting applies for current selected channel.

If you are in multiple-window mode, system automatically switches to the corresponding channel.  

[image: image6.jpg]View 1
View 4

Pan/TiltZoom
Color Setting

Search

CD-ROM Play
Record

REALTIME BACKUP
Alarm Output

Alarm Input

Main Menu





	
	Exit current menu without saving the modification.

	Press middle button
	In numeral input box:  Increase or decrease numeral value.

	
	Switch the items in the check box.

	
	 Page up or page down 

	Move mouse 
	Select current control or move control

	Drag mouse 
	Select motion detection zone

	
	Select privacy mask zone.


2.5 Virtual Keyboard 
The system supports two input methods: numeral input and English character (small and capitalized) input. 

Move the cursor to the text column, the text is shown as blue, input button pops up on the right. Click that button to switch between numeral input and English input (capitalized and small), Use > or < to shift between small character and capitalized character. 
3 Installation and Connections

Note: All the installation and operations here should conform to your local electric safety code.

3.1 Check Unpacked DVR

When you receive the DVR from the shipping agency, please check whether there is any visible damage to the DVR appearance. The protective materials used for the package of the DVR can protect most accidental clashes during transportation. Then you can open the box to check the accessories.  
Please check the items in accordance with the list on the warranty card. Finally you can remove the protective film of the DVR. 
3.2 HDD Installation  
3.2.1 Choose HDDs

We recommend Seagate HDD of 7200rpm or higher.

3.2.2 Calculate HDD Size

This series have no limit to HDD capacity. You can use 120G-750G HDD to guarantee higher stability.

The formula of total HDD size is:

Total Capacity (MB) = Camera Amount * Recording Hours * HDD Usage Per Hour (M/h)

H.264 compression is ideal for standalone DVRs. It can save more than 30% HDD capacity than MPEG4. When you calculate the total HD capacity, you should estimate the average HDD capacity per hour for each channel. 
For example, for a 4-ch DVR, the average capacity of HDD usage per hour per channel is 200M/h. Now if you hope the DVR can record the video 12 hours each day for 30 days, the total capacity of HDDs needed is: 4 channels * 30 days * 12 hours * 200 M/h = 288G. So you need to install one 300G HDD or 2*160G HDDs.

3.2.3  HDD Installation 

Data ribbons, fastening screws and smart HDD shelf design are already provided in the accessories.
              


1. Fix the HDD on the bracket.          2. Connet power and data cable.


3. Fix the HDD bracket on the top.             4.Fix the bracket on the side. 
After HDD installation, please check data ribbon connection and power cord.
3.3 CD/DVD Burner Installation

For built-in burner, you can dismantle front plate to install CD burner. This built-in burner should be set as MASTER. 

For USB burners, you need to install USB series burner. 
This series DVR is compatible with various burner brands popular in today’s market. You can consult our local technical support or visit our website for more information.

3.4 Desktop and Rack Mounting
3.4.1 Desktop Mounting
To prevent surface damage, please make sure that the rubber feet are securely installed on the four corners of the bottom of the unit.

Position the unit to allow for cable and power cord clearance at the rear of the unit. Be sure that the air flow around the unit is not obstructed.

3.4.2 Rack Mounting

The DVR occupies two rack units of vertical rack space. 
The hardware necessary to mount the DVR into a rack is supplied with the unit. 

Install the cabinet in ventilated place. Avoid extreme heat, humid or dusty conditions. You can use a soft dry brush to clean opening outlet, cooling fan and etc regularly.
3.5 Connecting Power Supply

Please check input voltage and device power button match or not. 

We recommend you use UPS to guarantee steady operation, DVR life span, and other peripheral equipments operation such as cameras. 

3.6  Connecting Video Input and Output Devices

3.6.1 Connecting Video Input

The DVR automatically detects the video standard PAL or NTSC BNC（1.0VP-P，75Ω）
, whenever you connect a video input. It accepts both color and black-and-white and analog video.

NOTE:

· Enabling line lock on cameras may cause video distortion. There may be noise in the camera’s power source. If video from one or more cameras is distorted, we recommend you disable line lock on the camera as your first troubleshooting step.

· If a video distribution amplifier is installed between the video source and the DVR, do not set the output video level above 1 Vp-p.

To connect each video input:

1. Connect a coaxial cable to the camera or other analog video source.

2. Connect the coaxial cable to the video in connector on the rear panel.
Please refer to Figure 3‑1 for more information.
NOTE:
You need to use a BNC installation tool to connect coaxial cables to the rear panel.
[image: image7.jpg]



Figure 3‑1
3.6.2 Connecting Video Output

This section provides information about physically connecting video display devices to the DVR.  See Figure 3‑2. 

There are BNC(PAL/NTSC　BNC（1.0VP-P ，75Ω) output and VGA output. 
If you connect the DVR with a TV monitor or VGA monitor, the DVR can automatically detects the monitor type. And without any output device, by default, the DVR is configured to use a TV monitor. In this case, if your application requires a VGA monitor, you have to press the button “FN” or Shift on the front panel.
NOTE: 

Video output 1 and VGA can’t display at the same time. But Video output 2 can display properly with Video Output 1 or VGA.

[image: image8.jpg]Video Output




Figure 3‑2
3.7 Connecting Audio Input & Output, Bidirectional Audio, Looping Video

3.7.1 Audio Input/One Audio Output
Audio input, bidirectional audio input and audio output
The DVR encodes audio and video signals simultaneously, which lets you control audio at the monitored location. 
The audio input adopts BNS port connection. The audio output signal is Line Out, 2Vpp 4Ω(BNC or RCA). It can connect to earphone (low resistance), sound box or use amplifier to driver other audio output device. 
To set up audio:

1. Make sure your audio input device matches the RCA input level. If the device and RCA input levels do not match, audio distortion problems may occur.

2. Make sure the audio connector is wired as follows:

3. Connect a line input device or pre-amplified microphone to the audio connector for the video channel on the rear panel.
Please refer to Figure 3‑3.
[image: image9.jpg]4

Audio Input

Audio Output




Figure 3‑3
3.7.2 Alarm Input and Relay Output

The DVR offers 4 alarm inputs for external signaling devices, such as door contacts or motion detectors. Each alarm input can be either normally open or normally closed. Once configured, an alarm input can invoke many different activities, including triggering a relay device, sending an alert to a security office or storing pre-alarm video to the DVR.
3.7.3 Alarm Input

You should check your alarm input mode is grounding alarm input or not. 

For this series DVR, grounding signal is needed for alarm input. 

If you need to connect two units or one DVR and other device, please use a relay to separate them.
Please refer to Figure 3‑4 for more information.

[image: image10.jpg]Alarm Input




Figure 3‑4
3.7.4 Alarm Output

Do not connect alarm output port directly with high power load (no more than 1 A) in case of heavy current. 
You can use the co-contactor to realize the connection between the alarm output port and the load.
Please refer to Figure 3‑5 for more information.
[image: image11.jpg]Alarm Output





Figure 3‑5
3.7.5 Alarm Input and Output Details
Please note this series product adopts ground alarm:  When alarm circuit connects to ground you can activate an alarm. 
Alarm input and alarm output description ：

a) In alarm 1, from the left to the right are alarm1 to alarm 4 respectively. The alarm becomes activated in low voltage.

b) In alarm1, 1-No c is normal open activation output. In alarm 2, 2-NO C NC is a group of normal close activation output. 
c) +12V is external alarm input. Here needs an external device to provide +12V power. The alarm device power shall be less than 1A,
d) A.B is A/B cable to control 485 device. You can connect to control decoder and etc. You need to use a 120Ohm resistance between A/B cable if there are too many PTZ decoder. 
e) “[image: image12.png]


”：ground cable；
[image: image13.jpg]



You can refer to the following sheet and Figure 3‑6 for alarm input and output information.

	Parameter
	Grounding Alarm  

	Ground
	Ground line

	Alarm Input
	1, 2, …,4

	Relay Output
	1,2,3,4: NO and C(Normally Open and Com)

5: NO,C and NC(Normally Open, Com, Normally Closed)

6: Ctrl 12V(This is used for reset the senor)

	485 A、B
	485 communication port. They are used to control devices such as PTZ.

	+12（C）
	This should input an external power input.


· 4-ch grounding alarm inputs. (Normal open or Normal close type)
· Please parallel connect COM end and GND end of the alarm detector (Provide external power to the alarm detector).

· Please parallel connect the Ground of the DVR and the ground of the alarm detector.

· Please connect the NC port of the alarm sensor to the DVR alarm input(ALARM)

· If you need to reset the touched-off alarm remotely, you can use DVR to supply controllable 12 V power to the alarm detector such as the smoke detector.

· Use the same ground with that of DVR if you use external power to the alarm device.
[image: image14.jpg]Alarm input public end should jump out with device power end
Alarm device connection terminal

12V _|SNo [CoM leg Alarm device
@ =) connection terminal
‘ | © ALARM

@ GND

12y GND





Figure 3‑6
3.7.6 Relay Output Description
· 3 ways relay alarm output. Provide external power to external alarm device.

· To avoid over loading, please read the following relay parameters sheet carefully. (See below table)

· The controllable +12v can be used to restore the smoke detector.
Please refer to Figure 3‑7 for alarm input module information.

[image: image15.jpg]Interior Circuit 1

vce

.
7
i

22K

4 External Signal
Input 1

Interior Circuit 2

22K

+ External Signal
Input 2

Interior Circuit 3

Kz
2
iad

22K

4 External Signal
Input 3

Interior Circuitd ¥

R
2
LaJ

22K

External Signal
Input 4





Figure 3‑7
Please refer to Figure 3‑8 for alarm output module information.

[image: image16.jpg]Interior  Interior

Cirtuit Cirtuit

Trmar 7
| [ awassoon
avamoUT.




Figure 3‑8
Relay Specification<0}
	Model: 
	JRC-27F

	Material of the touch 
	Silver 

	Rating（resistance load） 
	Rated switch capacity 
	30VDC 2A, 125VAC 1A 

	
	Maximum switch power 
	125VA 160W 

	
	Maximum switch voltage 
	250VAC, 220VDC 

	
	Maximum switch currency 
	1A 

	Insulation 
	between touches with same polarity 
	1000VAC 1minute 

	
	between touches with different polarity 
	1000VAC 1minute 

	
	between touch and winding 
	1000VAC 1minute 

	Surge voltage 
	between touches with same polarity 
	1500V (10×160us) 

	Length of open time 
	3ms max 

	Length of close time 
	3ms max 

	Longevity 
	Mechanical 
	50×106 times (3Hz) 

	
	Electrical 
	200×103 times (0.5Hz) 

	Temperature 
	-40℃ ~+70℃ 


3.8 RS232

You can connect the DVR with POS or Keyboard through RS232. 

With POS system, the DVR can communicate through RS232 and network. For the POS system, the DVR can integrate the text content and even search the record through the info.

The series DVR also support NKB operation. You can operate the DVR from the keyboard controls instead of using the control pad on the front panel of the unit.

To connect a NKB keyboard to the DVR:

1. Assemble the KBD keyboard according to the instructions in its accompanying installation manual.

2. Connect the KBD keyboard into one of the RS232 ports on the DVR or through network.
3.9 RS485

When the DVR receives a camera control command, it transmits that command up the coaxial cable to the PTZ device. RS485 is a single-direction protocol; the PTZ device can’t return any data to the unit. To enable the operation, connect the PTZ device to the RS485(A,B) input on the DVR. Since RS485 is disabled by default for each camera, you need to enable the PTZ settings first. This series DVRs support multiple protocols such as Pelco-D, Pelco-P. 
To connect PTZ devices to the DVR:

1. Connect RS485 A,B  on the DVR rear panel.

2. Connect the other end of the cable to the proper pins in the connector on the camera.

3. Follow the instructions for configuring a camera to enable each PTZ device on the DVR.
3.10  Other Interfaces

There are still other interfaces on the DVR, such as USB ports. You can refer to the Figure 3‑9 for more information.
[image: image17.jpg]RS232 RS485

VGA Output




Figure 3‑9
4 Overview of Navigation and Controls 

Before operation, please make sure you have properly installed HDD and all the cable connections are right. 

4.1 Login, Logout & Main Menu 
4.1.1 Login 

When system boots up, default video output is in multiple-window mode.

Click Enter or left click mouse, you can see the login interface. See Figure 4‑1. 
System consists of four accounts: 
· Username: admin.  Password: admin.

· Username: 888888. Password: 888888
· Username: 666666. Password: 666666(Lower user who can only monitor, playback, backup and etc.)
· Username: default. Password: default(hidden user)

For security reasons, please modify you password after first login. 

Please use remote control to input. Click [image: image18.png]


 to switch between numeral, English character (small/capitalized) and denotation.
Note: 
Three times login failure in 30 minutes will result in system alarm and five times login failure will result in account lock!
[image: image19.jpg]SYSTEM LOGIN

User Name (888888 5

Password 123}

3
6
9 ncel
=





Figure 4‑1
4.1.2 Main Menu 

After you logged in, the system main menu is shown as below. See Figure 4‑2.  
There are total six icons: search, information, setting, backup, advanced and shutdown. 
Move the cursor to highlight the icon, then double click mouse to enter the next menu.
[image: image20.emf]
Figure 4‑2
4.1.3 Logout 

In the main window, click shutdown button, you can see an interface is shown as below.  See Figure 4‑3.
[image: image21.jpg]UTDOWN

‘Logou( menu user
Password is needed to re-enter the menu
| after logout.

OK ] | Cancel |





Figure 4‑3
There are several options for you. See Figure 4‑4.
[image: image22.jpg]Logout menu user
Switch user
Restart application
Shutdown

Restart system





Figure 4‑4
Or you can click power button in the front panel for at least three seconds, system will stop all operations. Then you can click the power button in the real panel to turn off the DVR.
4.1.4 Auto Resume after Power Failure 

When DVR is in recording, schedule recording or alarm recording mode, system can auto backup video and resume previous working status after power failure.
4.1.5 Replace button battery 
Please always use the same type battery and check system time regularly. 
We recommend replace battery regularly (such as one-year) to guarantee system time accuracy.
4.2 Recording operation 
4.2.1 Live Viewing
When you login, system is in preview mode。 You can see system date, time and channel name.
You can go to general setup to modify these settings (Main Menu->Setting->General). 
Please refer to the following sheet for channel information.
	1
	[image: image23.png]8]




	Recording status
	3
	[image: image24.png]



	Video loss occurred 

	2
	[image: image25.png]



	Motion detection status
	4
	[image: image26.png]



	Monitor lock status


Please refer to the following sheet for channel status. [image: image27.png]


 stands for opening switch function,  [image: image28.jpg]


 stands for closing switch function.
4.2.2 Manual Record
You need to have proper right to implement the following operation.

4.2.2.1 Go to manual record interface

There are two ways for you to go to manual record interface.

· In the main menu, Advanced->Manual Record.

· In preview mode, click record button in the front panel or record button in the remote control.

Manual record interface is shown as in Figure 4‑5.
4.2.2.2 Basic operation
Here is for you to set manual record setup.
There are three statuses: schedule/manual/stop. Highlighted icon“○” to select corresponding channel.
· Manual: the highest priority. After manual setup, all selected channels will begin ordinary recording.

· Schedule: channel records as you have set in recording setup (Main Menu->Setting->Scheduled) 
· Stop: all channels stop recording.
Note:

These three recording types have the following priority:

Manual>Schedule>Stop.
[image: image29.jpg]RECORD

| Record Mode Al 123 4
Schedule

Manual
Stop eees

OK Cancel





Figure 4‑5
4.2.2.3 Enable/disable record
Please check current channel status: “○” means it is not in recording status, “●” means it is in recording status. 

Then use direction key to highlight channel number.
4.2.2.4 Enable all channel recording 
Highlight ○ below All, you can enable all channel recording. See Figure 4‑6.
· All channel schedule record

Please highlight “ALL” after “Schedule”. When system in schedule recording, all channels will records as you have previous set (Main menu->Setting->Schedule). 

The corresponding indication light in front panel will turn on.
[image: image30.jpg]RECORD

| Record Mode ~ All 1234
| Schedule L e
Manual
Stop

OK Cancel





Figure 4‑6
· All channel manual record

Please highlight “ALL” after “Manual.” When system in manual recording, all scheduled set up you have set in will be null ((Main menu->Setting->Schedule)). The indicator light in front panel will turn on. See Figure 4‑7.
[image: image31.jpg]Record Mode ~ All 1234

Schedule
Manual L ] LR B N J

Stop

OK Cancel





Figure 4‑7
4.2.2.5 Stop all channel recording 

Please highlight “ALL” after “Stop”. System stops all channel recording no matter what mode you have set in the menu (Main menu->Setting->Schedule) See Figure 4‑8.
[image: image32.jpg]



Figure 4‑8
4.3 Search & Playback 
4.3.1 Search Menu  

There are two ways for you to go to search interface.

· Click Pause/play button in the remote control.

· Click search icon in the main menu.

Search interface is shown as below. See Figure 4‑9.
There are three file types:

· R: regular recording file.

· A: external alarm recording file.

· M: motion detection recording file 

There are two display modes:

1p: one window playback.

[image: image33.jpg]1-window Playhack

o File List

3| Playback Window

! »| File Information

14

13

12





Figure 4‑9
Please refer to the following sheet for more information.

	Serial number 
	Function

	1
	Play

	2
	Backward

	3
	Stop

	4
	Slow play

	5
	Fast play

	6
	Previous frame

	7
	Next frame

	8
	Volume

	9
	Previous file

	10
	Next channel 

	11
	Next file

	12
	Previous channel

	13
	Search

	14
	Backup


4.3.2 Basic Operation 

4.3.2.1 Playback 
There are various search engines: video type, channel number or time. 

System can max display 32 files in one screen. You can use up/down button to turn page.

Select the file name and double click mouse (or click enter button), you can view file content.
4.3.2.2 Accurate playback

Input time (h/m/s) in the time column and then click playback button, system can operate accurate playback.
4.3.2.3 Synchronization playback function when playback
During playback process, click numeral key, system can switch to the corresponding channel video of the same time.
4.3.2.4 Digital zoom 
When system in full-screen playback mode, drag your mouse in the screen to select a section and then left click mouse, system operate partial enlargement function, right click mouse to exit. 

4.3.2.5 File backup 

System supports backup operation during search. You can draw a √ before file name (multiple choices). Then click backup button (Button 14 in Figure 4‑9).
4.3.2.6 Slow playback and fast playback 
Please refer to the following sheet for slow play and fast playback function.

	Button 
	Illustration
	Remarks

	Fast forward when playback：fast forward button (
(Or you can turn the shuttle outer ring clockwise.) 
	In playback mode, click this button to switch between various fast play modes such as fast play 1 or fast play 2.


	Frame rate may vary due to different versions.

	Slow play when playback ：Slow play button  ► 


	In playback mode, click this button to switch between various slow play modes such as slow play 1 or slow play 2.


	

	3、Play/Pause►          
	In slow playback mode, click this button to switch between play/pause modes.
	

	4、Previous/next 
	In playback mode, you can click  (and (　to view previous or next video in current channel.
	


4.3.2.7 Fast forward/fast backward and frame by frame playback
	Special Functions of Shuttle 
	Illustration 
	Remarks

	Fast forward(outer ring clockwise)
	When playback, turn the outer ring clockwise one round: you can view in fast speed 1

Turn it two rounds you get fast speed 2.  

You can continue turning to get different speed.


	In forward or backward mode, double click Pause/Play button to get normal playback.

Frame rate may vary due to different versions.

	Fast backward(outer ring counter clockwise)
	When playback, turn the outer ring counter clock-wise one round, you can view in backward speed 1.

Turn it two rounds, you get backward speed 2. 

You can continue turning to get different speed.


	

	Manual playback frame by frame
	In playback mode, click play/pause button, slowly turn the inner dial clock-wise to view frame by frame, counter clock wise to view I frame playback.
	


4.3.2.8 Backward playback and frame by frame playback
	Button 
	Illustration
	Remarks

	Backward play（
  ( in playback interface ）
	In normal playback mode, left click backward play button, system begins backward playback.

Double click backward play button again, system goes to pause mode.


	When system is in backward play or frame by frame playback mode, click play button system goes to normal playback

	Manual playback frame by frame.
	Click pause button in normal playback mode, slowly turn the inner dial clock-wise to view frame by frame, counter clock wise to view I frame playback.
	


4.3.3 Backup in search interface 
Draw a ( before the checkbox to select the file you want to backup (You can select files in two channels.). And then click backup button [image: image34.jpg]


 (Button 14 in Figure 4‑9.)
4.3.4 Calendar 

Click calendar icon[image: image35.jpg]


, system pops up calendar for your reference. Highlighted date means there are video files in that day. You can click blue date to view file list.  See Figure 4‑10. There are video files in March 13th and 14th. Double click date to view file list.

[image: image36.jpg]2007-06-26

< ) Uune ~| (2007 >
Sun Mon Tue Wed Thu Fri Sat
1 2

3JC4IC5]C 6L TIC8ICY
10 J[11 ][ 12 ) I s | 15 /[ 16
17 J(18](19 (20 J(21 J(22]((28
247](257)(267](27 )28 ](29 )30




 
  Figure 4‑10
4.4 Record setup (Schedule)
In the main menu, from setting to schedule, you can go to schedule menu. See Figure 4‑11.

There are three record types: R-Regular, MD-Motion detection, A- Alarm.  
[image: image37.png]Channel  [1__[~JPreRecord(10 |sec. Redundancy ll

WeekDay (Mon _|~] Record Type  Regular MD Alarm
Period1 [00 :00 -08:00 | [J | ] (]
Period2 (0800 -12:00 | W 3] ]
Period3 [12:00 20:00 | [J @) | ]
Period4 [00:00  -24:00 | [J (] O
Period5 (00 :00  -24 :00 | O ()] (]
Period 6 [00 :00  -24 :00 | O ()] (]
I Regular B MD [ | Alarm

_H 15 18 21 24
P ———





Figure 4‑11
4.4.2 Basic Operation
There are total six periods. See Figure 4‑11.
· Channel: Please select the channel number first. You can select “all” if you want to set for the whole channels.

· Week day: There are eight options: ranges from Saturday to Sunday and all.

· Redundancy: System supports redundancy backup function.  You can highlight Redundancy button to activate this function. Please note, before enable this function, please set at least one HDD as redundant.(Main menu->Advanced->HDD Management) 

· Prerecord: System supports prerecord function. The previous one to three seconds video before alarm occurs can be included in recorded video.

· Record types: There are three types: regular, motion detection (MD) and Alarm.

Please highlight icon [image: image38.jpg]


 to select the corresponding function. 
At the bottom of the menu, there are color bars for your reference. Green stands for regular recording, yellow stands for motion detection and red stands for alarm recording.

4.4.1.1 Quick Setup

This function allows you to copy one channel setup to another. After setting in channel 1, you can click paste button and turn to channel 2 and then click copy button. You can finish setting for one channel and then click save button or you can finish all setup and then click save button to memorize all the settings.

4.4.1.2 Redundancy 

Redundancy function allows you to memorize record file in several disks. These files are created, packaged and closed simultaneously.  When there is file damage occurred in one disk, there is a spare one in the other disk. You can use this function to maintain data reliability and safety.

In the main menu, from Setting to Schedule, you can highlight redundancy button to enable this function. See Figure 4‑11.
In the main menu, from Advanced to HDD management, you can set one or more disk(s) as redundant. You can select from the dropdown list. See Figure 4‑12. System auto overwrites old files once hard disk is full. 
Please note only read/write disk or read-only disk can backup file and support file search function, so you need to set at least one read-write disk otherwise you can not record video.

Note
About redundancy setup:

· If current channel is not recording, current setup gets activated when the channel begin recording the next time. 

· If current channel is recording now, current setup will get activated right away, the current file will be packet and form a file, then system begins recording as you have just set. 

After completed all the setups please click save button, system goes back to the previous menu.
[image: image39.jpg]o o DD ANAGEVENT, |

Capacity ~ 79.99 GB Format

Record time 00-00-00 00:00:00 / 00-00R85VEr

SATA i) Alarm Set
- Alarm Release
HDDNo. [1 ~| Setto [Read/Write Execute |
Read/Write
Type Read/Write Read only
Status Normal Redundant





Figure 4‑12
Playback or search in the redundant disk.

There are two ways for you to playback or search in the redundant disk.

· Set redundant disk(s) as read-only disk or read-write disk (Main menu->Advanced->HDD management).  See Figure 4‑12.System needs to reboot to get setup activated. Now you can search or playback file in redundant disk.

· Dismantle the disk and play it in another PC.

4.5 Motion Detect 
4.5.1 Go to Detect Menu 
In the main menu, from Setting to Detect, you can see motion detect interface. See Figure 4‑13. There are three detection types: motion detection, video loss, camera masking.
4.5.2 Motion Detect
Detection menu is shown as below. See Figure 4‑13.
· Channel: select the channel you want to implement motion detection.

· Event type: from the dropdown list you can select motion detection type.
· Channel: select the channel to activate recording function once alarm occurred. Please make sure you have set MD record in encode interface(Main Menu->Setting->Schedule) and schedule record in manual record interface(Main Menu->Advanced->Manual Record) 
· Latch: when motion detection complete, system auto delays detecting for a specified time. The value ranges from 10-300(Unit: second)
· Region: Click select button, the interface is shown as in Figure 4‑14.Here you can set motion detection zone. There are 396(PAL)/330(NTSC) small zones.

· Sensitivity: System supports 6 levels. The sixth level has the highest sensitivity.

· Show message: System can pop up a message to alarm you in the local host screen if you enabled this function.

· Send email: System can send out email to alert you when alarm occurs. 
· PTZ activation: Here you can set PTZ movement when alarm occurs. Such as go to preset, tour &pattern when there is an alarm. Click “select” button, you can see an interface is shown as in Figure 4‑15.
· Period: Click set button, you can see an interface is shown as in Figure 4‑16. Here you can set for business day and non-business day. In Figure 4‑16, click set button, you can see an interface is shown as in Figure 4‑17. Here you can set your own setup for business day and non-business day.

· Anti-dither: Here you can set anti-dither time.
· Sensitivity: there are six levels. The sixth level has the highest sensitivity.
· Alarm output: when alarm occurred, system enables peripheral alarm devices.
· Tour: Here you can enable tour function when alarm occurs.  It is a one-window tour. Please go to chapter 5.3.9 Display for tour interval setup. 
Please highlight icon [image: image40.jpg]


 to select the corresponding function. After all the setups please click save button, system goes back to the previous menu.
Note:

In motion detection mode, you can not use copy/paste to set channel setup since the video in each channel may not be the same.  

In Figure 4‑14, you can left click mouse and then drag it to set a region for motion detection. Click Fn to switch between arm/withdraw motion detection. After setting, click enter button to exit. 
[image: image41.jpg]Event Type Motion Detect ¥ ~ Channel (il %
Enable a

Region [ Select | Sensitivity 15 ]
Period [=—Sett) Anti-dither

M Alarm Out 2) Latch

W Show Message W Send Email

W Record Channel (2 3

WPTZ Activation Delay 10 sec.

[OTour




                                                                       
Figure 4‑13
[image: image42.jpg]



Figure 4‑14
[image: image43.jpg]CAM1 [None
CAM 3 [None





Figure 4‑15
[image: image44.png]WorkDay  |~| [ Set |

W (0000 24 :00 [)[00:00  -24 :00

()00 :00  -24 :00 ()00 :00  -24 :00

()00 :00  -24 :00 ()00 :00  -24 :00
[t} 3 9 12 15 18 21 24

Sun

Mon

Tue

Wed

Thu

Fri

Sat ———————————————
0 3 9 12 15 18 21 24

(Copy )( Paste )( Defaut ) OK ) ( Cancel )





Figure 4‑16
[image: image45.png]Sun  Mon Tue Wed Thu Fri Sat
Work Day ® L L L L (o] (o]
FreeDay O O O O O L] L]





Figure 4‑17
4.5.3 Video Loss 
In Figure 4‑13, select video loss from the type list. You can see the interface is shown as in Figure 4‑18.This function allows you to be informed when video loss phenomenon occurred. You can enable alarm output channel and then enable show message function.

· Channel: select the channel you want to enable lens shading alarm.

· Event type: please select video loss.

· Channel: select the channel to record when video loss occurred.
· Alarm output: activate peripheral alarm device when video loss occurred.
· Latch: when motion detection complete, system auto delays detecting for a specified time. The value ranges from 10-300(Unit: second)
· Show message: System can pop up a message to alarm you in the local host screen if you enabled this function.

· Send email: System can send out email to alert you when alarm occurs. 
· PTZ activation: Here you can set PTZ movement when alarm occurs. Such as go to preset, tour & pattern when there is an alarm. Click “select” button, you can see an interface is shown as in Figure 4‑15.
· Period: Click set button, you can see an interface is shown as in Figure 4‑16. Here you can set for business day and non-business day. In Figure 4‑16, click set button, you can see an interface is shown as in Figure 4‑17. Here you can set your own setup for business day and non-business day.
· Sensitivity: there are six levels. The sixth level has the highest sensitivity.
· Alarm output: when alarm occurred, system enables peripheral alarm devices.
· Tour; Here you can enable tour function when alarm occurs.  It is a one-window tour. Please go to chapter 5.3.9 Display for tour interval setup. 
[image: image46.jpg]Event Type VideoLoss |~| Channel 1 =

Enable ]

Period
W Alarm Out
W Show Message W Send Email

WRecord Channel [l
WPTZ Activation [

_ITour

Latch [10 sec.

Delay (10 sec.

Copy Paste ) [ Default ) | Save || Cancel |





Figure 4‑18
4.5.4 Camera Masking
When someone viciously masks lens, the system can alert you to guarantee video continuity. Camera masking interface is shown as in Figure 4‑19.

· Channel: select the channel you want to enable camera mask detection function.

· Event type: please select camera mask detect from the dropdown list.
· Channel: select the channel to record when camera mask occurred.
· Alarm output: activate peripheral alarm device when camera mask occurred.
· Enable tour: Here is for you to activate tour between different cameras.

· Latch: when motion detection complete, system auto delays detecting for a specified time. The value ranges from 10-300(Unit: second)
· Show message: System can pop up a message to alarm you in the local host screen if you enabled this function.

· Send email: System can send out email to alert you when alarm occurs. 
· PTZ activation: Here you can set PTZ movement when alarm occurs. Such as go to preset, tour &pattern when there is an alarm. Click “select” button, you can see an interface is shown as in Figure 4‑15.
· Period: Click set button, you can see an interface is shown as in Figure 4‑16. Here you can set for business day and non-business day. In Figure 4‑16, click set button, you can see an interface is shown as in Figure 4‑17. Here you can set your own setup for business day and non-business day.
· Sensitivity: there are six levels. The six-level has the highest sensitivity.
· Alarm output: when alarm occurred, system enables peripheral alarm devices.
· Tour:  Here you can enable tour function when alarm occurs.  It is a one-window tour: Please go to chapter 5.3.9 Display for tour interval setup. 
Note:

In this interface, copy/paste function is only valid for the same type, which means you can not copy a channel setup in video loss mode to camera masking mode.

[image: image47.jpg]Event Type
Enable L}

Period
B Alarm Out

B Show Message
WRecord Channel [}
BPTZ Activation [

OTour

Latch
W Send Email

Delay

(10

Jsec.

(10

Jsec.

[ Copy )[ Paste

Default

Al
) (

Save

) |

Cancel





Figure 4‑19
4.6 Alarm Setup and Alarm Activation 
Before operation, please make sure you have properly connected alarm devices such as buzzer.

4.6.1 Go to alarm setup interface

In the main menu, from Setting to Alarm, you can see alarm setup interface. See Figure 4‑20.

4.6.2 Alarm setup
Alarm interface is shown as below. See Figure 4‑20.
· Alarm in: here is for you to select channel number.
· Event type: there are two types. One is local input and the other is network input. 
· Type: normal open or normal close.
· PTZ activation: Here you can set PTZ movement when alarm occurs. Such as go to preset, tour& pattern when there is an alarm. Click “select” button, you can see an interface is shown as in Figure 4‑21.
· Period: Click set button, you can see an interface is shown as in Figure 4‑22. Here you can set for business day and non-business day. In Figure 4‑22, click set button, you can see an interface is shown as in Figure 4‑23. Here you can set your own setup for business day and non-business day.

· Anti-dither: Here you can set anti-dither time.

· Show message: System can pop up a message to alarm you in the local host screen if you enabled this function.

· Send email: System can send out email to alert you when alarm occurs. 
· Record channel: you can select proper channel to record alarm video (Multiple choices). At the same time you need to set alarm record in schedule interface (Main Menu->Setting->Schedule) and select schedule record in manual record interface (Main Menu->Advance->Manual Record).
· Latch: Here is for you to set proper delay duration. Value ranges from 10 to 300 seconds. System automatically delays specified seconds in turning off alarm and activated output after external alarm cancelled.
· Tour: Here you can enable tour function when alarm occurs.  It is a one-window tour: Please go to chapter 5.3.9 Display for tour interval setup. 
Please highlight icon [image: image48.jpg]


 to select the corresponding function. After all the setups please click save button, system goes back to the previous menu.

[image: image49.jpg]Event Type Local Alarm | ~| Alarmin 1 =
Enable | ] Type Normal Open  ~|
Period ~ Set Anti-dither 0 sec

W Alarm Out C Latch (10 |sec

M Show Message W Send Email

MRecord Channel |2

BPTZ Activation | Delay 10 |sec

W Tour 1110

0 7Copy [ Paste " Default [ save ~ Cancel





Figure 4‑20
[image: image50.jpg][~ — P12 Actvaiion | —

CAM 1 [None [<] 0] cAmM2 [None 5 L o |
CAM 3 [None <] [0_) CAM4  [None 5}

Cox ) (Cancel )





Figure 4‑21
[image: image51.png]WorkDay  |~| [ Set |

W (0000 24 :00 [)[00:00  -24 :00

()00 :00  -24 :00 ()00 :00  -24 :00

()00 :00  -24 :00 ()00 :00  -24 :00
[t} 3 9 12 15 18 21 24

Sun

Mon

Tue

Wed

Thu

Fri

Sat ———————————————
0 3 9 12 15 18 21 24

(Copy )( Paste )( Defaut ) OK ) ( Cancel )





Figure 4‑22
[image: image52.png]Sun  Mon Tue Wed Thu Fri Sat
Work Day ® L L L L (o] (o]
FreeDay O O O O O L] L]





Figure 4‑23
4.7 Real-time Backup and Burn
4.7.1 Real-time Backup
Right click mouse, system pops up the following shortcut menu. See Figure 4‑24.
[image: image53.png]View 1
View 4
View PIP

Pan/TiltZoom
Color Setting

Search
CD-ROM Play
Record

Realtime Backup

Alarm ®Dutput
Main Menu





Figure 4‑24
Click real-time backup, you can see an interface is shown as in Figure 4‑25.
[image: image54.png]Selected Device [Dev(IDE DVD/| ¥/

Record Channel [l B3]

Free space on the current disk (50

B Alarm Out [1](2][3)
B Show Message

MB [ Buzzer () Stop

Total Capacity  1.00 GB
Left Capacity 1.00 GB
Burn't Capacity  0.00 KB
Start Time 00:00:00

Tatal Dt 0n-00-00

(Synchronize ) ( Alternate ) (Cycling Burning | (Change Disk |





Figure 4‑25
You can select device from the dropdown list. System supports CD backup and DVR backup. Usually do not burn in these two types at the same time. 
· Record channel: now system max support backup 2 channels at the same time.

· Alarm out: System supports 3 channels.
· Show message: system pops up message in the screen to alert you.
· Synchronize: the selected device burns the same data. Once completes, system auto stop burning. Or you can terminate manually during the burning process.
· Alternative: system begins burning from the first device, then the second device. System repeats the previous process until completing the whole burning process.
· Cycling burning: If you have chose several devices, you can use this function to burn among these devices.
· Change disk: This function allows you to manually change disk. System will burn the last five minutes video of the previous disk into the new disk. Click change disk button, system goes back to device interface. (System auto cancel change disk function if you do not select device to backup after 15 minutes). Click synchronism button to continue burning. During the whole process please make sure record function is proper (Main Menu->Advanced->Manual Record). Do not select stop for current backup channel! See Figure 4‑26.
[image: image55.jpg]| Record Mode Al 1234
[ Schedue

Manual

Stop evee

OK Cancel





Figure 4‑26
When you select synchronize burning or alternative burning function, you can view current backup status, at the same time, synchronize (or alternative) button becomes stop button. See Figure 4‑27.                    
[image: image56.png]Selected Device [Dev(IDE DVD/| 7|

Record Channel [lB(Z](2)

Free space on the current disk (50

B Alarm Out 7]
B Show Message

MB [ Buzzer () Stop

Total Capacity  1.00 GB
Left Capacity 1022.53 MB
Burn't Capacity  1.50 MB
Start Time 14:09:07

St bt 0n-00:0:

i

( Stop ) (Alternate ) (Cycling Burning | (Change Disk |





  Figure 4‑27
During the backup process, you can click stop button to terminate current burning. System pops up warning dialogue box asking you to confirm. See Figure 4‑28.
[image: image57.png]Backup is processing, confirm cancel?





Figure 4‑28
You can see the following dialogue box when backup completed. See Figure 4‑29.
[image: image58.jpg]Backup completed!





Figure 4‑29
Since DVR file is huge, system can divide it into several files and each file size is 600M (usually the last file is smaller than 600M). System will adjust in accordance with you backup size to facilitate data backup. 

Tips:

During the backup process, you can overlay special time title so that you can search easily in the future. 

You can also connect DVR alarm in 1 with peripheral device, system will auto overlay time information in corresponding video when alarm occurred.
4.7.2 Backup 
You can go to backup interface from the main menu.  See Figure 4‑30.
[image: image59.jpg]s

INFO SETTING
ADVANCED BACKUP SHUTDOWN

#'Backup recorded files onto other mediums though CDRW drive or
USB harddrives.





Figure 4‑30
Click backup button, the interface is shown as in Figure 4‑31. Please note current burning is the same as real-time burning. .
[image: image60.png]4 Name(Type) Left space/Total space Device Statu;
1 |v Dev(USB DISK) 1.00 GB/1.00 GB Ready
2 || Dev(IDE DVD/CD-RW) 1.00 GB/1.00 GB Ready
3 |/ Dev(1394 DHFS) 1.00 GB/1.00 GB Ready
4 |V Dev(USB DVD/CD-RW) 1.00 GB/1.00 GB Ready

“tPage Up 14 Page Down r- SelectCancel backup device or file.
( Detect ) ( Backup | ( Burning |





Figure 4‑31
In Figure 4‑31, click backup button, the interface is shown as in Figure 4‑32.
System supports ordinary backup and PIP (Picture in picture).

If you want to enable PIP function, you need to note:
· Select two different channels 

· These two recorded files start time and end time shall be less than 2 seconds. 
[image: image61.png]Selected Device [Dev(USB DISK ~/

Type All ~| ® PP

Channel EOREE

Start Time [@)] (2008 - 09-10 [00: 00: 00 ("Remove )

TME [-] N@) (2008 - 11-10 [16: 14: 39 (Add )
0 |CH Type Start Time End Time Size(KB)

M Page Up !4 Page Down rn Select’Cancel backup device or file.

Space Required / Space Remaining:0.00 KB/1.00 GB





Figure 4‑32
Select the device, type, channel number and then input start time and end time in Figure 4‑32. Click add button, system auto begins searching files. There is a  √ before all matched file names. See Figure 4‑33.
You can continue setting time period and click add button to search. All matched files are listed below, you can use page up/down to view.

You can view backup information at the bottom of the interface. Please note system only backup those files that there is a √ before file name.
[image: image62.jpg]Selected Device |Dev(USB DISK v/

Type All ~| B PP
Channel A 3]
Start Time /@] (2008 - 05- 07 [00: 00: 00 Remove
TIVE |~ @) (2008 - 11-07 [14:05: 13 Add
0 |CH Type Start Time End Time Size(KB)
1|/ 2 R 080626 11:37:15  08-06-26 11:37:23 7504
2 |v 1 R 0806-2611:37:15  08-06-26 11:37:23 7484
3 |v 2 R 0806-2611:37:30 08-06-26 11:37:34 4124
4 |v 1 R 08-06-2611:37:30  08-06-26 11:37:34 4108
S PageUp 14 Page Down Fn SelectCancel backup device or file.
Space Required / Space Remained129.00 MB/1.00 . Start





Figure 4‑33
Click backup button, system begins burning. At the same time, the backup button becomes stop button. You can view the remaining time and process bar at the left bottom. See Figure 4‑34.
[image: image63.jpg]Selected Device |Dev(USB DISK v/

Type All ~| W PP

Channel A 3]

Start Time /@] (2008 - 05- 07 [00: 00: 00 Remove

TIVE |~ @) (2008 - 11-07 [14:05: 13 Add
0 |CH Type Start Time End Time Size(KB)
1| 2 R 08062611:37:15 08-06-26 11:37:23 7504
2 | 1 R 0806-26 11:37:15  08-06-26 11:37:23 7484
3 |V 2 R 0806-26 11:37:30  08-06-26 11:37:34 4124
4 |v 1 R 08:06-2611:37:30  08-06-26 11:37:34 4108
S PageUp 14 Page Down Fn SelectCancel backup device or file.
B Remaining time 0:4:26 Stop





Figure 4‑35
When backup completed, you can see the following interface. See Figure 4‑36.
[image: image64.jpg]Backup finished.





Figure 4‑36
Tips:
During backup process, you can click ESC to exit current interface; but system will not terminate backup process. 

Note:

When you click stop button during the burning process, there are two conditions for different devices:

· For CD-ROM device, the stop function becomes activated immediately and there is no data in the CD.

· For USB device, system can backup the data before you click stop button. For example, if there is a file of 10 minutes, when you click stop after five minutes backup, system only save the previous 5-minute data in the device.

You can use Fn or cancel button to delete  √ after file serial number. The file name format usually is: SN_CH+channel number+time Y+M+D+M+S，among which the YDM format is the same as  you set in general interface. (Main Menu ->Setting ->General).
You can visit our website to view listed CD-ROM type.

4.8 CD playback 
To verify real-time backup data, this series DVR supports CD playback.
It supports real-time burning data verification and backup data verification.
In Figure 4‑24, click CD playback button, you can see an interface is shown as below. See Figure 4‑37.
[image: image65.jpg]M The current cpu’s use rate is 25.000000

1p

B File Name

Start Time

End Time

Size(KB)





Figure 4‑37
Sg0 means it is a CD device. Click search button, you can view file list in current CD. See Figure 4‑38.
Double click file name, you can playback current file.

If you select channel 2, system auto playback files in channel 2.

Please note: real-time backup and CD playback are both operations for CD. You can not operate these two functions at the same time.

[image: image66.jpg]B The current cpu’s use rate is 25.000000

1p

B File Name
videol.dav
video2.dav
Vvideo3.dav
videod.dav
Videob.dav
Vvideob.dav
video?.dav
video8.dav

Start Time
06-08-17 15:31:40
End Time

Size(KB) 6291456
[ I EN Q)





Figure 4‑38
4.9 PTZ Control and Color Setup

Note:

All the operation here is based on PELCO protocol. 

For PELCO protocol users there might be slightly difference.  

4.9.1 Cable Connection

[image: image67.jpg]



Figure 4‑39
Please follow the procedures below to go on cable connection

· Connect the dome RJ 485 port to DVR 485 port. See Figure 4‑39.
· Connect dome video output cable to DVR video input port.

· Connect power adapter to the dome.

4.10  PTZ Setup

Note: The camera video should be in the current screen.
Before setup, please check the following connections are right:

· PTZ and decoder connection is right. Decoder address setup is right.

· Decoder A (B) line connects with DVR A (B) line.

Boot up the DVR, input user name and password.

Click system setup, and then click Pan/Tilt control button. The interface is shown as in Figure 4‑40. Here you can set the following items:
· Channel: select the current camera channel.
· Protocol: select corresponding PTZ protocol(such as PELCO)
· Address: default address is 1.
· Baud rate: select corresponding baud rate. Default value is 9600.
· Data bits: select corresponding data bits. Default value is 8.
· Stop bits: select corresponding stop bits. Default value is 1.
· Parity: there are three options: odd/even/none. Default setup is none.
[image: image68.png]Channel |1 7
Protocol |PELCOD 7
Address (1

Baudrate (9600 N
Data Bits |7 7
Stop Bits |2 9
Parity None -

( Copy ) Pase |( Defaut ) ( Save [ Cancel |





Figure 4‑40
After completed all the setting please click save button. 

In one window display mode, right click mouse (or click “Fn” key in the remote control). The interface is shown as in Figure 4‑41.
[image: image69.png]View 1
View 4
View PIP

Pan/Tilt’foom

Color Seﬁng

Search
CD-ROM Play
Record
Realtime Backup
Alarm Output
Main Menu





Figure 4‑41
Click Pan/Tilt/Zoom, the interface is shown as in Figure 4‑42.
Here you can set the following items:

· Zoom 

· Focus 

· Iris

Click icon [image: image70.bmp] and [image: image71.bmp] to adjust zoom, focus and Iris.
[image: image72.png]® zoom @

® Focus @

(] Iris (]
Set

Page Switch





Figure 4‑42
For detailed operation instruction, please refer to chapter 6.

4.11 Temperature/Humidity Overlay 
In PTZ interface (main menu(setting(Pan/Tilt/Zoom), you can set as below:

· Protocol: LTM8230C 

· Data bit: 7
· Stop bit: 2
Then you can select compounding baud rate and address. LTM8230 support three baud rates: 4800/9600/19200/ default setup is 9600.
Since temperature and humidity device baud rate is set by command, you need to check whether temperature and humidity is connected to the DVR or not first.  

Please follow the steps below:

Set one baud rate and then input proper address (the temperature and humidity value) . Click save button to exit. 

Then please go to PTZ setup interface, modify address and then click save button to exit. 

Now you can check temperature and humidity address changes or not.  If it changes, then current setup is O.K. you have enabled overlay function. 

If it does not change, you need to restore previous address and set again. 
4.12 Color Setting 
In Figure 4‑41, highlight color setting item and then left click mouse, you can see an interface is shown as in Figure 4‑43. You can set period and then set color.  Please note period here are from 00.00 to 24.00.
[image: image73.png]Period W (00 :00 24 :00 ) (00 :00 -24 :00

Hue — g 50 —a—— 50
Brightness —f— 50 —f— 50
Contrast —f— 50 —f— 50
Saturation —f— 50 —f— 50
Gain | —p5— 5 | —p5— 5
White Level B [low 7 o [low 7





Figure 4‑43
5 Understanding of Menu Operations and Controls
5.1 Menu Tree

This series DVR menu tree is shown as below.
[image: image74.jpg]Menu —

Backup

Information

Setting

@

Search
Advanced

Shutdown

Detect Device
Backup Device

HDD Info
BPS

Log

Version
Online Users

General
Encode
Schedule
Rs232
Network
Alarm

Detect

Pan/ TiltZoom
Display
Default

HDD Management
Alarm Output

Abnormity
Manual Record
Account

Auto Maintain
TV Adjust




5.2 Main Menu 

After you logged in, the system main menu is shown as below. See Figure 5‑1 .  There are total six icons: search, Information, setting, backup, advanced and shutdown. Move the cursor to highlight the icon, then double click mouse to enter the sub-menu.

[image: image75.emf]
Figure 5‑1 

5.3 Setting

In main menu, highlight setting icon and double click mouse. System setting interface is shown as below. See Figure 5‑2.

[image: image76.emf]


Figure 5‑2 

5.3.1 General 

General setting includes the following items. See Figure 5‑3.

· System time: here is for you to set system time 

· Date format: there are three types: YYYYY-MM-DD: MM-DD-YYYYY or DD-MM-YYYY.

· Date separator: there are three denotations to separate date: dot, beeline and solidus.
· DST: Here you can set DST time and date. Please enable DST function and then click set button. You can see an interface is shown as in Figure 5‑4. Here you can set start time and end time by setting corresponding week setup. In Figure 5‑4, enable date button, you can see an interface is shown as in Figure 5‑5. Here you can set start time and end time by setting corresponding date setup.
· Time format: there are two types: 24-hour mode or 12-hour mode. 

· Language: system supports various languages: Chinese (simplified), Chinese (Traditional), English, Italian, Japanese, French, Spanish (All languages listed here are optional. Slight difference maybe found in various series.)

· HDD full: Here is for you to select working mode when hard disk is full. There are two options: stop recording or rewrite.

· Pack duration: Here is for you to specify record duration. Default value is 60 minutes.

· DVR No: when you are using one remote control to control several DVRs, you can give a name to each DVR for your management.

· Video standard: There are two formats: NTSC and PAL.

· Auto logout: Here is for you to set auto logout interval once login user remains inactive for a specified time. Value ranges from 0 to 60 minutes.

Note:
Since system time is very important, do not modify time casually unless there is a must!
After completing all the setups please click save button, system goes back to the previous menu.

[image: image77.png]System Time 2008 - 05- 05 |11: 00: 08 [ Save ]
Date Format YYYYMMDD |~

Date Separator |- ~ W osT
Time Format  [24-HOUR 7

Language ENGLISH 9

HDD Full Overwrite N

Pack Duration (60 min.

DVR No. 8

Video Standard [PAL 7

Auto Logout 10 min.





Figure 5‑3 
[image: image78.png]® Dayof Week O Date

Start: [Jun MELED | (Sat_|~] 00 : 00

End: [Sep MELED ~| |Sun |~ 00 : 00

(cancsi )





Figure 5‑4
[image: image79.png]O Day of Week ® Date

Start: @2008 -06-01 00: 00
End: @2008 -09-01 00: 00





Figure 5‑5
5.3.2 Encode

Encode setting includes the following items. See Figure 5‑6.
Please note some series do not support extra stream. 
· Channel: Select the channel you want.

· Compression: system supports H.264. Or you can select from the dropdown list.
· Resolution: System supports various resolutions, you can select from the dropdown list. For this model, we can support D1/CIF.
· Bit rate: system supports two types: CBR and VBR. In VBR mode, you can set video quality.
· Quality: There are six levels ranging from 1 to 6. The sixth level has the highest image quality.

· Frame rate: there are six levels: 1 f/s,2f/s,3f/s, 6f/s,12f/s,25f/s. (Some series DVRs only support PAL 25f/s )
· Video/audio: you can enable or disable the video/audio.
· Overlay: click overlay button, you can see an interface is shown in Figure 5‑7.
· Cover area (Privacy mask): Here is for you to set window blanking section. You can drag you mouse to set proper section size.
· Preview/monitor: privacy mask has two types. Preview means the privacy mask zone can not be viewed by user when system is in preview status. Monitor means the privacy mask zone can not be view by the user when system is in monitor status.
· Time display: You can select system displays time or not when you playback.

· Channel display: You can select system displays channel number or not when you playback.
System default setup is:

· Channel:1

· Compression:H.264

· Resolution: CIF/D1

· Bit rate: CBR

· Quality: 4
· Frame rate: 25f/s
Please highlight icon [image: image80.jpg]


 to select the corresponding function.

[image: image81.jpg]| ENCODE

-
Compression H.264 &)
Resolution CIF
Frame Rate(FPS) (25 16 ké|
Bit Rate Type [cBR ) [cER =)
BitRate(Kb/S)  [Customil~] (620 (896 |-
Reference Bit Rate 480-760Kb/S 736-1056Kb/S
AudionVideo 0O m 00
'OVERLAY |
( Copy ,| l Paste | [ Default l [ Save | Cancelj





Figure 5‑6
[image: image82.png]Cover-Area () Preview [_] Monitor Set

Time Display B Monitor Set
Channel Display B Monitor Set





Figure 5‑7
5.3.3 Schedule 

Please refer to chapter 4.4 schedule.
5.3.4 RS232

RS232 interface is shown as below. Here are five items. See Figure 5‑8.

· Function: There are various devices for you to select. Console is for serial port or min-end platform to upgrade program. Keyboard is for you to use special keyboard to control current device. 
· Baud rate: You can select proper baud rate.

· Data bit: You can select proper data bit.

· Stop bit: There are three values: 1/1.5/2.

· Parity: there are three choices: none/odd/even.

After completing all the setups please click save button, system goes back to the previous menu.
[image: image83.emf]


                                                     Figure 5‑8 

5.3.5 Network 

Here is for you to input network information. See Figure 5‑9. 
· IP address: Here you can input IP address.

· DHCP: It is auto search IP function. When enable DHCP function, you can not modify IP/Subnet mask /Gateway. These values are from DHCP function. If you have not enabled DHCP function, IP/Subnet mask/Gateway display as o. You need to disable DHCP function to view current IP information.  Besides, when PPPoE is operating, you can not modify IP/Subnet mask /Gateway. 
· TCP port: Default value is 37777. (System server port 37778 is reserved for network UDP use.)

· UDP port: Default value is 37778.
· HTTP port: Default value is 80.
· Max connection: system support maximal 10 users. 0 means there is no connection limit.

· Transfer mode: Here you can select the priority between fluency/video qualities.

· Network download: System can process the downloaded data first if you enable this function. 

After completing all the setups please click save button, system goes back to the previous menu.
[image: image84.png]IP Address 192 . 168 . 1 . 108 [J pHeP

Subnet Mask [255 . 255 . 255 . 0
Gateway 192 . 168 . 1 . 1

TCP Port 1 HTTP Port 1

UDP Port 0 Max Connection [0

()] Transfer Mode  [Fluency '

O Network Download

ADVANCED SETTING.
IP FILTER Trusted Sites:0 ]
NTP 0.0.0.0: 10 I
MULTICAST 239.255.42.42 B

| PPPOE username





Figure 5‑9 
5.3.5.1 Advanced Setup
Advanced setup interface is shown as in Figure 5‑10. Please draw a circle to enable corresponding function and then double click current item to go to setup interface. 
[image: image85.png]ADVANCED SETTING.

IP FILTER
NTP
MULTICAST
v PPPOE
DDNS

Trusted Sites:0
0.0.0.0: 10
239.255.42.42
username





Figure 5‑10
5.3.5.2 IP Filter 

IP filter interface is shown as in Figure 5‑11. You can add IP in the following list.  The list supports max 64 IP addresses.  

Please note after you enabled this function, only the IP listed below can access current DVR. 

If you disable this function, all IP addresses can access current DVR.
[image: image86.png]Restricted Type |Trusted Sites |~
0 -0 .0 .0 |( AddP )

| (Delete P | (Deleteal P)





Figure 5‑11
5.3.5.3 Multiple Cast Setup 

Multiple-cast setup interface is shown as in Figure 5‑12. 
[image: image87.png]IPAddress [0 . 0 . 0 . 0

Port 36666





Figure 5‑12
Here you can set a multiple cast group. Please refer to the following sheet for detailed information. 
· IP multiple cast group address 

-224.0.0.0-239.255.255.255

-“D” address space

· The higher four-bit of the first byte=”1110”
· Reserved local multiple cast group address

-224.0.0.0-224.0.0.255

-TTL=1 When sending out telegraph
-For example

224.0.0.1 All systems in the sub-net

224.0.0.2 All routers in the sub-net

224.0.0.4  DVMRP router

224.0.0.5  OSPF router

224.0.0.13 PIMv2 router 

· Administrative scoped addressees
-239.0.0.0-239.255.255.255

-Private address space

· Like the single broadcast address of RFC1918 

· Can not be used in Internet transmission 

· Used for multiple cast broadcast in limited space. 
Except the above mentioned addresses of special meaning, you can use other addresses. For example:

Multiple cast IP: 235.8.8.36
Multiple cast PORT: 3666.

5.3.5.4 PPPoE

PPPoE interface is shown as in Figure 5‑13. 
Input “PPPoE name” and “PPPoE password” you get from your ISP (Internet service provider). 

Click save button, you need to restart to activate your configuration. 

After rebooting, IP camera will connect to internet automatically. The IP in the PPPoE is the DVR dynamic value. You can access this IP to visit the unit. 
[image: image88.png]User Name

Password
IP Address

username





Figure 5‑13
5.3.5.5 NTP Setup 

You need to install SNTP server (Such as Absolute Time Server) in your PC first. In Windows XP OS, you can use command “net start w32time” to boot up NTP service.
NTP setup interface is shown as in Figure 5‑14.
· Host IP: Input your PC address.

· Port:  This series DVR supports TCP transmission only. Port default value is 123.

· Update interval: minimum value is 15(Unit: minute)
· Time zone: select your corresponding time zone here.
Here is a sheet for your time zone setup.

	City /Region Name
	Time Zone

	London
	GMT+0

	Berlin
	GMT+1

	Cairo
	GMT+2

	Moscow
	GMT+3

	New Deli
	GMT+5

	Bangkok
	GMT+7

	Beijing (Hong Kong)
	GMT+8

	Tokyo
	GMT+9

	Sydney
	GMT+10

	Hawaii
	GMT-10

	Alaska
	GMT-9

	Pacific Time(P.T)
	GMT-8

	American  Mountain Time(M.T)
	GMT-7

	American Central Time(C.T)
	GMT-6

	American Eastern Time(E.T)
	GMT-5

	Atlantic Time 
	GMT-4

	Brazil
	GMT-3

	Middle Atlantic Time 
	GMT-2


[image: image89.png]HostlP 0 .0 .0 .0
Port 1024

TimeZone 12 N
Update Period [10 min.





Figure 5‑14
5.3.5.6 Email Setup

Email setup interface is shown as in Figure 5‑15. Here you can set email server information.
Note: 

You need to get the email address from your email service provider first.
Please use semicolon to separate the addresses. 
[image: image90.png]IP Address
Port

User Name
Password
Sender
Title
Receiver

0 .0

0 .0

25

Name

dvr@163.com

DVREmail

Effective Time (] [00 :00

24 :00

[*) (00 :00

24 :00





Figure 5‑15
5.3.5.7 DDNS Setup 

DDNS setup interface is shown as in Figure 5‑16.

You need a PC of fixed IP in the internet and there is the DDNS software running in this PC. In other words, this PC is a DNS (domain name server).

In network DDNS, input your PPPoE name you get from you IPS and server IP (PC with DDNS ) . Click save button and then reboot system.

Click save button, system prompts for rebooting to get all setup activated.

After rebooting, open IE and input as below: 

http：//(DDNS server IP)/(virtual directory name)/webtest.htm 

e.g.: http：//10.6.2.85/DVR _DDNS/webtest.htm.) 

Now you can open DDNSServer web search page.
[image: image91.png]DDNS Type  [CN99 DDNS |~| I Enable
Server IP 0 .0 .0 .0
| Port 0
Domain Name

| HostName

Password





Figure 5‑16
5.3.5.8 Alarm Server 
You can set alarm in accordance with different alarm protocols. System can inform the alarm server when alarm occurs. See Figure 5‑17.
[image: image92.png]Protocol Type

Server IP
Port

Private 7

10 .1 .0 .2

0





Figure 5‑17
5.3.5.9 FTP
You need to download or buy FTP service tool (such as Ser-U FTP SERVER) to establish FTP service.

Please install Ser-U FTP SERVER first. From “start” -> “program” -> Serv-U FTP Server -> Serv-U Administator. Now you can set user password and FTP folder. Please note you need to grant write right to FTP upload user. See Figure 5‑18.
[image: image93.png]Local Server
View Window Help

L serv-
Fie Edt

User

WXBo n@ i=eeal?

50 Senl Servers
58 << LocalServer>>
° License
21 Settings
g Aty
=48 Domain
= shonghy
21 Settings

@ aciviy

€ soommn] 1 Ganera (B Ot pocess |TB o hoomss| 4 ULADL Pt

DDVR WD TRl

¥ wiite
¥ Append
¥ Delete

Ditectores:
¥ List
¥ Create

¥ Remove

rectres
% Inkeit

Add Delte Edt

i Apely

3 Restore

<< Local Server >> [System Admiistrator]

| Down: 0.000 kBps / Up: 0.000 kBps | 3 of 32767 Sockets |0 (0) Users |0fers

=lalx]





Figure 5‑18
You can use a PC or FTP login tool to test setup is right or not.

For example, you can login user ZHY to FTP://10.10.7.7  and then test it can modify or delete folder or not.  See Figure 5‑19.
[image: image94.jpg]Internet Explorer

9 oottt o e o

FlPserver:  10.10.7.7
User name;
password:

After youlog on, you can add tis server to your Favories and return to t easiy.

[Jog on anonymousty





Figure 5‑19
System also supports upload multiple DVRs to one FTP server. You can create multiple folders under this FTP. 

In Figure 5‑9, select FTP and then double click mouse. You can see the following interface. See Figure 5‑20.
[image: image95.png]Type Record FTP |~/

Server IP 0 .0 .0 .0 Port (1024

Alternate IP 0 .0 .0 .0

User Name

Password [J Anonymous

Remote Directory File Length [0

Upload Time  [[J [00 :00  -24 :00 [ [oo:00 24 :00

Channel 1 9

Weekday Fri v Alarm Motion General
Time Period 1 (00 :00  -24 :00 0O 0O O
Time Period 2 (00 :00  -24 :00 0O 0O O





Figure 5‑20
Please highlight the icon [image: image96.jpg]


 in front of Enable to activate FTP function.
Now FTP can upload alarm video and motion detection video. Please note, when you are using this function, please make sure current upload channel is in motion detection or alarm record status and there is video available. 
Here you can input FTP server address, port and etc. 
[image: image97.jpg]vosTe
oRnaE DR

T





Figure 5‑21
· File length: upload file length. When setup is larger than the actual file length, system will upload the whole file. When setup here is smaller than the actual file length, system only uploads the set length and auto ignore the left section.

· When interval value is 0, system uploads all corresponding files. 

· Period 1 and period 2: you can set two periods for one each channel.

System file name is shown as in 
Figure 5‑22.

[image: image98.jpg]By artoom: s 142800

Type Channel Time.





Figure 5‑22
5.3.6 Alarm 
Please refer to chapter 4.6 Alarm Setup and Activation.
5.3.7 Detect

Please refer to chapter 4.5 Detect.
5.3.8 Pan/Tilt/Zoom

The pan/tilt/zoom setup includes the following items. Please select channel first. See Figure 5‑23.

· Protocol: select corresponding PTZ protocol such as PELCOD.

· Address: input corresponding PTZ address.

· Baud rate: select baud rate.

· Data bit: select data bit.

· Stop bit: select stop bit.

· Parity: there are three choices: none/odd/even. 

After completing all the setups please click save button, system goes back to the previous menu.
For detailed setup, please refer to chapter 4.9 preset/patrol/pattern/scan.
[image: image99.png]Channel |1 7
Protocol |PELCOD 7
Address 1 |
Baudrate (115200 7
Data Bits (8 7
Stop Bits [1 9
Parity None -

( Copy ) Pase |( Defaut ) ( Save [ Cancel |





Figure 5‑23 
5.3.9 Display

Display setup interface is shown as below. See Figure 5‑24.

· Transparency: Here is for you to adjust transparency. The value ranges from 128 to 255.
· Channel name: Here is for you to modify channel name. Please note all your modification here only applies to DVR local end. You need to open web or client end to refresh channel name.
· Time display: You can select to display time or not when system is playback. 

· Channel display: You can select to channel name or not when system is playback. 
· Overlay information: System displays some information in the screen for your reference.

· Display mode: you can select from the dropdown list: self-adaptive/VGA/TV.

· Enable tour: activate tour function.

· Interval: Input proper interval value here. The value ranges from 5-200 seconds. In tour process, you can use mouse or click Shift to turn on window switch function. [image: image100.png]


 Stands for opening switch function, [image: image101.jpg]


 stands for closing switch function.
· Motion tour type: System support 1/8 window tour.
· Alarm tour type: System support 1/8 window tour. 

Please highlight icon [image: image102.jpg]


 to select the corresponding function.

After completing all the setups please click save button, system goes back to the previous menu.
[image: image103.png]GUI

Transparency 200 Channel Name
Time Display | ] Channel Display

Qverlay Info | ]

Display Mode Self-adaptive | ~| Resolution 1024x768 9
Enable Tour ] Interval 5 sec.
View 1

View 4 a

Motion Tour Type |View 1 ~| Alarm Tour Type |View 1 9





Figure 5‑24 
In Figure 5‑24, click modify button after channel. You can see an interface is shown as in Figure 5‑25. Please note all your modification here applies to local end only. You need to refresh web or client-end to get the latest channel name. System max support 25-digital character. 
[image: image104.png]M Channel Name

CAM 1 CH1 CAM 2 CH2
CAM 3 CH3 CAM 4 CH4

Default Save Cancel





Figure 5‑25
In tour mode, you can see the following interface. On the right corner, right click mouse or click shift button, you can control the tour. There are two icons: [image: image105.png]


 stands for enabling window switch and [image: image106.png]


 stands for enabling window function. See Figure 5‑26.
[image: image107.png]



Figure 5‑26
5.3.10 Default 

Click default icon, system pops up a dialogue box. You can highlight [image: image108.jpg]


 to restore default factory setup. See Figure 5‑27.

· Select all

· General 

· Encode

· Schedule

· RS232

· Network

· Alarm

· Detect

· Pan/tilt/zoom

· Display

· Channel name
Please highlight icon [image: image109.jpg]


 to select the corresponding function.

After all the setups please click save button, system goes back to the previous menu.
Warning!

System menu color, language, time display mode, video format, IP address, user account will not maintain previous setup after default operation!
[image: image110.emf]


Figure 5‑27 

5.4  Search 
Please refer to chapter 4.3 Search.
5.5 Advanced 

Double click advanced icon in the main window, the interface is shown as below. See Figure 5‑28. There are total seven function keys: HDD management, alarm output, abnormity, manual record, account, auto maintain, and TV adjust.
[image: image111.jpg]HDD MANAGEMENT

MANUAL RECORD

TV ADJUST

ABNORMITY

&

ACCOUNT

S

ALARM OUTPUT

%

AUTO MAINTENANCE





Figure 5‑28 
5.5.1 HDD Management 

Here is for you to view and implement hard disk management. See Figure 5‑29.
You can set proper mode for each hard disk from the dropdown list.

When you use redundant backup function, you can set one or more redundant HDD(s).

Please note you need to set at least one read-write disk, otherwise system will not record video. For detailed information you can refer to chapter 4.4 Schedule.
After all the setups please click save button, system needs to reboot to get all the modification activated.
[image: image112.jpg]SATA

HDD No.

Type
Status
Capacity

Record time00-00-00 00:00:00 / 00-00R8SVEr

HDD MANAGEMENT

2 | Alarm Set

joR el | Alarm Release

1 [+] setto [ReadWrite _ Execute |
Read/Write

Read/Write Read only

Normal Redundant

79.99 GB o





Figure 5‑29 
Click alarm set button, the interface is shown as below. See Figure 5‑30(This interface is just like the abnormity setup).
Please highlight icon [image: image113.jpg]


 to select the corresponding function.

You can enable one or more alarm setups. The lower limit ranges from 1% to 99%. Alarm channel number ranges from 1 to 6. Delay value is from 0 to 240 seconds.

Please note when HDD capacity is not full system only alarms once!

After all the setups please click OK button, system goes back to the previous menu
[image: image114.jpg]Event Type
Enable

W Alarm Out
B Show Message

[NoDisk ||
]
| [B[E)]

Latch

(JSend Email

(

Save

[10 Jsec.

) [ Cancel

== ABNORMITY

)





Figure 5‑30 
5.5.2 Abnormity 
Abnormity interface is shown as in Figure 5‑31.

· Event type: There are several options for you such as disk error, no disk and etc. 

· Alarm output: alarm activation output port (multiple choices), among which is controllable 12V output.

· Latch: here you can set corresponding delaying time. The value ranges from 10s-300s. System automatically delays specified seconds in turning off alarm and activated output after external alarm cancelled.
· Show message: system can pop up the message in the local screen to alert you when alarm occurs. 

· Send email: System can send out email to alert you when alarm occurs. 

[image: image115.jpg]Event Type
Enable

W Alarm Out
B Show Message

[NoDisk ||
]
| [B[E)]

Latch

(JSend Email

(

Save

[10 Jsec.

) [ Cancel

== ABNORMITY

)





Figure 5‑31
5.5.3 Alarm Output

Here is for you to set proper alarm output.
Please highlight icon [image: image116.jpg]


 to select the corresponding alarm output.

After all the setups please click OK button, system goes back to the previous menu. See Figure 5‑32.

[image: image117.jpg]1 OUTPUT

Alarm Type Al 123

Schedule O eee
| Manual O 000
Stop O 000
Status 000

(oK ) (Cancel )





Figure 5‑32 

5.5.4 Manual Record 
Please refer to chapter 4.2.2 manual record.

5.5.5 Account

Here is for you to implement account management.  See Figure 5‑33. Here you can:

· Add new user

· Modify user

· Add group 

· Modify group

· Modify password.
For account management please note:

· System account adopts two-level management: group and user. No limit to group or user amount. 

· For group or user management, there are two levels: admin and user.

· The user name and group name can consist of eight bytes. One name can only be used once. There are four default users: admin/888888/666666 and hidden user “default”. Except user 6666, other users have administrator right.

· Hidden user “default” is for system interior use only and can not be deleted. When there is no login user, hidden user “default” automatically login. You can set some rights such as monitor for this user so that you can view some channel view without login.

· One user should belong to one group. User right can not exceed group right.

· About reusable function: this function allows multiple users use the same account to login.
After all the setups please click save button, system goes back to the previous menu.

[image: image118.emf]


Figure 5‑33 

5.5.6 Auto Maintain

Here you can set auto-reboot time and auto-delete old files setup. See Figure 5‑34.

You can select proper setup from dropdown list.

After all the setups please click save button, system goes back to the previous menu.
[image: image119.png]Auto-Reboot System

Never

~| at [00:00 '

Auto-Delete Old Files

Never





Figure 5‑34
5.5.7 TV Adjust

Here is for you to adjust TV output setup. See Figure 5‑35.

Please drag slide bar to adjust each item.

After all the setups please click OK button, system goes back to the previous menu.

[image: image120.emf]


Figure 5‑35
5.6 Information 

Here is for you to view system information. There are total five items: HDD (hard disk information), BPS (data stream statistics), Log and version, and online user. See Figure 5‑36.

[image: image121.emf]


Figure 5‑36
5.6.1 HDD Information

Here is to list hard disk type, total space, free space, video start time and status. See Figure 5‑37.

Note: 

Please remove the broken hard disk before you add a new one.

Once there is a hard disk confliction, please check hard disk time and system time is the same or not. Please go to setting then general to modify system time.  At last, reboot the system to solve this problem. 
If disk is damaged, system shows as “?”
[image: image122.jpg]2 Type Total Space Free Space Status |
Al - 79.99 GB 79.99 GB -
1 | ReadWite 79.99 GB 79.99 GB Normal

*tPage Up 14 Page Down





Figure 5‑37 

5.6.2 BPS

Here is for you to view current video data stream (KB/s) and occupied hard disk storage (MB/h). See Figure 5‑38.
[image: image123.png]Channel Kb/S MB/H Wave

1 12273 4743 £l f
2 12273 4743 £l f
3 12273 4743 £l f
4 12273 443 ) £





Figure 5‑38
5.6.3 Log 

Here is for you to view system log file. System lists the following information. See Figure 5‑39. System max supports 1024 log files.
Log types include system operation, configuration operation, data management, alarm event, record operation, log clear and etc. 

Pleased select start time and end time, then click search button. You can view the log files. Please page up/down button to view if there are more than ten files. 
[image: image124.png]Type All 9

Start Time 2008 - 10 - 28 00: 00 : 00

End Time 2008 - 10-29 00: 00 : 00
22 Log Time Event

14 |08-10-28 10:44:44 Reboot with Flag [0x01]

15 |08-10-28 10:45:00 <default>User Logout

16 |08-10-28 10:45:00 <888888>User Login

17 |08-10-28 14:31:26 <default>User Login

18 |08-10-28 14:31:27 Save PAN/TILT/ZOOM Config!
19 |08-10-28 14:31:27 Shut down at [08-10-28 10:45:26] m
20 |08-10-28 14:31:27 Reboot with Flag [0x01]
21 |08-10-28 14:31:37 <default>User Logout
22 |08-10-28 14:31:37 <888888>User Login

M Page Up !4 Page Down





Figure 5‑39 

5.6.4 Version 

Here is for you to view some version information. See Figure 5‑40.
· Channel

· Alarm in 

· Alarm out

· System version:

· Build Date
[image: image125.jpg]Channels 4

Alarm In 4

Alarm Out 3

System  2.400.0000.0
Build Date 2008-01-23





Figure 5‑40 

5.6.5 Online Users 

Here is for you manage online users. See Figure 5‑41.

You can disconnect one user or block one user if you have proper system right.
[image: image126.jpg]User Name P
Jw 10.6.2.37
JW 10.6.2.37
Jw 10.6.2.37
Jw 10.6.2.37
Jw 10.6.2.37
Jw 10.6.2.37
W 10.6.2.37
W 10.6.2.37
JW 10.6.2.37
JW 10.6.2.37
Disconnect | Blockfor | 160 sec.





Figure 5‑41 

5.7 Backup

Please refer to chapter 4.7.2.

5.8 Exit

Double click exit button, system pop up a dialogue box for you to select. See Figure 5‑42.

· Logout menu user: log out menu. You need to input password when you login the next time.

· Restart application: reboot DVR.
· Shutdown: system shuts down and turns off power.

·  Restart system: system begins rebooting.

· Switch user: you can use another account to log in.
[image: image127.png]Logout menu user

Logout menu user u
afidRestart application
Shutdown

Restart system
Switch user

#°Logout the OSD menu or resetrestart the system. After idle for
standby time (no inputs), the system will exit OSD automatically.





Figure 5‑42 
6 About Auxiliary Menu 
6.1 Go to Pan/Tilt/Zoom Menu

In the single window surveillance screen, right click mouse (or click AUX key in the remote control). The interface is shown as below: See Figure 6‑1.
[image: image128.jpg]View 1 »
View 4

Pan/TiltZoom
Color Setting

Search
CD-ROM Play
Record

REALTIME BACKUP
Alarm Output

Alarm Input

Main Menu





Figure 6‑1
Click Pan/Tilt/Zoom, the interface is shown as in Figure 6‑2.

Here you can set the following items:

· Zoom 

· Focus 

· Iris

Click icon [image: image129.bmp] and [image: image130.bmp] to adjust zoom, focus and Iris.

[image: image131.jpg]Page Switch





Figure 6‑2
In Figure 6‑2, please click direction arrows (See Figure 6‑3 ) to adjust PTZ position. There are totally eight direction arrows. (Please note there are only four direction arrows in DVR front panel.)


Figure 6‑3 

6.1.1 3D Intelligent Positioning Key

In the middle of the eight direction arrows, there is a 3D intelligent positioning key. See Figure 6‑4 . Click this button, system goes back to the single screen mode. Drag the mouse in the screen to adjust section size. 

Figure 6‑4 

Here is a sheet for you reference.
	Name
	Key 
	Function 
	Shortcut Key 
	Key 
	Function 
	Shortcut Key

	Zoom
	[image: image132.bmp]
	Near 
	►
	[image: image133.bmp]
	Far
	(

	Focus
	[image: image134.bmp]
	Near 
	(
	[image: image135.bmp]
	Far
	►

	Iris
	[image: image136.bmp]
	Close
	((
	[image: image137.bmp]
	Open 
	((


6.2 Preset  /Patrol / Pattern /Border  Function 

In Figure 6‑2 click the set button. The interface is shown as below:

Here you can set the following items:

· Preset

· Patrol

· Pattern

· Border

[image: image138.jpg]PAN/TILT/ZOOM

Function Preset [ | 3

Preset

Patrol No.
Patrol b E]
Pattern Set

Border Del Preset





Figure 6‑5  

In Figure 6‑2, click page switch button, you can see an interface as in Figure 6‑6.

Here you can activate the following functions:

· Preset

· Tour(Patrol)
· Pattern

· Auto scan

· Auto pan

· Flip

· Page Switch
[image: image139.png]No. 0 (

Preset

Tour

AutoScan AutoPan

Flip

Page Switch





Figure 6‑6 

6.2.1 Preset Setup

Note: The following setups are usually operated in the Figure 6‑2, Figure 6‑5 and Figure 6‑6.

In Figure 6‑2, use eight direction arrows to adjust camera to the proper position.

In Figure 6‑5, click preset button and input preset number. The interface is shown as in Figure 6‑7.
Add this preset to one patrol number 

[image: image140.jpg]Function Preset D

Eiezel Patrol No.

0
Patrol D
Pattern Set

Border





Figure 6‑7 

6.2.2 Activate Preset 

In Figure 6‑6 please input preset number in the No. blank, and click preset button.

6.2.3 Patrol Setup

In Figure 6‑5, click patrol button. The interface is shown as in Figure 6‑8.

Input preset number and then add this preset to one patrol.

[image: image141.jpg]Function Preset E]
Preset

b PatrolNo. [0 |
Pattern
Border





Figure 6‑8 

6.2.4 Activate Patrol

In Figure 6‑6, input patrol number in the No. blank and click patrol button 

6.2.5 Pattern Setup

In Figure 6‑5, click pattern button and then click begin button. The interface shows like Figure 6‑9.

Please go to Figure 6‑2 to modify zoom, focus, and iris.  Go back to Figure 6‑9 and click end button. 

You can memorize all these setups as pattern 1.

[image: image142.jpg]Function _Pattern D Ly
Preset
e PatrolNo. [0 |

Pattern Begin
s





Figure 6‑9 

6.2.6 Activate Pattern Function 

In Figure 6‑6 input mode value in the No. blank, and click pattern button.

6.2.7 Border Setup

In Figure 6‑5, click border button. The interface is shown as in Figure 6‑10.

Please go to Figure 6‑2, use direction arrows to select camera left limit, and then please go to Figure 6‑10 and click left limit button 

Repeat the above proceduress  rmation:ng information:orage































































































 to set right limit.

[image: image143.jpg]ILT/ZOOM

Function Pattern
Preset
Patrol No.
Patrol E A

Pattern Left

Border





Figure 6‑10 

6.2.8 Activate Border Function 

In Figure 6‑6, click auto scan button, the system begins auto scan. Correspondingly, the auto scan button changes to stop button.

Click stop button to terminate scan operation.
6.2.9 Flip
In Figure 6‑6, click page switch button, you can see an interface is shown as below. See Figure 6‑11. Here you can set auxiliary function. 
Click page switch button again, system goes back to Figure 6‑2.
[image: image144.png]Direct Aux Oper

Light ~] (on) (of)

Aux Num Oper

i)





Figure 6‑11
7 WEB CLINET OPERATION
Please note, all the operation here in chapter seven is based on our 4-ch DVR.
There might be slightly difference in the interface due to different series.
7.1 Network Connection 

Before web client operation, please check the following items:

· Network connection is right

· DVR and PC network setup is right. Please refer to general-system setting-network setup

· Use order ping ***.***.***.***(* DVR IP address) to check connection is OK or not. Usually the return TTL value should be less than 255.

7.2 Login and logout

Open IE and input DVR address in the address column. For example, if your DVR IP is 10.10.3.16, then please input http:// 10.10.3.16 in IE address column. See Figure 7‑1
[image: image145.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 7‑1
System pops up warning information to ask you whether install webrec.cab control or not. Please click yes button.
If you can’t download the ActiveX file, please modify your settings as follows. See 
.Figure 7‑2
[image: image146.png]Internet Options

General | Security |Privacy || Content | Connections | Programs | Advanced |

Select a zone to view or change security sektings.

@ ) v e

oot oca Tustedstes ¥
< ] 8
eret =
‘This zone is for Internet websites, =5
ks e ratios o
e

Security level for tis zone
Allowed levels For this zone: Medum ta Hih

Medium-high
- Appropriate for most websites

- Prompts before dowrloading potentialy unsafe
cantent.

~Unsigned Actve controls il not be downloaded

Custom level Default fevel

Reset all zones to defaul evel

m





Figure 7‑2
After installation, the interface is shown as below. See Figure 7‑3.
Please input your user name and password. 
Default factory name is admin and password is admin. 

Note: For security reasons, please modify your password after you first login.
[image: image147.png]WEBservice - Windows Internet Explorer (x]

OO - [ %] (x] [ g
& & @i = i@ @ - [ page - 3 Took - @+

bone T [ | | (g @ e | wnon -





Figure 7‑3
After you logged in, you can see the main window.  See 
Figure 7‑6
. 
This main window can be divided into the following sections. 

· Section 1: there are five function buttons: configuration (chapter 7.3), search (chapter 7.4), alarm (chapter 7.5), about (chapter 7.6), log out (chapter 7.7). 

· Section 2: there are channel number and five function buttons: open all/start dialog/refresh/ local play/burning.
· Section3: there are PTZ (chapter 7.2.2), color (chapter 7.2.3) button and you can also select picture path and record path. 

· Section 4:real-time monitor window. Please note current preview window is circled by a green rectangle zone.
· Section 5: Here you can view window switch button.  You can also select video priority between fluency or real-time.

· System monitor window switch supports full screen/1-window/4-window/6-window/8-window/9-window/13-window/16-window/20-window/25-window/36-window. See Figure 7‑4.

[image: image148.png]BEOEPEEEEEE@®E




Figure 7‑4
· Preview window switch. System support 1/4/8/9/16-window real-time preview. Please you need to have the proper rights to implement preview operation. You can not preview if you have no right to preview the either channel. See Figure 7‑5.

[image: image149.png](=05::00-- 08 -0 |





Figure 7‑5

[image: image150.png]WERB senics SEARCH ALARM CONFIG ABouT LoGouT

B0 X

860 X

1500 X

1800 X

DEEOEPEEEEEHEZ=E =R

ConvRiaht 2008 Al Rights Reserved.





Figure 7‑6
7.2.1 Real-time Monitor
 In section 2, left click the channel name you want to view, you can see the corresponding video in current window. 

For detailed function key information, please refer to Figure 7‑7. 

[image: image151.png]Ea a0 X





Figure 7‑7
· 1: Digital zoom: Click this button and then left drag the mouse in the zone to zoom in. right click mouse system restores original status. 

· 2: Change show mode: resize or switch to full screen mode.

· 3: Local record. When you click local record button, the system begins recording and this button becomes highlighted. You can go to system -.local record to set video file path.

· 4: Capture picture. You can snapshoot important video. All images are memorized in system client folder \download\picture (default).

· 5: Audio :Turn on or off audio.(It has no relationship with system audio setup )

· 6: Close video.

Please refer to Figure 7‑8  for main stream and extra stream switch information. 
[image: image152.png]A cam1

Main Stream
Extra Stream

3 cam 2
9 cam3
3 cam 4

Open All || Refresh
StartDialog| ¥

Local Play/| Burning




Figure 7‑8
Open all

You can click this button to enable four channels at the same time.

 Refresh

You can use button to refresh camera list. 

Start Dialogue 

You can click this button to enable audio talk. There are three options: Default/PCM/G711a. See Figure 7‑9.
[image: image153.png]Open Al || Refresh

Star\D\a\DQ"





Figure 7‑9
Local Play
Click local play button, system pops up the following interface for you to select local play file. See Figure 7‑10.
[image: image154.jpg]Open

lookin [D B o]

@y Documents S orton Anins
@y Computer G Thinkvantage Technologies
@y NotvorkPaces 58 ircless Manager
Blccece o0 2008_04_08
HroLvoecicktosot 1

Bearthlink Internet 30 Days Free £ CCF090+2008_00000

&1 m ] B)

Flepame: [ Gpen
Files of type: | Record files 7] & Cancel





Figure 7‑10
7.2.2 Burning 
In Figure 7‑8, click burning button. You can see an interface is shown as in Figure 7‑11.

Click start searching button, you can view current CD information such as total space, free space and current status. 

Please select the CD and then input password and set title. 

Then you can select record channel, system max support 2 channels at the same time. 

Finally, you can click start burning button. 
Start searching: just like the detect button in DVR main menu,. System can display available CD and corresponding status. 

· Preview: you can input relative information here. When playback, system can display information on player right side. 

· Password. You can set playback password here. Please input password first and then click set button. After you enabled this function, you need to input password to begin playback each time. 

· Channel number: if you select one channel, then system begins ordinary burning. If you have enabled two channels, then system begins PIP burning.

· Burning type: please note web end supports synchronize burning only. 

· Mark: it is a index in the recorded file. You can use this function to search file and locate to the corresponding time. When playback, please right click menu to view mark information. Double click mouse you can go to the corresponding time to play.
· Customized overlay: during the burning, you can overlay customized information. When you playback, you can see customized information is on the top left corner. Each time, the overlay information lasts for five seconds. Usually you can use mark and overlay function together. 

Before you enable burning, system still check current CD status. 

There are following situations for you selected CD:

If there is backup CD, system prompts there is backup in process, system can not perform the burning at the same time. 

If there is a burning in progress, you need to stop current burning first.

If there are all idle CD. The types include: erasable (new/old) and non-erasable (new). System can begin burning. 

During the burning process, you can view corresponding burning information such as begin time, end time, total capacity, free capacity.
During the burning process, you can use mark and overlay function to symbolize the important position in the video. After completed the burning, you can use mark function to search desired position and view overlay to understand the video more clearly.

If you want to stop burning during the process, you can click stop burning button. 
[image: image155.png]Search Available Burning Davice Previen

Wans Type | Total Space Trpe Tairy <] Title
fdefs0 IR 437 GB
Dlfdevisel oW 437 G

Briaf Record Mo,

20061110 o 15:00:18

Date

Tnguisitor Tnguiry Flace

| Interpreter Inquiry Dept.

Start Searching| Recordist Seeord T,
Pazsword
Pazsword [ererer File Mo, Record Content

set e

[V Chennel 1 [V Chamnel 2 |~ Chemnel 3 | Chemnel 4

Start Burning | Stop Burning
Burning Frogress

I: Burning 0 Begin Tine 0 Elapsed Tine 00:00:00
Total Disk Capacity Renanent Disk Capacity0 Burned Disk Capacity 0
Buning State 0

Mark Castonized Overlay





Figure 7‑11
7.2.3 PTZ 

Before PTZ operation, please make sure you have properly set PTZ protocol. (Please refer to chapter 7.3.2 Setting-> Pan/Tilt/Zoom).

Click PTZ button, the interface is shown as in Figure 7‑12 .

[image: image156.png]P O ¢

< >

Al (v) (4
Spesd(1-8) 5

+) zoom (-

+) Focus

+ s~





Figure 7‑12 

7.2.3.1 Direction key and 3D positioning key

In Figure 7‑12, there are eight direction keys.

In the middle of the eight direction keys, there is a 3D intelligent positioning key. 
Click SIT button, system goes back to the single screen mode. Drag the mouse in the screen to adjust section size. It can realize PTZ automatically.

7.2.3.2 Speed 

System supports eight-level speed. You can select from the dropdown list. Speed 2 is faster than speed 1.

7.2.3.3 Zoom/Focus/Iris

Here is a sheet for you reference.

	Name 
	Function

key 
	Function
	Function

 key
	Function

	Zoom
	[image: image157.jpg]



	Near 
	[image: image158.jpg]



	Far 

	Focus
	[image: image159.jpg]



	Near 
	[image: image160.jpg]



	Far 

	Iris
	[image: image161.jpg]



	close 
	[image: image162.jpg]



	Open 


Then click triangle icon in Figure 7‑12, you can see the following interface. See Figure 7‑13.

[image: image163.png]P O ¢
< >
Al (v) (4

Spesd(1-8) 5

+) zoom (-
+) Focus

+oms -~

No.(1-127): 1

Preset | Auto-Tour
Auto Pan || Auto-Scan
Patiern | AUX Open
AUX Close|| PTZ et





Figure 7‑13
In Figure 7‑13, click PTZ setup button you can see the following interface. See Figure 7‑14.

[image: image164.png]Right Linit

Delats Grouwp

[ Pattern

0

Stop Record

[ Assistant

foLc





Figure 7‑14
7.2.3.4 Auto Scan 

In Figure 7‑14, move the camera to you desired location and then click left limit button. 

Then move the camera again and then click right limit button to set a right limit. 

7.2.3.5 Pattern 

In Figure 7‑14, you can input pattern value and then click start record button to begin PTZ movement. Please go back to Figure 7‑13 to implement camera operation. Then you can click stop record button. Now you have set one pattern.
7.2.3.6 Preset 

In Figure 7‑14, move the camera to your desired location and then input preset value. Click add button, you have set one preset.

7.2.3.7 Auto tour 

In Figure 7‑14, input auto tour value and preset value. Click add button, you have added one preset in the tour.

Repeat the above procedures you can add more presets in one tour.

7.2.3.8 Assistant
You can select the assistant item from the dropdown list. See Figure 7‑15.
[image: image165.png]Left Linit | Right Linit

[ Bresat
f A Delete
[ hute-Tour
f A Delete Delete Grouwp
[ Pattern

f Start Record | Stop Record Delete

[ Assistant





Figure 7‑15
7.2.4 Color 

Click color button in section 3, the interface is shown as Figure 7‑16. 

Here you can select one channel and then adjust its brightness, contrast, hue and saturation. (Current channel border becomes green). 

Or you can click default button to use system default setup.

[image: image166.png]Color

Foset

More





Figure 7‑16 

7.2.5 Picture Path and Record Path 
Click more button in Figure 7‑16, you can see an interface is shown as in Figure 7‑17.

[image: image167.png]



Figure 7‑17
Click picture path button, you can see an interface is shown as in Figure 7‑18.

[image: image168.png]Path  [F\Picturelowalosd Choose





Figure 7‑18
Click record path button, you can see an interface is shown as in Figure 7‑19.

[image: image169.png]Path £ Recor@omalead Choose





Figure 7‑19
7.2.6 Menu Interface Switch

Put your mouse on the PTZ control bar until you see the following icon. See Figure 7‑20.

[image: image170.png]



Figure 7‑20
Left click your mouse and then drag it to the channel control status bar. You can see the two menus interface switched. See Figure 7‑21. You can compare the following interface with 
Figure 7‑6
.
[image: image171.png]WEB senvics SEARCH ALaRM conFiG AgouT Locour

B0 X
D& X D& X
DEEOEPEEEEEHEZ=E =R

ConvRiaht 2008 All Riahts Reserved.




Figure 7‑21
7.3 Configure

In the main window, click config button, you can see an interface is shown as in Figure 7‑22.

[image: image172.png]Configuration

] Control Penl
= @ stsmn TiFo
2 vERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
ETHORK
3
DeTECT
PAN/TILT/ZO0N
DEFAILL/BACKIE
= G apvaED
HOD NARAGENERT
M 1/0
RECORD CONTROL
accomn
AITO REBOOT
=) (1 AIDTIONAL FUNCTION
sm/pos
=

VERSION

Tten

Stauts

El

Video Tn/Out
Audio Tn/Out
Aarn Tn/0ut
Ethernet Port
RS2,
Parallel Port
Bios Version

YA 10450305AE20008
41

2/1

43

1

1

0

2.606.0000. 0, Buil4:2006-11-10





Figure 7‑22
7.3.1 System Information

Click system information button, you can see the following interface. See Figure 7‑23.
Here you can view:

· Version

· HDD information

· Log 
[image: image173.png]Configuration

S Contral Panet

(3 VERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
ETHORK
3
DeTECT
PAN/TILT/ZO0N
DERALT/BACKUE
= G apvaED
HID MAAGEENT
M 1/0
RECORD CONTROL,
sccowr
AUTO REBO0T
= (3 AUDTIONAL FUNCTION
sm/pos
=

VERSION

Tten

Stauts

El
Video Tn/Out
Audio Tn/Out
Aarn Tn/0ut
Ethernet Port
RS2,

Parallel Port
Bios Version

YA 10450305AE20008
41

2/1

43

1

1

0

2.606.0000. 0, Buil4:2006-11-10





Figure 7‑23
· Version 

Click version button, you can see corresponding HDD information and system information for your reference. See Figure 7‑24.

[image: image174.png]Configuration

] Control Panel
= & stsmn TiE

[2/ 100 10
[ 106
= I sermG

Ewone
scrmz
w2
R
ALARN
DETECT
'PAN/TILT/ZOOW
DEFAULT/BACKUP
= 8 xovmrm
HDD MANAGENENT
Rem 170
RECORD CONTROL
ACCOUNT
AUTO REBOOT
= ADDTIONAL FUNCTION
ATW/POS
s

VERSION

Tten

Stauts

El

Video Tn/Out
Audio Tn/Out
Aarn Tn/0ut
Ethernet Port
RS2,
Parallel Port
Bios Version

YA 10450305AE20008
41

2/1

43

1

1

0

2.606.0000. 0, Buil4:2006-11-10





Figure 7‑24
· HDD Information 

Here you can view HDD amount, HDD status, total volume and free space. See Figure 7‑25.

[image: image175.png]Configuration

o0 TF0

S Contral Panel

= [ stsTEn TiEo o

Free/Total Space

[ VERSTON Total
Egion o] D4 et
B e
I sertmG
caERAL
swcone
scirae
pe
wEoRK
A
DericT
FRY/TILT/Z000
DEFALT/BACIP

HID WARAGENENT
Marn 1/0
RECORD CONTROL
AccomT
AUTO REBOOT

=5 ADDTIONAL FUKCTION
AT/0S
ms

HID Status

Working

/15262018
0/15252018





Figure 7‑25
· Log 
Click log button, you can see an interface is shown as in Figure 7‑26. Here you can view current device log information. 
You can select corresponding log type from the dropdown list and then click search button to view.
[image: image176.png]Configuration

] Control Panel
= @ stsan Tivo
2 vERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
ETHORK
AL
DeTECT
PAYTILL/Z000
DEFAILL/BACKIE
= G apvaED
HOD NARAGENERT
A 1/0
RECORD CONTROL
accomn
AITO REBOOT
=) (1 AIDTIONAL FUNCTION
Amwyos
s

3

a1

e

Log Tine

Event

2006-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-03
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04

User Login: 838585
Video Loss: Channel No.: 3
User Login' 1

User Logout: 585883

User Login: default

User Logout: 1

User Login: 1

User Logout: 1

User Logout: default

User Login: 838588

User Logout: 585883

User Login: default

User Logout: default

User Login: 838588

User Login' 1

Export Config: Config Info
User Logout: 583883

User Login: default

User Logout: default

User Login: 838588

User Logout: 585883

User Login: default

User Logout: default

User Login: 838588

User Logout: 1

User Login: default

Export Config: Pan/TiLt/Toon
Devica Shut Down, Time: 2006-11-04 &





Figure 7‑26
7.3.2 Setting 

Setting includes the following items:

· General 

· Encode 
· Schedule

· RS232

· Network

· Alarm 
· Detect

· Pan/Tilt/Zoom

· Default/backup 
· General 

General interface is shown as in Figure 7‑27.

· System time: Here is for you to modify system time. Please click Save after your modification 

· Data format: Here you can select data format from the dropdown list.

· Data separator: Please select separator such as – or /.

· Time format: there are two options: 24-H and 12-H.

· HDD full: there are two options: stop recording or overwrite the previous files when HDD is full.

· Pack duration: Here you can select file size. Default setup is 60 minutes.

· Device No.: when you are using one remote control to manage multiple DVRs, you can give these DVRs serial numbers respectively. Click address button in your remote control and then input the correct device number, now you can control the DVR now.

· Video standard: PAL. (For your reference only)

[image: image177.png]Configuration
=) Contral Panel GENERAL
- @ st o Systen Tine 001110 =] [15:05:05 = _sw_| Spmerc
[ VERSTON
[ 1on 10F0
o Dete Fornat [rrrr 0 st
- iy simmme Bate Separator [
S
o
soomminz R ez
oy o pat
TR Rk Duation o
o
el Bevice Mo
PAN/TILT/Z0OM Video Standard
DEFAULT/BACKUP
= G apvaED
DD MANAGEWENT
Han 10
RECORD CONTROL
ACCOUNT
AUTO REBOOT
G aovTIons. crIOn
wecs
s

Tine Fornat  [24-HIR

Overmrite





Figure 7‑27
· Encode 

Encode setup includes the following items. See Figure 7‑28.

Here you can select 

· Channel: Here is for you to select a channel.

· Channel name: Modify channel name.

· Data stream:  Regular and extra data stream.                   

· Video/Audio. System only displays video by default. You need to manually enable audio function. 

· Bit rate: There are two options: CBR and VBR. You can only set video quality in VBR mode.

· Frame rate: The value ranges from 1f/s to 25f/s (PAL) and 1f/s to 30f/s (NTSC).

· Compression: There are two options: H.264 and MPEG 4.

· Resolution: D1/HD1/DCIF/CIF

· Quality: The value ranges from 1 to 6. The level 6 is the best video quality.       

· Color setting: you can set brightness /contrast ness /saturation/hue. The value ranges from 0 to 100. Default value is 50.
· Cover area: here you can set privacy mask zone position. System supports max 8 zones.

· Preview means current zone can not be seen in preview status. 

· Monitor means current zone can not be seen in monitor status.

· All means current zone can be seen in neither preview nor monitor.

· Time display: Here you can enable time title display function.

· Channel display: Here you can enable channel title display function.
[image: image178.png]Configuration

S Contral Panel
= [2f SYSTEN THFO
[ VERSTON
[ 1on 10F0
[Bf e
= it sermmv
cRERL
=]
5 S
2
VoK
ALY
DeTECT
PRYTILY/ Z000
DEFAILI/BACK
= % e
1D WGENENT
ALarn /0
RECORD CONTHOL
socomT
A0 RSB0
= £ ANDTIONAL FURCTION
Am/pos
s

ENCODE

Channal

Canpression
Audiof¥ideo
Resclution
Frane Rate (FES)
Bit Rate Type

Bit Rate (0/5)
Referance Bit
I Frane

Calor Setting

Overlay
Cover-ires

[ Tine Dizpley

=s

Chamnal 01

== ~]
[ Video [~ hudio
0 ~]

= -

Chennel Name  [CAM 1

Betra Strean  [Extrs Streanl v]
udiofVideo [ Video [ Audio
Resolution  [qCIF

Frane Rate (P5) [25

Bit Rate Type

Customize= 960
T68Kbp="4096Kbps
143 07148

set

Bit RateG/s) [258 <]
Reference Bit  45Kbps"258Kbps

set

[ Channel Display

e Retresh





Figure 7‑28
· Schedule 

Schedule includes the following interface. See Figure 7‑29.

When DVR boots up, it is in 24-hour continuous record. In this interface you can set record type, record time and period. Record type includes regular record(R), motion detection record (M) and alarm record (A).

· Channel: select the channel number you desire. 

· Prerecord: system can record the three to five seconds video before activating the record operation into the file.(Depends on data size)

· Period: There are six periods for you to set. Please click set button of the corresponding period. See Figure 7‑30.Please note all the setup here shall be in one day. E.g. 00.00 to 24.00.

[image: image179.png]Configuration

=) Contrel Panel

= [&f StsTen R0
[2f veRsTon
[2/ 100 TwvD
[ 106

= I sermG
5 GRNERAL
3 BiCODE

[ DETECT
(5] PAN/TILT/ZOO
{51 DEFAULT/BACKUR
= 8 xovmrm
(5] DD MANAGENENT
[ Marn I/0
[ RECORD CONTROL.
(5] ACCOUNT
{51 AUTO REBOOT
= [ ADDTIONAL FUNCTION
[ ATW/POS
=h

SCHEDVLE

Channal
Period Info

Chamnal 01 <]

I Regular
4

Prerecord I Redundancy

mm Wil
8 1





Figure 7‑29
[image: image180.png]Period 1

Period 2

Period 3

Period 4

Period 5

Period

0:00:00

0:00:00

0:00:00

0:00:00

0:00:00

0:00:00

[ M1 Wesk

-

I The

I~ hon
I





Figure 7‑30
Tip:

After you finished setup for one channel, you can click “save as” button, system pops up the following interface. See Figure 7‑31.Now you can copy one channel setup to other channels. 

[image: image181.png]



Figure 7‑31
· Network
Network interface is shown as in Figure 7‑32. 
[image: image182.png]Configuration

] Control Panel
= @ stsan Tivo
2 vERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
= 51 NETHORK.
BIATL
s
ns
e
ALABN CENTER
AL
DeTECT
PAYTILL/Z000
DEFAILL/BACKIE
= G apvaED
HOD NARAGENERT
A 1/0
RECORD CONTROL
accomn
AITO REBOOT
=) (1 AIDTIONAL FUNCTION
Amwyos
s

NETHORK

Ethernet Port
TP Address
Submet Hask
Gatensy

Device Hane
TCP Port
VP Port
I Transter

Service Type
Vser Nane

Pazsword

Ethernet Portl

I 5

[ 5
=
0

e

TP Port

BTT%8  Max Comnectio

WLTICAST

Mae Adress

o
o

Fstacfaiese

[ Highspeed domnloading

I Easble

TP Address

[z

B

Port

freess

2

2
0Bssas





Figure 7‑32
This interface includes the following items:

Max address: Here you can set max connection amount. The value ranges from 0 to 10. 0 means no network connection is allowed.
TCP port: default setup is 37777. Please note port 37778 is for network UDP port use only.

HTTP port: default setup is 80.
Transfer: here you can select the priority between fluency/video quality.
· PPPoE

In remote item, enable PPPoE function and then input “PPPoE name” and “PPPoE password” you get from your ISP (Internet service provider). See Figure 7‑33.

Click save button, you need to restart to activate your configuration. 

After rebooting, DVR will connect to internet automatically. The IP displayed in the IP address item is the dynamic value. You can use client-end software to visit current IP now.

[image: image183.png]Configuration

=) Contrel Panel

= [&f StsTen R0
[2f veRsTon
[2/ 100 TwvD
[ 106

= I sermG

o
soEnLE
2
= &3 TR
NI
ms
s
b
ALY CRITER
AL
DericT
PRYTILY/ Z000
DEFAILI/BACK
= % e
1D WGENENT
ALarn /0
RECORD CONTHOL
socomT
A0 RSB0
= £ ANDTIONAL FURCTION
Am/pos
=

NETHORK

Ethernet Port
TP Address
Submet Hask
Gatensy

Device Hane
TCP Port
VP Port
I Transter

Service Type
Vser Nane

Pazsword

Ethernet Portl

W mee

TP Port

BTT%8  Max Comnectio

WLTICAST

Mae Adress

o
o

Fstacfaiese

[ Highspeed domnloading

gy

TP Address

[z

B

Port

freess

2

2
0Bssas





Figure 7‑33
· NAS

Here you need to input host IP, user name, password, port and etc. See Figure 7‑34.

You can click set button after each date to set periods. See Figure 7‑35.

[image: image184.png]Configuration

=) Contrel Panel

= [&f StsTen R0
[2f veRsTon
[2/ 100 TwvD
[ 106

= I sermG

o
soEnLE
2
VoK
0L
s
s
e
ALY CRITER
AL
DericT
PRYTILY/ Z000
DEFAILI/BACK
= % e
1D WGENENT
ALarn /0
RECORD CONTHOL
socomT
A0 RSB0
= £ ANDTIONAL FURCTION
Am/pos
=

WS Ensble
Host TP 024 085535
o ]

Vser Nane

Pazsword

WS path
Period Info

Cmit [ 0 =] Mrcae M [T,
0 < 5 P s ) 2

——— ]

Set

e ——

Set

e





Figure 7‑34
[image: image185.png]Defanlt Tine Carrent Tine
Regilar WD Alarn

Period 1 [0:00:00 = [esseise rror

Peried2 [0 = [essese rror

[ M1 Wesk
4 [Mon [ Tee [ Wed
[T R T Sat





Figure 7‑35
· Email

Here you can input email server information. See Figure 7‑36.
Note you can get the server IP from your ISP or Ping email SMTP.

Please use semicolon to separate the email addresses.
[image: image186.png]Configuration

= Contral Panal EMAIL

= [2f SIsTEN INFO Server SWTP  [ailServer [~ Ensble
2 vERSTON

8¢ 100 T Port b5 0B
[ w6 Vser Nane

=i sErTING. Password

Sender

'ENCODE
SCHEDULE
Rs232 Ain Address 1
WETHORK Ain Address 2

S Ain Address 3

DDNS.

S

WP

ALARM CENTER
ALARI

Derect
'PAN/TILT/ZOOW
DEFAULT/BACKUP

= 8 xovmrm
HDD MANAGENENT
Alarn I/0
RECORD CONTROL
ACCOUNT
AUTO REBOOT
= ADDTIONAL FUNCTION

ATW/POS
=h

Subject.





Figure 7‑36
· NTP

In the LAN, you need to install a SNTP server in the PC. For MS XP OS, you can use net start w32time to boot up/

If you are in the internet, you can follow the above mentioned steps. But pleas note you still need a SNTP server.

In Figure 7‑37, you need to input server IP, port and update interval. You can select time zone from the dropdown list.  Please refer to the following sheet. 

	City /Region Name
	Time Zone

	London
	GMT+0

	Berlin
	GMT+1

	Cairo
	GMT+2

	Moscow
	GMT+3

	New Deli
	GMT+5

	Bangkok
	GMT+7

	Beijing (Hong Kong)
	GMT+8

	Tokyo
	GMT+9

	Sydney
	GMT+10

	Hawaii
	GMT-10

	Alaska
	GMT-9

	Pacific Time(P.T)
	GMT-8

	American  Mountain Time(M.T)
	GMT-7

	American Central Time(C.T)
	GMT-6

	American Eastern Time(E.T)
	GMT-5

	Atlantic Time 
	GMT-4

	Brazil
	GMT-3

	Middle Atlantic Time 
	GMT-2


[image: image187.png]Configuration

] Control Panel
= @ stsan Tivo
2 vERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
mETHORK
AT
s
ns
e
ALABN CENTER
AL
DeTECT
PAYTILL/Z000
DEFAILL/BACKIE
= G apvaED
HOD NARAGENERT
A 1/0
RECORD CONTROL
accomn
AITO REBOOT
=) (1 AIDTIONAL FUNCTION
Amwyos
s

gy
Host 7

Port
Tine Zone

Update Tnterval

PTEServar

iz

qi+az:00

0





Figure 7‑37
· Enable DDNS

You need a PC of fixed IP in the internet and there is the DDNS software running in this PC. In other words, this PC is a DNS (domain name server).see Figure 7‑38.

Please enable DDNS function and then input PC IP. Click save button and then reboot device. Now you can login via DDNS. Please open IE and then input http：//(DDNS server IP)/ DDNServer / default.htm. for example, input http：//10.5.2.149/DDNServer/default.htm, you open a DDNS server web page.
[image: image188.png]Configuration

] Control Panel
= @ stsan Tivo
2 vERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
mETHORK
AT
5 D0¥s.
ns
e
ALABN CENTER
AL
DeTECT
PAYTILL/Z000
DEFAILL/BACKIE
= G apvaED
HOD NARAGENERT
M 1/0
RECORD CONTROL
accomn
AITO REBOOT
=) (1 AIDTIONAL FUNCTION
Amwyos
s

s

Server Type

Server TP
Server Port
Donsin Nans
Usernane
Pasznord

Alive Tine (sec.)

o 1o

I B

16853

130000





Figure 7‑38
· Alarm Centre 
Alarm centre is shown as in Figure 7‑39.

You can click enable button to activate alarm centre function.
Then you can input host I, port and set schedule time. 

System can alert you when there is an alarm. 
[image: image189.png]Configuration

] Control Panel
= @ stsan Tivo
2 vERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
mETHORK
AT
s
ns
e
5] ALARN CENTER
AL
DeTECT
PAYTILL/Z000
DEFAILL/BACKIE
= G apvaED
HOD NARAGENERT
M 1/0
RECORD CONTROL
accomn
AITO REBOOT
=) (1 AIDTIONAL FUNCTION
Amwyos
s

¥ Easble
Host TP

Port

v fining

Dy

Hour





Figure 7‑39
· RS232

RS232 includes the following items. See Figure 7‑40.
· Function: There are various devices for you to select. Console is for serial port or min-end platform to upgrade program. Keyboard is for you to use special keyboard to control current device. 
· Baud rate: Please select as you desire.

· Data bit: The value ranges from 5 to 8.

· Stop bit: There are two options: 1/2.

· Parity: There are three options: none/odd /even.
[image: image190.png]Configuration

] Control Panel

= @ stsan Tivo
2 vERSTON
2 10 0
[# 16

= §i senmG
GENERAL
BrConE
somnLE
SE=m
METHORK
3
DeTECT
PAYTILL/Z000

DEFAILL/BACKIE

= G apvaED
HOD NARAGENERT
M 1/0
RECORD CONTROL
accomn
AITO REBOOT

=) (1 AIDTIONAL FUNCTION
Amwyos
=

Rs232 Con
Function
Data Bits
Stop Bits
Buadrate

Parity

con o1

Consele

g

2

115200

None





Figure 7‑40
· Alarm 
Please note before alarm setup, you need to properly connect alarm input and output device, send address and receive address. Click save button confirm current setup. 
Alarm setup includes the following items. See Figure 7‑41.

· Event Type: you can select event type from the drop down list: Local alarm/Net alarm.

· Alarm in: Select corresponding alarm in channel 

· Type: There are two options: normal open and normal close. 

· Record channel: select record channel when alarm occurs. Please note you need to select alarm record in DVR schedule interface and enable schedule function in manual record interface. 

· Anti-dither: system support anti-dither function. The value ranges from 0 to 15 seconds.

· Alarm latch: After alarm completes, system can still alarm for the specified time. The value ranges from 10 to 300 seconds. 

· Record latch: After motion detection completes, system can still record for the specified time. The value ranges from 10 to 300 seconds

· Email: System can send out email to alert you when alarm occurs. 
· PTZ activation: Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm. Click set button, the interface is shown as in Figure 7‑42.

· Tour: Here you can enable tour function when alarm occurs.  System supports multiple-window tour. Please go to chapter 5.3.9 Display for tour interval setup. 

[image: image191.png]Configuration

=) Contrel Panel

= [&f StsTen R0
[2f veRsTon
[2/ 100 TwvD
[ 106

= I sermG

'ENCODE
SCHEDULE
RS232
NETHORK
=l
DETECT
'PAN/TILT/ZOOW
DEFAULT/BACKUP
= 8 xovmrm
HDD MANAGENENT
Karm 10
RECORD CONTROL
ACCOUNT
AUTO REBOOT
= ADDTIONAL FUNCTION
ATW/POS
s

Event Type

Mam In

Marn Lateh

Record
Hiem

Record Latch
I wail

I~ Tour

I PIZ Activatio
o

3

Hornal Open v

Anticdither [0 sec 015

107300
4

3

107300

w Message [ Upload

4

set





Figure 7‑41
[image: image192.png]Pan/Tilt/Zoon

Chamnel  Event Type
o

o

o

0





Figure 7‑42
· Detect

Detect interface is shown as in Figure 7‑43.  

Here includes the following items:

· Channel: Select channel name from the dropdown list.
· Type: There are three types: motion detection/Video loss/Camera mask detection.

· Record channel: Here you can select record channel (Multiple choices). Please make sure you have set MD record in encode interface(Main Menu->Setting->Schedule) and schedule record in manual record interface(Main Menu->Advanced->Manual Record) 

· Period: Here is for you to set record period. Click set button, you can see an interface is shown as in Figure 7‑44. In Figure 7‑44, click time set button, you can see an interface is shown as in Figure 7‑45. Here you can set time period. 

· Sensitivity: There are six levels.  The sixth level has the highest sensitivity.

· Region: If you select motion detection type, you can click this button to set motion detection zone.  The interface is shown as in Figure 7‑46. There are PAL22X18/NTSC22X15 zones. Right click mouse you can go to full-screen display mode. Do remember clicking OK button in Figure 7‑46 to save your motion detection zone setup.

· Anti-dither: system support anti-dither function. The value ranges from 0 to 15 seconds.

· Alarm latch: After alarm completes, system can still alarm for the specified time. The value ranges from 10 to 300 seconds. 

· Record latch: After motion detection completes, system can still record for the specified time. The value ranges from 10 to 300 seconds

· Mail: system can send out email to alert you when alarm occurs.

· Tour: Here you can enable tour function when alarm occurs in corresponding channel. System supports multiple-window tour. Please go to chapter 5.3.9 Display for tour interval setup.
· PTZ activation: Click set button, you can see an interface is shown as in Figure 7‑47. System can go to corresponding preset when alarm occurs. 

[image: image193.png]Configuration

=) Contrel Panel

= [&f StsTen R0
[2f veRsTon
[2/ 100 TwvD
[ 106

= I sermG

BNCODE
SCHEDVLE
RSz
METHORK

ALARN
SR
'PAN/TILT/Z00M
DEFAULT/BACKUP

= G apvaED
DD MANAGEWENT

10
RECORD CONTROL
ACCOUNT
AUTO REBOOT

S ADDTIONAL FUNCTION
ATH/POS.

s

DETECT

Event Type
Channal
Region

Period

Nornel Out

Marn Lateh

Record
Hiem

Record Latch
I wail

I~ Tour

I PIZ Activatio
o

Wotion Detec v

Channal 01

Select

set
3

3

4

k2l

107300

3

4

107300

set

Sensitivity

Anti-dither

w Message [ Upload





Figure 7‑43
[image: image194.png]



Figure 7‑44
[image: image195.png]Period 1

Period 2

Period 3

Period 4

Period 5

Period

0:00:00

0:00:00

0:00:00

0:00:00

0:00:00

0:00:00

[ M1 Wesk

-

I The

I~ hon
I





Figure 7‑45
[image: image196.png]Notion Detect.





Figure 7‑46
[image: image197.png]Pan/Tilt/Zoon

Channal
o

o

o

0

Event Type

Preset

Preset

[toTour





Figure 7‑47
· Pan/Tilt/Zoom 

Pan/Tilt/Zoom interface is shown as in Figure 7‑48.

Please note you have properly set dome address and all connections are right.
· Channel: Select the dome connected channel.

· Protocol: select the corresponding dome protocol.(such as Pelco)

· Address: Set corresponding dome address. Default value is 1. Please note your setup here shall comply with your dome address; otherwise you can not control the speed dome.

· Baud rate: Select the dome baud rate. Default setup is 9600.

· Data bit: Default setup is 8.

· Stop bit: Default setup is 1.

· Parity: Default setup is none.
· Matrix ID: Default value is 0. The value ranges from 0 to 255.
[image: image198.png]Configuration

=) Control Panel
= [2f SYSTEN THFO
[ VERSTON
[ 1on 10F0
[Bf L6
& i Sae
GENERAL
ENCODE
SCHEDULE
RS232
WETHORK
ALARN
DETECT
= ENREED
DEFAULT/BACKUP
= G apvaED
DD MANAGEWENT
Alarn 1/0
RECORD CONTROL
ACCOUNT
AUTO REBOOT
S ADDTIONAL FUNCTION
ATH/POS.
s

PAN/TILT/Z000

Chaanal
Protozel
Address

Buadrate
Data Bits
Stop Bits
Parity

Wertix I0

Chamnal 01

Lezanc

0





Figure 7‑48
· Default/backup

 This function allows you to export or import corresponding information. See Figure 7‑49.
[image: image199.jpg]Configuration

=) Contrel Panel

= [&f Ststen v
[2F veRsTon
[27 1m0 Twv0
[ 106

= I e

ENCODE
SCHEDULE
RS232
# HETHORK
ALARN
DETECT
'PAN/TILT/Z0OM
b=t i
= G ADVAICED
D MANAGEWENT
R
RECORD CONTROL
ACCOUNT.
AUTO REBOOT
& ADDTIONAL FUNCTION
ATH/POS.
s

DEFKULT/BACKUE.

%)

Please select setting entries that you want to default

I Select all

I~ GENERAL
I~ sonue
I~ METHORK
I~ nerCT

I~ DIseLar

[

I~ Bcone
I~ w2
I Man
I~ PAW/TILT/Z00K

I Chennel Nane

Defanlt Path: C:\Documents end Settings\Adninistrator

Eeport Config

Tnport Confie





Figure 7‑49
7.3.2.1 Advanced
Advanced includes the following items. See Figure 7‑50.

· Account
· HDD management
· Alarm I/O
· Record control 
· Auto reboot
· Account 

Account interface is shown as in Figure 7‑50. Here you can add/delete user, add/delete group, modify user or group right, modify user password.

[image: image200.png]Configuration

=) Control Panel

= [2f SYSTEN THFO e
[2f veRsTon o ain
¥ o0 0 ssssee
105

= i s T Theasessiel
GENERAL =) user
'ENCODE BEBEEE
SCHEDULE default
RS232
WETHORK
ALARN Modi fy Password
DETECT —
'PAN/TILT/ZOOW

TERRILT/BACHE

= 8 woniee P
HDD MANAGENENT —
Alarn I/0
RECORD CONTROL Hodify Group
£ ACCOUNT —
AUTO REBOOT

= ADDTIONAL FUNCTION
ATW/POS
s

[Tou canf t edit yoursel£ except the password!





Figure 7‑50
Click add group button, you can see the following interface. See Figure 7‑51.

Here you can add one new group, and then select corresponding rights for the whole group accounts. 

[image: image201.png]Add Group

L —
Weno [

Authority
I an

DJContral Panel
Dl Skutdown the device
Etoni tor

Eoni tor_CH01

D toni tor_CH02

Dl oni tor 0105

D oni tor_CH08
DRerley

D Replay 01

D Replay CH02

D Replay CH03

D Replay 04
Cltetbrevien

I HetPrevi v CH01

D etbrevi eu Chi2
[

D et evi u Ot

I RecoRD coNTROL
EDownlosd Backr)

] Hardi s Hamagenst
EJPex/Tile/Zoon Flatforn Conteol
[ ——
Fiststan Tiro

e





Figure 7‑51
Click add user button, you can see the following interface.  See Figure 7‑52.

Here you can input a new user and then select corresponding rights. 

Please note one user must belong to one group and user right shall not exceed group rights limit.

[image: image202.png]User Name [ [ Reusashle
Reeeert! |
C—
Growp  Jadnin  v]
T

Authority

NetPravien CHIZ
e tPr avi en_CHIS
NetPravien CHI
RECORD CONTEOL
A Nl nad (R

e





Figure 7‑52
· Record Control

Record control interface is shown as in Figure 7‑53. Here you can enable record status for corresponding channel.

[image: image203.png]Configuration

=) Control Panel
= [2f SYSTEN THFO
[ VERSTON
[ 1on 10F0
[Bf L6
& i Sae
GENERAL
ENCODE
SCHEDULE
RS232
WETHORK
ALARN
DETECT
PAN/TILT/Z0OM
LT e
= G apvaED
DD MANAGEWENT
Alarn 1/0
=i com ]
ACCOUNT
AUTO REBOOT
S ADDTIONAL FUNCTION
ATH/POS.
s

RECORD CONTROL

Record Contral

Wode
Schedule
Memel
Step





Figure 7‑53
· HDD management 

This function needs DVR device supported. 

Please select the HDD first and then you can see the items on your right become valid. You can check the corresponding item here.  See Figure 7‑54.

After you completed setup, please click control execute button, you can see the DVR begins restarting. 

[image: image204.png]Configuration

] Control Panel HID MATAGENENT
= @ stsan Tivo HOD NARAGENERT
2 vERSTON —on
2 10 0 2 wot
3 e hreal
o 47 senmG & Feadfiiis
GENERAL
BiConE
SoHEDILE
Rs232
METHORK
AL
DeTECT
PAN/TILT/ZO0N
DERALT/BACKUE

= 8 xovmem o
5] HDD WANAGEMENT xecute

(3 Mam /0
RECORD CONTROL
Accourt
AUTO REBOOT

=) [ ADDTIONAL FURCTION
AT/0S
ms

€ Read Daly

3





Figure 7‑54
· Auto Reboot

System allows you to set auto reboot time. System also support auto delete files function. See Figure 7‑55.

[image: image205.png]Configuration

=) Contral Panel AUTO REBOOT
= [2f SYSTEN THFO
& veRston IuteReboot Systen
[2 100 TiF0 Ve
B e
= it sermmv
caERAL hute-Delete 0L Files
'ENCODE
SCHEDULE
RS232
WETHORK
ALARN
DETECT
'PAN/TILT/ZOOW
DEFAULT/BACKUP
= % e
HDD MANAGENENT
K 1/0
RECORD CONTROL
ACCOUNT
b= o)
= ADDTIONAL FUNCTION
ATW/POS
s

vEvER =





Figure 7‑55
· Alarm I/O

Here you can select alarm out put channel. See Figure 7‑56.

[image: image206.png]Configuration

5] Control Panel
= [2f SYSTEN THFO
[ VERSTON
[ 1on 10F0
[Bf L6
= it sermmv
caERAL
'ENCODE
SCHEDULE
RS232
WETHORK
ALARN
DETECT
'PAN/TILT/ZOOW
DEFAULT/BACKUP
= % e
HDD MANAGENENT
=l o]
'RECORD CONTROL
ACCOUNT
AUTO REBOOT
= ADDTIONAL FUNCTION
ATW/POS
s

Marn 1/0





Figure 7‑56
7.3.2.2 Additional function 
· DNS
Here you can set server or local service provider DNS address. See Figure 7‑57
[image: image207.png]Configuration

] Control Panel
= @ stsan Tivo
2 vERSTON
2 10 0
[# 16
= §i senmG
GENERAL
BrConE
soHmLE
Rz
ETHORK
AL
DeTECT
PAYTILL/Z000
DEFAILL/BACKIE
= G apvaED
HOD NARAGENERT
A 1/0
RECORD CONTROL
accomn
AITO REBOOT
=) (1 AIDTIONAL FUNCTION
Amwyeos

SE





Figure 7‑57
7.4 Search 

Here you can select video type, channel number and time to search the file you want.
Click search button, the interface is shown as below. See Figure 7‑58
Please use page up/down key to view the search results. 
Double click file name, you can view the file and system will automatically backup the image in you installation directory.

[image: image208.png]Paranstar

Begin Tine [2006-11- 4

153852

Operation
[Sewrah] Playback

Ed Tine  [2006-11-10

Chamnel  [a1

p
€ card

i tiple-chennel PLaghack
Cir2rar

153852

DownLoad Type [Fil

Downlosd

Open Local Record.

File Siz... | Bagin Tine

End Tine

Record Type

750414 2006-11-04
a11085 2008-11-04
S35 2008-11-04
7a094 2008-11-04
246385 2008-11-04
T17EeT 2008-11-04
750420 2008-11-04
ese04s 2008-11-04
agse 2008-11-04
480313 2008-11-04
12517 2008-11-04
e8sds 2008-11-04
100085 2008-11-04

Download Spesd: 0 ks

2006-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04

Regular
Regular
Regular
Regular
Regular
Regular
Regular
Regular
Regular
Regular
Regular
Regular
Regular





Figure 7‑58 
In the search result interface, you can select one or more files to download to your local PC.

The playback bar is shown as below. See Figure 7‑59.

1: Play
2: Pause
3: Stop 

4: Slow play

5: Fast play 

[image: image209.png]&G0 X E o X
2000-04-06 09

Ho video o Ea 0 X

EEOHEEEHHS=E




Figure 7‑59
7.4.1 Download 

You can select one or more files you want to download and then click down load button. System pops up a dialogue box asking you specify directory.  See Figure 7‑60.

[image: image210.png]swen [CHEEE -l & @ ckE-

Document On James.
DR

IDVRData

VDATA

File pame: Save
Save as pe: [da FesC dav) < Cancel

2





Figure 7‑60
Then you can input file name and click save to backup file in your local pc.

During the download process, there is a process bar for you reference and you can see download button becomes stop button. See Figure 7‑61.

[image: image211.png]Paranstar

Begin Tine [2006-11- 4

153852

Ed Tine  [2006-11-10

Chamnel  [a1

.
© caa

Wil tiple-chennel Flayback
Fir2rara

153852

Operation

Search | Plapback
DownLoad Type [Fil

St

Open Local Record.

File Siz... | Bagin Tine

End Tine

Record Type -

750414 2006-11-04
2 a11085 2008-11-04
3 S35 2005-11-04
246385 2006-11-04
T17EeT 2008-11-04
750420 2008-11-04
ese04s 2008-11-04
agse 2008-11-04
480313 2008-11-04
12517 2008-11-04
e8sds 2008-11-04
100085 2008-11-04

Download Speed: 1536 K/s
—

2006-11-04
2008-11-04
2005-11-04

2006-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04
2008-11-04

Regilar ... 1
Remilar | 1
Regilar 1
Regular

Regular

Regular

Regular

Regular

Regular

Regular

Regular

Regular





Figure 7‑61
7.5 Alarm
Here you can set alarm type and alarm prompt audio file. See Figure 7‑62.

[image: image212.png]| Event Type
T Video Loss

T Disk Full
T Disk Error
T~ Video Mask

T~ Encoder Alarn

T~ Motion Detect [ Extern Alarm

T hudio Alarn

[Operation

T Message Popup [ Video Poprup | Prompt

[ Mlarn Sound

Sound Path

I~ Sound Pop-up

>

Tine

Devics T

Type

Alarn Port/Channel





Figure 7‑62
7.6 About

Click about button, you can view current web client information. See Figure 7‑63.

[image: image213.png]Webree Control, Version: 2.1.4.8
NETSI, Version: 3.3.2.9

PLATSIE, Version: 3.24.1.908
Copyright (©) 2008

i





Figure 7‑63
7.7 Log out

Click log out button, system goes back to log in interface. See Figure 7‑64.

[image: image214.png]WEBservice - Windows Internet Explorer.

G- ] hetprinio. 103,161
& & @i =

pore. T T LT Gl a0 -




Figure 7‑64
7.8 Un-install Web Control

You can use web un-install tool “uninstall web.bat” to un-install web control.
Please note, before you un-installation, please close all web pages, otherwise the un-installation might result in error.
8 Enterprise Professional Surveillance System
In this chapter, we introduce how to add devices and how to enable monitor function. For detailed operation, please refer to enterprise professional enterprise surveillance system user’s manual.

8.1 Log in 

Double click enterprise profession surveillance platform icon ([image: image215.png]i~



).If it is your first time to use the system, you can see the following interface. See Figure 8‑1.
[image: image216.jpg]Select Language

Thiz iz the first tine you run
this progran, plesse select the
Language ou want to use

[English.





Figure 8‑1
After selecting a language, you can see the following interface. Here you can input user name and password to log in you selected server. The log in interface is shown as in Figure 8‑2.

If it is your first time to use the system, you need to click add button to add a device first.  Default central control server name is Local. 

System default username and password both are admin.
Note:

For security reasons please modify your password after you first logged in.

[image: image217.png][SYSmEM]

Vzer

P adnin

[“save Password

Prepare for logging in

Select Login Server

SN Server Nane| Server IP adiress  Server Port Yusber
1 Local 10.10.2.131 £

A [ zae (Dot [ Clew ] [Bmmi €]

Register Service
Please select register type

entrel Center [P

ransfer Server o0





Figure 8‑2
8.2 Enable Monitor
After successfully logged in, please select the device and then click connect/disconnect button（[image: image218.bmp]）. 

Select the channel you wan to view, click connect/disconnect button（[image: image219.bmp]） again.

[image: image220.png]& Enterprise Pro Surveillance Platfora(Business) Verd.6.0.0) (build 2008-03-26) (UnRegister)

EEODHEEHHHSS=E





Figure 8‑3 
There are totally six sections:

· Section 1: There are eight function keys: monitor, E-map, record, save, alarm, configuration, log and system. 
· Section 2: Here is for you to view channel video. 
· Section 3: Here is for you to select display mode. System supports various display modes. HD item is for you to select priority between real-time and video fluency. 
· Section 4: Here is for you to view current help information.

· Section 5: Here is to display data flux and CPU status. 

· Section 6: There are four function buttons: PTZ, color, device, and plan. Please note system only supports one window in full-screen. Here we recommend resolution 1024*768.

Select a device name and then click connect/disconnect button, you can connect current device to network. You can see a little red -cross below the name disappeared.

Click one window on you left side and then double click channel name on you right hand, you can see the corresponding video in current window.

Please refer to Figure 8‑4 for connection/disconnection information.
[image: image221.jpg]% Device List

5 - GEE
& ot
oz
o o3
o G4
W GBES.
W GBES.
W GBE_T
W GBES.

[t




Figure 8‑4
8.3 Add New Device

Click configuration button, you can see a network management unit shown as in Figure 8‑5.
There are five function buttons, from left to right: device management, user manager, E-map, device configuration and upgrade. 

[image: image222.png]5 Fet Nanager UnitVerd.6.0.0 (build 2008-03-26) (UnRegistex)

Devica List

Divica tiama| TP dar sz 0 Port omber | Uzes i | Chmaallmoua? - T

& buiLaing 1 1010306 1T wnin B e
60191 50,122 3TTTT o °

FERTCC I win s

£ building 1





Figure 8‑5
.
9 Player Operation 
9.1 General Introduction 

This player integrates audio play and video play together. It supports most popular audio file and support AVI convert also.
It supports the following file format:

· Dahua file format(record, snapshot, frame setup).

  *.DAV、 *.MP4  *.264  *.MP1

· Compression format(does not support record and snapshot)

*.MP4  *.264
· Image format
JPG & BMP
In player interface, there are following items. See Figure 9‑1.
· 1: Frame rate. There are nine options:1f/2f/3f/6f/12f/25f/50f/75f/100f， You can click corresponding pane to select.

· 2 :Play

· 3:Pause

· 4:Stop

· 5 :Start record 

· 6: Previous file

· 7:Backward

· 8 Forward

· 9 Next file

· 10: Capture: Here is for you to snapshot important image.

· 11: Sound: + is to increase volume, - is to decrease volume. Click icon “[image: image223.png]


” to turn on or off volume.

· 12: Current frame rate: Here you can view current frame rate.

· 13: Current status: Here you can view play status bar, play mode and volume status (on or off).

· 14: Convert to AVI: System can convert the video file of suffix name such as mp4, Dav, mpeg, 264 to avi format.

· 15: Full-screen: There are two ways: click full-screen button to review in full-screen mode. The other way is to double click current window to review in full-screen. 

· 16: Play mode: shuffle or repeat.

· 17: Change language: The player supports two languages: English/Chinese. System pops up dialogue box asking for restarting. Click OK, system automatically restart. 

· 18: Parameter setting. You can set parameter setup/.

[image: image224.jpg]LTt




Figure 9‑1
9.2 Play File
Click play button (button 2 in Figure 9‑1), you can see an interface is shown as in Figure 9‑2.
[image: image225.png]Open Media File:.

Loskin: [ < Local Disk () + @ ok B

= =T
e} {Eavecumentssndsetngs

MyRecent  (S0ownLoad
Documerts | =y

@ =
(S

Deskon  (Spiure
(Sprogran s

‘s) (S
| oo
My Documents =

My Computer

Mgmk e = oen

Places
Fils of type: AL Media Fies - Cancel

‘





Figure 9‑2
Or you can just select the file and then drag it to the player to play.

If there are recorded files in player folder, you can double click player to begin auto play. Click previous button (button 6 in Figure 9‑1) and next button (button 9 in Figure 9‑1) to view corresponding files. 
When it is PIP mode, left click mouse to drag the small window. 

Left double click small window to switch it to big window. 

9.3 Right Mouse Menu 
Right click mouse, you can see an interface is shown as in Figure 9‑3.

[image: image226.png]Pl Sereen
Fils List
Start Racord
Step

Copture
Comvert to NI
ey

Panse

Forward
Backward

Hext File

Pre File

Paramster Setting





Figure 9‑3
9.3.1 Convert to AVI

System supports to convert *.mp4,*.Dav,*.mpge,*.264 file to *.avi

 In Figure 9‑3, select convert to AVI item. You can see an interface is shown as in Figure 9‑4.
[image: image227.png]T DVR To AVI

Target File

Wake AVT File





Figure 9‑4
Please select the source file and then click browser button after the “target file” to specify the storage location.
[image: image228.jpg]T DVR To AV

Sowrca Fila

[P0 5ot e Gomer 21 ULy gbh X5 V1 .10, & K DR0 3\ EU0B 102 22T Browss
Target Fils
[ emente e St imes i st Demwmens [eowee ]

Wake AVT File





Figure 9‑5
Now click make AVI file button, you can see there is a processes bar for your reference. See Figure 9‑6.
[image: image229.jpg]T DVR To AVI

Sowrca Fils

[P0 St CimeR Gomer 21 R ayer b 15 V15105 K DB0S 3\ Z00B 102 o221 o Browss
Target File
T e (e ]

ek AVT File





Figure 9‑6
Please note; system does not support play and convert one file at the same time. 
9.3.2 Parameter Setting 

In Figure 9‑3, highlight parameter setting item.  You can see an interface is shown as in Figure 9‑7. Frame rate value ranges from 0 to 100. 

[image: image230.png]Parameter Setting

OCE/S G200 [ Fixed

S Lightneszi 0 %

[ File hssociated
[l S 1V

| Save Picturs Path

Changs Path| [P Frozen Files\layer —

T~ Use DirectShow

s





Figure 9‑7
· Modify path 

System default path is player installation location. Click path button, you can see an interface is shown as in Figure 9‑8. Now you can modify file storage location.

[image: image231.jpg]Browse for Folder X

Select the il drectory for pictures

Deskiop
=3

% % Local Disk (C1)
& < Local Parttion 01) 1
& % Local Parttion (£2)
5 .2 DVD Drive () =
5 e Removable Dik (G1)
5 O Control Panel
5 (2 Shered Documents
(= (5 Adniistrator's Documerts

2 Dowrioads

] o)





Figure 9‑8
· Use DirectShow
If you can not play current file, you can enable this function to try again. If it does not work, then player does not support current file format. 

10  FAQ
· System can not boot up.
· Input voltage is not right 

· Power button connection is not good.

· Power button is damaged 

· System upgrade error

· HDD malfunction or HDD cable is damaged 

· Some series is not compatible with Seagate DB35.1，DB35.2，SV35， and Maxtor 17G. Please upgrade to the latest software. 
· Front panel error

· DVR main board is damaged.

· System auto reboots after several minutes or it often goes down. 

· Input voltage is not stable or it is too low 

· HDD malfunction 

· Power button is not sufficient 

· Ventilation problem or there is too much dust.

· DVR hardware problem.
· System can not detect the HDD

· Check HDD power cable connection 

· HDD cable is damaged 

· HDD malfunction 

· SATA port is damaged 
· No video in single (several or all) channel(s)
· Program error. Please upgrade to the latest version. 
· Brightness becomes o. Restore default setup. 
· No video input signal or it is too weak
· Enabled channel protection function
· DVR hardware error. 
· Real-time video brightness or color is distorted. 

· When BNC output, NTSC and PAL setup is not right. The video may becomes black and white 

· DVR and monitor resistance is not right 

· Transmission distance is too long or the video transmission cable attenuation is too heavy.

· DVR color setup is not right.  

· Can not find local video file 

· HDD cable error 

· HDD malfunction 

· Program version problem 

· The files has been overwritten 

· Searched file has not been open. 

· Local searched video is distorted 

· Video quality setup is too low.
· Error occurs when program read and write the data. Thought the displayed bit stream is low, it is full of mosaic when playback. Please reboot.
· HDD cable problem 

· HDD malfunction 

· DVR hardware error 

· There is no audio when monitor 

· It is not a power pick up 

· It is not a power speaker

· Audio cable is damaged 

· DVR hardware error 
· There is audio when monitor. But it is no audio when playback 

· Setup problem. Please enable audio function. 

· Corresponding channel has not enabled video; The playback may not fluent when screen is blue. 

· Time display is not right 

· Setup error 

· Battery contact is not right or voltage is too low. 

· Crystal Oscillator error 
· DVR can not control PTZ
· Front-end PTZ error 

· PTZ setup is not right. Cable connection or installation is not right.

· DVR PTZ setup error 
· Protocol setup is not compatible. 

· PTZ address is not right 

· When connect several decoder please use a 120Ohm between the decoder far A end and B end to eliminate reflection and resistance matching. Otherwise the PTZ control is not stable 

· The distance is too long.

· Motion detection function does not work 
· Time period setup is not right 

· Motion detection zone setup is not right. 

· Sensitivity is too low. 
· Hardware problem for some series.

· Can not log in web or client-end 

· Can not install client-end or can not display properly. For Windows ME or Windows 98 user, please upgrade to Windows 2000 sp4 or install the low version client-end.

· ActiveX control installation is blocked. 

· System needs Dx8.1 (or higher) support. Please upgrade display card driver, 

· Network connection problem.

· Network setup error

· Password and user name does not match.

· Client-end version is not compatible.

· There is no video or there is mosaic when preview or playback remotely. 

· Network is not fluent 

· Client-end PC resource is not sufficient 

· Enable multiple cast function in network setup interface. Usually we do not recommend this mode. 

· Enable local privacy mask setup or channel protection. 
· Current account has no monitor right.

· Local output video is not good.

· Network connection is not stable 
· Network is not stable 

· IP address conflict 

· Mac address conflict 

· PC or DVR network card is not good.
· Burning/USB backup error 

· Burner and HDD are using the same data cable. 

· Data is too much and occupying too much resources.

· Data amount is more than burner free space, which may result in burner error. 

· Backup device is not compatible.

· Backup device is damaged. 

· Keyboard can not control DVR
· DVR RS232 setup is not correct. 

· Address is not correct.

· Power is not sufficient when there are several switchers. 
· Transmission distance is too long. 
· Alarm signal can not be cancelled
· Alarm setup is not correct.

· Enable alarm output manually 

· Input device error or connection is not correct. 

· Some series has this problem. Please upgrade to the latest version. 

· Alarm function does not work 

· Alarm setup is not correct. 

· Alarm ground cable connection is not correct.

· Alarm input signal is not correct. 

· Connect two circuits to one alarm device,.

· Remote control does not work 

· Remote address is not right 

· Remote control distance is too long or angle is too narrow.
· Remote control battery voltage is too low.

· Remote control is damaged or DVR front panel is damaged, 

· Video storage time is not sufficient 

· Front-end camera quality is too bad or lens is too dirty.

· HDD capacity is not sufficient. 

· HDD damaged. 

·  Can not playback the downloaded file.
· There is no player

· There is no DX8.1 (or higher).

· When convert file to .AVI format, please use MEDIA PLAYER to playback. There is no DivX503Bundle.exe plug-in.
· Forgot local admin account password or network password. 

· Please contact our technical support engineer or local customer service person for help. 

Slight difference may be found in user interface.

All the designs and software here are subject to change without prior written notice.

Please visit our website for more information.
Appendix A
HDD Capacity Calculation
Calculate total capacity needed by each DVR according to video recording (video recording type and video file storage time). 

Step 1: According to Formula (1) to calculate storage capacity 
[image: image232.wmf]i

q

 that is the capacity of each channel needed for each hour, unit Mbyte. 


[image: image233.wmf]1024

3600

8

¸

´

¸

=

i

i

d

q

                            (1)

                   In the formula: 
[image: image234.wmf]i

d

 means the bit rate, unit Kbit/s
Step 2: After video time requirement is confirmed, according to Formula (2) to calculate the storage capacity
[image: image235.wmf]i

m

, which is storage of each channel needed unit Mbyte. 


[image: image236.wmf]i

m

 = 
[image: image237.wmf]i

q

×
[image: image238.wmf]i

h

×
[image: image239.wmf]i

D

                                 (2)

In the formula: 


[image: image240.wmf]i

h

 means the recording time for each day (hour)


[image: image241.wmf]i

D

 means number of days for which the video shall be kept
Step 3: According to Formula (3) to calculate total capacity (accumulation) 
[image: image242.wmf]T

q

 that is needed for all channels in the DVR during scheduled video recording. 


[image: image243.wmf]å

=

=

c

i

i

T

m

q

1

                               (3)

In the formula: 
[image: image244.wmf]c

means total number of channels in one DVR 

Step 4: According to Formula (4) to calculate total capacity (accumulation) 
[image: image245.wmf]T

q

that is needed for all channels in DVR during alarm video recording (including motion detection). 


[image: image246.wmf]å

=

=

c

i

i

T

m

q

1

×a%                          (4)

In the formula：a% means alarm occurrence rate
Appendix B
Compatible USB Drive List
NOTE: Please upgrade the DVR firmware to latest version to ensure the accuracy of the table below. If you use the USB drive, please confirm the format FAT or FAT32.

	Manufacturer 
	Model 
	Capacity

	Sandisk
	Cruzer Micro 
	512M

	Sandisk
	Cruzer Micro 
	1G

	Sandisk
	Cruzer Micro 
	2G

	Sandisk
	Cruzer Freedom 
	256M

	Sandisk
	Cruzer Freedom 
	512M

	Sandisk
	Cruzer Freedom 
	1G

	Sandisk
	Cruzer Freedom 
	2G

	Kingston
	DataTraveler Ⅱ
	1G

	Kingston
	DataTraveler Ⅱ
	2G

	Kingston
	DataTraveler
	1G

	Kingston
	DataTraveler
	2G

	Maxell
	USB Flash Stick
	128M

	Maxell
	USB Flash Stick
	256M

	Maxell
	USB Flash Stick
	512M

	Maxell
	USB Flash Stick
	1G

	Maxell
	USB Flash Stick
	2G

	Kingax
	Super Stick
	128M

	Kingax
	Super Stick
	256M

	Kingax
	Super Stick
	512M

	Kingax
	Super Stick
	1G

	Kingax
	Super Stick
	2G

	Netac
	U210
	128M

	Netac
	U210
	256M

	Netac
	U210
	512M

	Netac
	U210
	1G

	Netac
	U210
	2G

	Teclast
	Ti Cool
	128M

	Teclast
	Ti Cool
	256M

	Teclast
	Ti Cool
	512M

	Teclast
	Ti Cool
	1G

	Teclast
	Ti Cool
	2G


Appendix C
Compatible CD/DVD Burner List

NOTE: Please upgrade the DVR firmware to latest version to ensure the accuracy of the table below. And you can use the USB cable with the model recommended to set USB burner.
	Manufacture 
	Model 
	Port 
	Type 

	Sony
	DRX-S50U
	USB
	DVD-RW

	Sony
	DRX-S70U
	USB
	DVD-RW

	Sony
	AW-G170S
	SATA
	DVD-RW

	Samsung
	TS-H653A
	SATA
	DVD-RW

	Panasonic
	SW-9588-C
	SATA
	DVD-RW

	Sony
	DRX-S50U
	USB
	DVD-RW

	BenQ
	5232WI
	USB
	DVD-RW


Appendix D       Compatible CD/DVD CD List

Please refer to the following sheet for compatible CD/DVD burner list. 

	Manufacture
	Capacity 
	Type 

	Mitsubishi
	4.37GB
	DVD+RW

	Mitsubishi
	4.37GB
	DVD+R

	Mitsubishi
	702MB
	CD+RW

	Sony
	4.37GB
	DVD+R

	Sony
	4.37GB
	DVD-R

	Maxell
	4.37GB
	DVD-R

	Maxell
	4.37GB
	DVD+R

	BenQ
	4.37GB
	DVD+RW

	Verbatim
	4.37GB
	DVD-R

	TDK
	4.37GB
	DVD-R

	Tsinghua Tongfang
	4.37GB
	DVD+R


Appendix E       Compatible SATA HDD List

NOTE: Please upgrade the DVR firmware to latest version to ensure the accuracy of the table below. And SATA HDD should be used for the DVR with SATA port.
	Manufacturer 
	Series 
	Model 
	Capacity 
	Type 

	Seagate
	Barracuda.10
	ST3750640AS
	750G
	SATA

	Seagate
	Barracuda.10
	ST3500630AS
	500G
	SATA

	Seagate
	Barracuda.10
	ST3400620AS
	400G
	SATA

	Seagate
	Barracuda.10
	ST3320620AS
	320G
	SATA

	Seagate
	Barracuda.10
	ST3250620AS
	250G
	SATA

	Seagate
	Barracuda.10
	ST3250820AS
	250G
	SATA

	Seagate
	Barracuda.10
	ST3160815AS
	160G
	SATA

	Seagate
	Barracuda.10
	ST380815AS
	80G
	SATA

	Seagate
	Barracuda.9
	ST3160811AS2
	160G
	SATA

	Seagate
	Barracuda.9
	ST3120811AS2
	120G
	SATA

	Seagate
	Barracuda.9
	ST380811AS2
	80
	SATA

	Seagate
	Barracuda.9
	ST380211AS2
	80G
	SATA

	Seagate
	Barracuda.11
	 ST3750330AS 
	750G
	SATA

	Seagate
	Barracuda.11
	ST3500320AS
	500G
	SATA

	Maxtor
	DiamondMax 20
	STM3320820AS
	320G
	SATA

	Maxtor
	DiamondMax 20
	STM3250820AS
	250G
	SATA

	Maxtor
	DiamondMax 21
	STM3160211AS
	160G
	SATA

	Maxtor
	DiamondMax 21
	STM380211AS
	80G
	SATA

	Maxtor
	DiamondMax 21
	STM340211AS
	40G
	SATA

	Western Digital
	Cariar SE
	WD3200JD
	320G
	SATA

	Western Digital
	Cariar SE
	WD3000JD
	300G
	SATA

	Western Digital
	Cariar SE
	WD2500JS
	250G
	SATA

	Western Digital
	Cariar SE
	WD2000JD
	200G
	SATA

	Western Digital
	Cariar SE
	WD1600JD
	160G
	SATA

	Western Digital
	Cariar SE
	WD1600JS
	160G
	SATA

	Western Digital
	Cariar SE
	WD1200JS
	120G
	SATA

	Western Digital
	Cariar SE
	WD800JD
	80G
	SATA

	Western Digital
	Cariar 
	WD1600AABS2
	160G
	SATA

	Western Digital
	Cariar 
	WD800BD
	80G
	SATA

	Western Digital
	Cariar  SE16
	WD7500KS2
	750G
	SATA

	Western Digital
	Cariar  SE16
	WD5000KS2
	500G
	SATA

	Western Digital
	Cariar  SE16
	WD4000KD2
	400G
	SATA

	Western Digital
	Cariar  SE16
	WD3200KS2
	320G
	SATA

	Western Digital
	Cariar  SE16
	WD2500KS2
	250G
	SATA


Slight difference may be found in user interface.

All the designs and software here are subject to change without prior written notice.

Please visit our website for more information.
You can click this icon to display or hide the PTZ control platform.








Connection








2            3                    4





�





1          






























































�





�








1                         2               3





4                         





1   2     3   4    5      6





Section 5





Section 2





Section 4





Add organization structure 





Add device








Connected channel





Disconnected channel








Section 3





�





Section 4





Section 5





Section 3








Section 6





Section 2





Section 1











Section1 





Input your IP address here.

















[image: image248.jpg]
PAGE  
74

_1155635033.unknown

_1155635040.unknown

_1155635042.unknown

_1246548990.unknown

_1155635041.unknown

_1155635037.unknown

_1155635038.unknown

_1155635039.unknown

_1155635034.unknown

_1155635014.unknown

