
HD IR Network Dome Camera Quick Start Guide

Version 1.0.2

Welcome

Thank you for purchasing our network camera!

This quick start guide is designed to be a reference tool for your system. 

Please keep this start guide well for future reference.

Please open the accessory bag to check the items one by one in accordance with the list below. Contact your local retailer ASAP if something is missing or damaged in the bag.

1．Electrical safety

All installation and operation here should conform to your local electrical safety codes. 

The power shall conform to the requirement in the SELV (Safety Extra Low Voltage) and the Limited power source is rated 12V DC or 24V AC in the IEC60950-1. (Refer to general introduction) 
Please note: Do not connect two power supplying sources to the device at the same time; it may result in device damage!

We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation. 

We are not liable for any problems caused by unauthorized modification or attempted repair.

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Do not apply power to the camera before completing installation. 

Please install the proper power cut-off device during the installation connection.

Always follow the instruction guide the manufacturer recommended.

4．Qualified engineers needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

This series network camera should be installed in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

Please keep it away from the electromagnetic radiation object and environment.

Please make sure the CCD (CMOS) component is out of the radiation of the laser beam device. Otherwise it may result in CCD (CMOS) optical component damage.

Please keep the sound ventilation. 

Do not allow the water and other liquid falling into the camera.

Thunder-proof device is recommended to be adopted to better prevent thunder.

The grounding studs of the product are recommended to be grounded to further enhance the reliability of the camera.

6. Daily Maintenance

Please shut down the device and then unplug the power cable before you begin daily maintenance work. 

Do not touch the CCD (CMOS) optic component. You can use the blower to clean the dust on the lens surface.

Always use the dry soft cloth to clean the device.  If there is too much dust, please use the water to dilute the mild detergent first and then use it to clean the device. Finally use the dry cloth to clean the device.

Please put the dustproof cap to protect the CCD (CMOS) component when you do not use the camera.

Dome enclosure is the optical component, do not touch the enclosure when you are installing the device or clean the enclosure when you are doing maintenance work. Please use professional optical clean method to clean the enclosure. Improper enclosure clean method (such as use cloth) may result in poor IR effect of camera with IR function.  

7. Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components are included.

Contact your local retailer ASAP if something is broken in your package.

	Accessory Name
	Amount

	Network Camera Unit
	1

	Quick Start Guide
	1

	Installation Accessories Bag
	1

	CD
	1


Table of Contents 

11
Structure


11.1
Components


11.2
Framework and Dimension


32
Device Installation


32.1
Installation Steps


53
Quick Configuration Tool


53.1
Overview


53.2
Operation


74
Web Operation


74.1
Network Connection


74.2
Login and Main Interface


10Appendix Toxic or Hazardous Materials or Elements





1 Structure 

1.1 Components
You can refer to the following figure for multiple-function combination cable information. See Figure 1‑1.

[image: image1.png]



Figure 1‑1 Multiple-function combination cable

Please refer to the following sheet for detailed information.
	No.
	Component Name
	Description

	1
	IR LED 
	IR LED for lighting

	2
	Built-in MIC
	Audio collection

	3
	Lens
	Fixed focus M12 port

	4
	Network Port
	Connect to standard Ethernet 

	5
	Power
	DC 12V


1.2 Framework and Dimension

Please refer to the following two figures for dimension information. The unit is mm. See Figure 1‑2 and Figure 1‑3.
[image: image2.png]



Figure 1‑2 Dimension illustration 1
[image: image3.png]



Figure 1‑3 Dimension illustration 2
2 Device Installation 
2.1 Installation Steps 

Important

Before the installation, please make sure the installation environments can at least support 3x weight of the camera. 
Please follow the steps listed below to install the device.  Please refer to Figure 2‑1 for reference.

Step 1

Please take the installation position map in the accessories bag, and then paste it on the ceiling or the wall according to your monitor area requirements.
Step 2
Draw and then dig three plastic expansion bolts holes in the installation surface and then insert three expansion bolts in the holes. Secure these three bolts firmly. 

Step 3
Rotate dome enclosure counterclockwise, and open the device.                                                                                                                                                                                                                                                                                                                                                                                                                 

Step4

Select run the wire through bottom or side, and insert multi-function cable into exit hole on installation surface.

Step 5
Adjust pedestal position, aim at bottom hole. Take out three ST4.2 self-tapping screws in accessories bag. Insert them into the plastic expansion bolt bottom hole and fix the pedestal on installation surface. 

Step 6

Connect GND hole to the ground. GND hole is next to exit hole at dome tail. GND screw is M2X4 pan screw, effective depth is 4mm. See Figure 2‑2.

[image: image4.png]Expansion
Bolt

Ceiling or wall
Pedestal

Self.

tapping
screw

Dome
body

Enclosure





Figure 2‑1 Device installation illustration 1
[image: image5.png]



Figure 2‑2 Device installation illustration 2

3 Quick Configuration Tool

3.1 Overview 

Quick configuration tool can search current IP address, modify IP address. At the same time, you can use it to upgrade the device. 

Please note the tool only applies to the IP addresses in the same segment.

3.2 Operation

Double click the “ConfigTools.exe” icon, you can see an interface is shown as in Figure 3‑1.

In the device list interface, you can view device IP address, port number, subnet mask, default gateway, MAC address and etc. 

[image: image6.png]ConfigTool V1.0.5

e

vice List

s

TP Address

Port

Subnet Mask

Defanlt Gatevsy

Mae Adress

<l

182,188, 1.108

Edit

7

255, 255.255.0

182188 1.1

90:02:49:Tb:50

Tip: You can click “login® button directly and
then input corresponding infornation to login

]

Uperade





Figure 3‑1 Search interface
Select one IP address and then right click mouse, you can see an interface is shown as in Figure 3‑2.

Select the “Open Device Web” item; you can go to the corresponding web login interface.
[image: image7.jpg]Device List

ou can click “login” button directly and
then imput. corresponding infornation to login.

parate | Refresh Login Logeut |

&





Figure 3‑2 Search interface 2
If you want to modify the device IP address without logging in the device web interface, you can go to the configuration tool main interface to set. 

In the configuration tool search interface (Figure 3‑1), please select a device IP address and then double click it to open the login interface. Or you can select an IP address and then click the Login button to go to the login interface. See Figure 3-3.

In Figure 3-3, you can view device IP address, user name, password and port. Please modify the corresponding information to login. 

Please note the port information here shall be identical with the port value you set in TCP port in Web Network interface. Otherwise, you cannot login the device. 

If you are using device background upgrade port 3800 to login, other setups are all invalid.

[image: image8.png]1P Ackress:

User Name:

Passuord:

Port

152.168.1.108

e —





Figure 3‑3 Login prompt
After you logged in, the configuration tool main interface is shown as below. See Figure 3-4.

[image: image9.jpg]etk Frsnten 108 Systen Tnernationl Svten Vst

~General Parameter

Coveenbe [ 2]
T —
Subnet Mask: ~ [255.255.0.0

O —
O

= =





Figure 3‑4 Main interface
For detailed information and operation instruction of the quick configuration tool, please refer to the Quick Configuration Tool User’s Manual included in the resources CD.

4 Web Operation

This series network camera products support the Web access and management via PC.

Web includes several modules: Monitor channel preview, system configuration, alarm and etc.

4.1 Network Connection 

Please follow the steps listed below for network connection.

· Make sure the network camera has connected to the network properly.

· Please set the IP address, subnet mask and gateway of the PC and the network camera respectively. Network camera default IP address is 192.168.1.108. Subnet mask is 255.255.255.0. Gateway is 192.168.1.1

· Use order ping ***.***.***.***(* network camera address) to check connection is OK or not. 

4.2 Login and Main Interface 

Open IE and input network camera address in the address bar. See Figure 4- 1.

[image: image13.jpg]E E sl onTrRoL”




[image: image14.jpg]


[image: image10.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 4- 1 IP address
The login interface is shown as below. See Figure 4- 2.

Please input your user name and password. 

Default factory name is admin and password is admin. 

Note: For security reasons, please modify your password after you first login.
[image: image11.png]



Figure 4- 2 Web login
After you successfully logged in, please install WEB plug-in unit. Please refer to the Web Operation Manual included in the resource CD for detailed operation instruction.
See Figure 4- 3.
[image: image12.png]L ol (I ST T AT
Lisncin ] oo RS- LA Lo

pre— 1280720





Figure 4- 3 Web monitoring window
Please refer to the Web Operation Manual included in the resource CD for detailed operation instruction.

Appendix Toxic or Hazardous Materials or Elements

	Component Name
	Toxic or Hazardous Materials or Elements

	
	Pb
	Hg
	Cd
	Cr VI
	PBB
	PBDE

	Circuit Board Component
	○
	○
	○
	○
	○
	○

	Device Case
	○


	○
	○
	○
	○
	○

	Wire and Cable
	○


	○
	○
	○
	○
	○

	Packing Components
	○
	○
	○
	○
	○
	○

	Accessories
	○
	○
	○
	○
	○
	○


O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.
X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. During the environmental-friendly use period (EFUP) period, the toxic or hazardous substance or elements contained in products will not leak or mutate so that the use of these (substances or elements) will not result in any severe environmental pollution, any bodily injury or damage to any assets.  The consumer is not authorized to process such kind of substances or elements, please return to the corresponding local authorities to process according to your local government statutes.
Note:
· This quick start guide is for reference only. Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice. 
· All trademarks and registered trademarks mentioned are the properties of their respective 
owners.
· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website or contact your local service engineer for more information.
































































































































































Input your IP address here























PAGE  
iii

