Villa Door Station Quick Start Guide

-----VTO6110B series
V1.1.0
Table of Contents
31 Product Appearance


42 Basic Function Introduction


42.1 Call Manager Center


52.2 Call User


52.2.1 Connecting Status


52.2.2 Calling Status


52.3 Monitor


52.4 Unlock


52.4.1 Unlock under Connecting Status (access control module required)


52.4.2 Unlock under Calling Status (access control module required)


52.4.3 Unlock under Monitoring Status (access control module required)


62.4.4 Unlock via IC Card (access control module required)


62.5 Compensation of Light


62.6 Vandal Proof


63 Introduction to Web


63.1 Login Interface


73.2 System Config


73.2.1 Local Config


93.2.2 Indoor Station (VTH) Manager


93.2.3 LAN Config


103.2.4 Network Config


113.2.5 Change Password


113.3 Info Search


113.3.1 Call History


123.4 Status Statistics


123.4.1 VTH status


123.5 Logout


123.5.1 Reboot Device


123.5.2 Logout


134 Technical Specifications


145 Device Port Illustration


156. Installation Guide


167 FAQ





1 Product Appearance
Please wait about 10s for the indicators in touch button to turn on after you plug the device to power supply. It takes about 60s for all the indicators to turn on and then off. After the system boots up properly, you will see its front as shown in Figure 1- 1(black) and Figure 1- 2 (white).
[image: image1.jpg]



Figure 1- 1
[image: image2.png]



Figure 1- 2
	No.
	Name
	Function

	1
	Compensation
	Light compensation will automatically turn on during connecting status if there is no enough light in environment.

	2
	Camera
	It monitors corresponding door region.

	3
	Card Swiping Area
	You can swipe card in this area.

	4
	Touch Button
	You can touch this button (the blue indicator will be on) to call VTH or manager center. 


2 Basic Function Introduction
2.1 Call Manager Center


Within the time that allows you to call manager center, you can touch the button to call manager center. Manager center’s time can be set on management platform or villa VTO’s web-end. Once manager center picks up the call, you can perform a visual bidirectional talk with the manager center. You can touch the button on VTO to end call at any time. 
2.2 Call User 
2.2.1 Connecting Status
Within the time that allows you to call VTH, you can touch the button to call VTH. Excluding time to call manager center, all remaining time supports call to VTH. During connecting, you can touch the button on VTO to end call at any time. 
2.2.2 Calling Status
Under connecting status, if the call is picked up, you will enter calling status. You can perform a visual bidirectional talk with the VTH. During calling status, you can touch the button on VTO to end call at any time. 
2.3 Monitor
Both VTS and VTH are able to monitor this VTO by enabling the camera to capture local circumstance. 
2.4 Unlock
2.4.1 Unlock under Connecting Status (access control module required)
Under connecting status, VTS or VTH can remotely unlock door. VTO will return to standby interface after call ends or countdown stops. 
2.4.2 Unlock under Calling Status (access control module required)
Under calling status, VTS or VTH can remotely unlock door. VTO will return to standby interface after call ends or countdown stops. 

2.4.3 Unlock under Monitoring Status (access control module required)
Under monitoring status, VTS or VTH can remotely unlock door. VTO will return to standby interface after call ends or countdown stops. 

2.4.4 Unlock via IC Card (access control module required)
By swiping authorized IC card, you can unlock door after local verification. 

2.5 Compensation of Light
In dark environment or at night, the VTO adopts auto photoreception technology which achieves light compensation in connecting status. 
2.6 Vandal Proof
There is one channel of vandal proof which will generate alarm sound and report to the manager center once VTO is forced to leave the wall. 
3 Introduction to Web
3.1 Login Interface
In Internet Explorer, input the IP address of villa VTO, a webpage pops up as in Figure 3- 1.

[image: image3.png]



Figure 3- 1
Default username: admin

Default password: admin.
Click login to enter WEB interface. 

The WEB config interface consists of the following:

	No. 
	Name 
	Function

	1
	System Config
	Here you can config VTO6110B’s device parameter and LAN info.

	2
	Info Search
	Here you can search VTO6110B’s call records.

	3
	Status Statistics
	Here you can video VTO6110B’s VTH status statistics.

	4
	Logout
	Here you can reboot VTO6110B and logout WEB-end.


3.2 System Config


System config consist of: local config, indoor station manager (VTH), LAN config, network config and change password.

3.2.1 Local Config
Local Config

Local config includes issuing card, unlock and time setup. See Figure 3- 2.

[image: image4.png]System Config
> Indoor Station Manager
> LAN Config
> Network Config

Info Search
Status Statistics

Logout

Local Config ASC Manager System Time
Term No 1
Section No 1
Building No 101
Building Unit No. 1
District Unit S 101
Unit Layer Amount 10

Room Amountin One Layer: |4

Device Type Villa Station <
VTO Model V1081108 <

Video Format

Reboot Date [Muesssy =]
Volume Config 20 <
Frame Rate % <
Version Info 2012-11-23 V1.100.0.0

Default Reffesh oK

Defaul All





Figure 3- 2
Please note:

1) Frame rate: set 30 as frame rate for NTSC and 25 as frame rate for PAL standard.

2) Delete all: Click this button and then confirm. VTO will restore default settings. Be cautious! 
3) Video format: select between D1 and WVGA. D1 resolution: 704x576; WVGA resolution: 800x480.

A&C Manager

A&C manager interface is shown in Figure 3- 3.

[image: image5.png]System Config Local Config A8C Manager

> Indoor Station Manager
> LAN Config
> Network Config

> Change Password

Unlock Period

Info Search
Check Door Sensor Signal Before Lock
Status Statistics
FTP P

Logout

FTP Port
FTP User
FTP Password

System Time

Issue Card

Default

Reffesh

oK





Figure 3- 3

Under local config interface, click on A&C manager. The initial villa VTO password is 123456 and to unlock, input #123456#. Here you can change password, set unlock responding interval and unlock period. 

FTP IP: FTP IP is used in storing snapshot taken by the VTO while user can view them by logging in this FTP IP.


Issue card: On web-end, click on issue card button and put the IC card on the card swiping area. When you hear beep sound, you have successfully issued card. You can use this new card to unlock door. (Access control module required)
System Time

System time interface is shown in Figure 3- 4.

[image: image6.png]System Config Local Config ASC Manager

Date Format [ Yearbonth-Day <

System Time

> Indoor Station Manager

Time Format

? URIEET Y Time Zone

> Network Config 5 5
ystem Time

> Change Password

Info Search Refresh oK





Figure 3- 4
Under local config interface, click on system time where you can set time and sync it with PC.
3.2.2 Indoor Station (VTH) Manager


Indoor station manager interface of VTO 6110b consists of add VTH, delete VTH and modify VTH user.
Digital Indoor Station

Digital indoor station interface is shown in Figure 3- 5.

[image: image7.png]System Config
> Local Config

> LAN Config

> Network Config

> Change Password
Info Search
Status Statistics

Logout

Indoor Station

7 Display Resident Info Enable
Digital Indoor Stafion

FamilyName
Firsthame
VTH Short No.

1P Address

10110106
10110103

3301030110110106

3301030110110103¢

4« 111D B Gotg





Figure 3- 5
Under digital indoor station interface, check display residence info enable to show existing VTH info. Click add button at lower left, and input user info in the prompt box. In default, only VTH short no. is mandatory.

3.2.3 LAN Config
LAN config interface is shown in Figure 3- 6.
[image: image8.png]System Config LAN Config

> Local Config

VTH Middle No. Prefix 1011 (4-bit)
> Indoor Station Manager

VTH Full No. Prefix 010301 (8-bit)

VTO Short No. (4-bit)
> Network Config

VTO Middle No. Prefix (4-bit)
> Ghange Password

VTO Full No. Prefix (8-bit)
Info Search

VTS Full No. 01030100008000: (18-bit)
Status Statistics
Logout

¥ Registerto the WGT Centre

Registration Password

MGT Centre IP Address 36 120

MGT Port No. 12801

Call vis time 00 x]:o0 =] 7o [171=] .00 =] @' cailuts or not

Default Reffesh oK





Figure 3- 6

Default config is OK if you merely want to ensure the LAN connection between villa VTO and VTH. If you want to config manager center, the config here must match info of the manager center, and you need to check box of register to the MGT center. 

 
Furthermore, if you would like to call manager center within configured period, you have to set call VTS time. During this period, the VTO can only call VTS. 
3.2.4 Network Config

Network config interface is shown in Figure 3- 7.

[image: image9.png]System Config
> Local Config
> Indoor Station Manager

> LAN Config

> Change Password

Info Search
Status Statistics

Logout

Network Config

1P Address
Subnet Mask

Default Gateway

Default

Reffesh

oK





Figure 3- 7
Here you can set IP info of villa VTO which includes IP address, subnet mask and default gateway. After you modified the IP address, the web interface will reboot and you will be transferred to the new IP address. 

3.2.5 Change Password
Change password interface is shown in Figure 3- 8.
[image: image10.png]System Config Change Password

P lemEny 0ld Password

> Indoor Station Manager
New Password

> LAN Config .
Confirm
> Network Config
Change Password

Info Search
Status Statistics

Logout





Figure 3- 8

Here you can change the web login password of villa VTO. You need to input old password and new password and confirm new password. Click on OK to save.
3.3 Info Search 
3.3.1 Call History
Call history interface is shown in Figure 3- 9.
[image: image11.png]IP VDP Door Station Web Server V1.0

System Config VTO Call History

Info Search Talk Time{m) End State

Missed
Status Statistics lissed
Logout o Missed
Missed
Missed
Missed
Missed
Missed
Missed
Missed

Missed

44 138 b B Gotg

®





Figure 3- 9
Under info search interface, click on call history. Here you can search call history of the VTO with up to 1124 records. 
3.4 Status Statistics
3.4.1 VTH status 
VTH status interface is shown in Figure 3- 10.
Under status statistics interface, click on VTH status. Here you can view connection state of VTH.
[image: image12.png]System Config

Info Search
Status Statistics

VTH Status.

online
Online

nline
offine
offine
offine
online
Online

nline

Online

10.45.2.34:16802

2012:11:23 1450:16
11:2314;
0

0
1123 14
2012:11:23 14:50:16
2012:11:23 14:

2012:11:23 14:26:39

4« 111D B Gotg




Figure 3- 10
3.5 Logout
3.5.1 Reboot Device

Reboot device interface is shown in Figure 3- 11.
[image: image13.png]System Config Reboot Device
Info Search
Status Statistics
Logout

B

> Logout




Figure 3- 11

Here you can reboot the device.
3.5.2 Logout

Logout interface is shown in Figure 3- 12.
[image: image14.png]System Config
Info Search
Status Statistics
Logout

> Reboot Device

B





Figure 3- 12

Here you can logout the device.
4 Technical Specifications
	Model
	VTO6110B

	System

	Main processor
	Embedded microcontroller

	OS
	Embedded LINUX OS

	Video

	Video compression standard
	H.264

	Input/Approaching Induction
	1.3 mega pixels CMOS HD camera

	Night vision
	Support

	Audio

	Input
	Omnidirectional microphone

	Output
	Built-in loudspeaker

	Bidirectional talk
	Support bidirectional talk

	Operation Mode

	Input
	One-key input（with backlight）

	Swipe card
	Built-in IC card sensor

	Alarm

	Vandal proof
	Support

	Network

	Ethernet
	10M/100Mbps self-adaptation

	Internet protocol
	TCP/IP

	Storage

	Memory
	128MB

	General

	Power supply
	DC 10~15V

	Power consumption
	Standby ≤1W ；working ≤10W

	Environment
	 -10℃~+60℃、10~95%RH

	Dimensions
	115mm*45mm*155mm（L*W*H）

	Weight
	0.5kg


5 Device Port Illustration

Ports of VTO are illustrated in Figure 5- 1.
[image: image15.png]



Figure 5- 1
	No.
	Port Name
	Description

	1
	Vandal proof switch
	When this device is forced to leave wall, it will generate alarm sound and report to the manager center. 

	2
	Network port
	Connect to RJ45 port. 

	3
	RS485
	May connect to external module, such as access control extension module.

	4
	Power supply port
	Connect to 12V DC.

	5
	Loudspeaker output
	It is the sound output of local loudspeaker.


6. Installation Guide
Installation guide of VTO is illustrated in Figure 6- 1.
[image: image16.png]



Figure 6- 1
Installation step:

1. Fix installation holder onto wall: 

a) Use the M4 screw accompanied with the VTO to fix holder onto the 86 box (as  eq \o\ac(○,4) in Figure 6- 1); 
b) In order to firm the VTO, use the ST4.0 screw accompanied with VTO to fix the 86 box onto wall after locking the 86 box. (as  eq \o\ac(○,5) in Figure 6- 1);

2. Install  eq \o\ac(○,2) in Figure 6- 1 onto the holder by pushing from top edge as  eq \o\ac(○,1) in Figure 6- 1 to lower edge as  eq \o\ac(○,2) in Figure 6- 1. 

3. After you complete the installation between VTO and holder, use the set screw accompanied as  eq \o\ac(○,3) in Figure 6- 1 to fix the VTO and its holder. 
7 FAQ
1. Q: How do I know if the power supply to VTO is working normally?
A: After you plug the device to power supply, wait about 10s for the indicators in touch button to turn on. Approximately 60s later, all indicators will turn on as the device works normally. 
2. Q: You pressed the touch button, and the indicator turned on, but the VTO did not start a call?
A: Please check your operation process. 
3. Q: How to end a call when I am calling?
A: You can end a call by touching the button on VTO lightly.
4. Q: The device could not boot up and there was no sound or light.
A: Please check if power supply is well plugged. 
5. Q: My call did not go though.
A: It is network connection error; please check the cables of the device and its extension.
6. Q: The number I am calling does not exist.
A: Please check if the number is correct. 
7. Q: VTO did not respond after I swiped IC card.
A: Please check if your card is authorized.
8. Q: I heard a beep sound when I swiped IC card, but the door did is still locked.
A: Please check if your card is authorized.
9. Q: I have other problems not included above.
A: Please contact technical staffs for assistance. 
Note:
· This manual is for reference only. Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice. 

· All trademarks and registered trademarks are the properties of their respective owners. 

· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website or contact your local service engineer for more information.
8

